3GPP TSG-SA5 Meeting #164	S5-255492

Dallas, USA, 17 - 21 November 2025

Source:	Huawei, China Mobile

Title:	Pseudo-CR on TR 28.881 Add evaluation and conclusion for Use case #1 Enhancement of radio service delivering and assurance scenarios

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	0.2.0

Work Item:	FS_IDMS_MN_Ph4

Comments

Current civic location is limited to the civic address which refers to the CivicAddress Data Type defined in clause 6.1.6.2.14 in TS 29.572, however, in some scenarios, the MnS consumer may express the intent for a specified area om the form of location name (e.g. XXX hotel, XXX school). It proposes to change the CivicAddressContext to CivicAreaContext which support both civic address and location name.

This pCR also proposes to add evaluation and conclusion for Use case #1: Enhancement of radio service delivering and assurance scenarios.

Proposed Changes

* * * First Change * * * *

4.1 Use case #1: Enhancement of radio service delivering and assurance scenarios

4.1.1 Description

In 3GPP TS 28.312 [1], the existing use case and requirements for intent containing an expectation for delivering a radio service is described in clause 5.1.2. The RadioServiceExpectation is defined to represent MnS consumer's expectations for radio service delivering and assurance in the specified area. However, following scenarios are not supported:

- MnS consumer expresses the radio service delivering and assurance expectation with service reliability information.

- MnS consumer expresses the radio service delivering and assurance intent expectation for a specified area described in the form of civic location. The civic area can be represented by a civic address (e.g. country A, Building B, ROOM C) or location name (e.g. hotel name, school name). For example, MnS consumer wants to ensure the radio service targets (dLThptPerUETarget and dLLatencyTarget) for a specific civic address (e.g., the CivicAddress defined in clause 6.1.6.2.14 in TS 29.572 [3]).

- MnS consumer expresses the radio service delivering and assurance expectation for a specified assurance time duration. Using the concert as an example, MnS consumer expresses the intent indicating a radio service to be delivered and assured for two hours, or before concert, MnS consumer request to explore the supported maximum number of UE for the Radio Service which to be assured for two hours after it being delivered.

4.1.2 Potential requirements

REQ-IDMS_RadioServiceIntent -CON-1: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express service reliability requirements.

REQ-IDMS_RadioServiceIntent -CON-2: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express radio service delivering and assurance for a specified area described in the form of civic location.

REQ-IDMS_RadioServiceIntent -CON-3: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express radio service assurance time duration requirements.

4.1.3 Potential solutions

This solution proposes to reuse and enhance the existing RadioServiceExpectation defined in 3GPP TS 28.312 [1].

Enhancement Aspect1: Add following attributes as the ExpectationTargets for the RadioServiceExpectation to enable the MnS consumer to express service reliability requirements

- ReliabilityTarget, it represents the reliability target for the radio service that the intent expectation is applied.

Enhancement Aspect2: Add following attributes as the ObjectContexts for the RadioServiceExpectation to enable the MnS consumer to express radio service delivering and assurance requirements for a specified area described in the form of civic address.

- CivicAreaContext, the coverage areas for the Radio Service that the intent expectation is applied in the form of civic area (i.e. represented by CivicArea <<dataType>>). The CivicArea <<cdataType>> can support civic address (e.g. streets, districts, seats, etc.) or location label (e.g. hotel name, school name). The detailed definition for civicAddress reuses the CivicAddress Data Type defined in clause 6.1.6.2.14 in TS 29.572 [3]. The type for locationLabel is string. Following is the proposed definition for CivicArea <<dataType>>.

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| civicAddress | O | T | T | F | F |
| locationLabel | O | T | T | F | F |

Note: The locationLabel can be used if the location can not be expressed by civic address. The locationLabel should be used without ambiguity.

Enhancement Aspect3: Add following attributes as the Expectation Contexts for the RadioServiceExpectation to support assurance duration information:

- AssuranceDurationContext, it describes the time duration at which the radio service instance should be scheduled to be delivered and available. The type is integer and unit is hour.

4.1.4 Evaluation of potential solutions

Only one potential solution has been identified, which is feasible.

* * * Next Change * * * *

5	Conclusions and Recommendations

5.1 Use case #1: Enhancement of radio service delivering and assurance scenarios

The use case of enhancement of radio service delivering and assurance scenarios is introduced in clause 4.1 and following management capabilities are identified:

- MnS consumer expresses the radio service delivering and assurance intent expectation with service reliability information.

- MnS consumer expresses the radio service delivering and assurance intent expectation for a specified area described in the form of civic location.

- MnS consumer expresses the radio service delivering and assurance expectation for a specified assurance time duration.

It is recommended to enhance the RadiServiceExpectation defined in 3GPP TS 28.312 [1] to support above identified management capabilities:

- add "ReliabilityTarget " as the ExpectationTarget.

- add "CivicAreaContext" as the ObjectContext, which support both civic address and location label.

- add "AssuranceDurationContext" as the ExpectationContext.

The detailed solution in clause 4.1.3 is used as baseline for normative work.

* * * End of Changes * * * *