3GPP TSG-SA5 Meeting #164	S5-255496

Dallas, USA, 17 – 21 November 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on Rel-20 TR 28.881 Enhance solution for Use-Case#11 and add evaluation, recommendations

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	0.2.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This pCR provides enhancements to the potential solution of Use-Case#11 for removing editor’s note, and adds evaluation and recommendations for the use-case.

Proposed Changes

* * * First Change * * * *

## 4.11	Use case #11: Enhancing intent feasibility check capability

### 4.11.1	Description

This use case describes proposed enhancements for feasibility check, enabling intent-driven MnS consumer to receive information regarding how to make an infeasible intent feasible together with feasibility check in case the intent is deemed as infeasible. This will support MnS consumers to understand the changes needed for a feasible intent.

In 3GPP TS 28.312 [1], clause 5.3.3.1 states the following:

“In case the result of intent fulfillment feasibility check is infeasible, MnS producer notifies the MnS consumer the reason of infeasibility and corresponding recommendations, then the MnS consumer decides how to handle the issue that intent is infeasible, e.g. update the intent, suspend the intent, delete the intent, etc.”

However, the issue is the IntentFeasibilityCheckReport specified in 3GPP TS 28.312 [1], clause 6.2.1.3.10 does not include corresponding recommendations. The MnS consumer may request exploration as specified in 3GPP TS 28.312 [1] Clause 5.3.5, based on inFeasibleExpectationInfos which is an optional attribute indicating the infeasible expectations including infeasible targets.  However, if the inFeasibleExpectationInfos attribute is not supported, then it is very difficult for the intent-driven MnS consumer to update an infeasible intent as they do not have the recommendation on how to make it feasible.

### 4.11.2	Potential requirements

REQ-Intent_FEAS1: The intent driven MnS producer should have the capability to enable the authorized MnS consumer to receive the intent exploration report together with intent feasibility check report, if the feasibility check result is infeasible.

### 4.11.3	Potential solutions

#### 4.11.3.1	Potential solution #1 for a new procedure of feasibility check with exploration

This solution proposes to reuse the existing generic intent exploration report and intent feasibility check report defined in 3GPP TS 28.312 [1] and to enhance the Intent <<IOC>>.

In order to enable an MnS consumer to request a feasibility check with corresponding recommendations, a procedure is proposed with ensuring feasibility check with exploration is supported and enabled by a new allowed value “FEASIBILITYCHECK_WITH_EXPLORATION” that is to be included for intentMgmtPurpose attribute of Intent <<IOC>> which is specified in 3GPP TS 28.312, clause 6.2.1.4. When feasibility check with exploration is requested by the intent-driven MnS consumer, the intent driven MnS producer can report corresponding recommendations for infeasible expectations with existing IntentExplorationReport <<dataType>>, together with existing IntentFeasibilityCheckReport <<dataType>>.

This solution does not have any impact on the intent lifecycle management states studied in clause 4.13 of the present document.

* * * Second Change * * * *

### 4.11.4 Evaluation of potential solutions

Two potential solutions are identified in clause 4.11.3. Both potential solution #1 and potential solution #2 support the capability to allow MnS consumer to receive recommended values for infeasible targets in an infeasible intent.

The implementation of both potential solutions is not complex. Therefore, both potential solution described in clause 4.11.3 are feasible solutions.

* * * Third Change * * * *

# 5	Conclusions and Recommendations

Editor's note: this clause will contain conclusions and recommendations for corresponding key issues identified in clause 4.

## 5.X	Use case #11: Enhancing intent feasibility check capability

The use case of enhancement of intent feasibility check capability is introduced in clause 4.11 and following management capabilities are identified:

- The capability enabling the MnS consumer to receive recommended values for infeasible targets.

The potential solution described in clause 4.11.3.1 proposes a new allowed value “FEASIBILITYCHECK_WITH_EXPLORATION” that is to be included for intentMgmtPurpose attribute.

The potential solution described in clause 4.11.3.2 proposes to extend the type for attribute “inFeasibleTargets” from “String” to “inFeasibleTargetInfo <<dataType>>”, which includes targetName and recommendedValue.

* * * End of Changes * * * *