3GPP TSG-SA5 Meeting #164	S5-255528

Dallas, USA 17 - 21 November 2025                                              revision of S5-255163

Source:	Huawei

Title:	pCR on TR 28.883 Add solution of NDT reporting nethod

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	0.1.0

Work Item:	FS_NDT_Ph2

Comments

Add a potential solution for Use Case #5: Enhancement on NDT reporting method.

Proposed Changes

* * * First Change * * * *

## 5.5	Use Case #5: Enhancement on NDT reporting method

### 5.5.1	Description

In TS 28.561 [3], the reporting method of NDT output is notification based reporting. The NDT MnS consumer receives an NDT report from NDT producer by invoking generic provisioning management service operations and notifications in TS 28.532 [4]. However, different NDT job may have different requirement on reporting method. For example, in the scenario of signalling storm, it is more appropriate to use streaming data reporting service to timely report the simulation/emulation information on potential network impact.

It is proposed to enable the NDT MnS consumer to select the reporting method of simulation/emulation output based on different requirement. In addition to notification based reporting, streaming data reporting service also need to be supported for NDT output reporting.

### 5.5.2	Potential requirements

REQ-NDTRM-01: The 3GPP management system should support a capability to allow an authorized MnS consumer to request NDT report to be provided by streaming data reporting service.

REQ-NDTRM-02: The 3GPP management system should support a capability for NDT MnS producer to support delivery of NDT report by streaming data reporting service.

### 5.5.3	Potential solutions

It is proposed to make the following changes to TS 28.561 [3] clause 6.2.1.3.2:

- update the NDTJob IOC attribute to add a new optional attribute reportingMethod, which can be used to indicate the required reporting method of NDT report, the allowed values of reportingMethod can include streaming and notification. The new attribute can only be used when the requested NDTFunction support the capability that allow a MnS consumer to indicate the reporting method.

### 5.5.4	Evaluation of potential solutions

* * * End of Changes * * * *