3GPP TSG-SA5 Meeting #164	S5-255290

Dallas, USA, 17 - 21 November 2025

Source:	L.M. Ericsson Limited

Title:	Rel-20 pCR TR 28.883 Create and Execute NDT Job

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.1.0

Work Item:	FS_NDT_Ph2

Comments

The existing 28.561 specification lacks an operation to execute the NDTJob after its creation. 
There is only the createMoi operation for the NDTJob, and the sequence diagram in 6.4.1 indicates there should be a separate operation to execute the NDTJob. This pCR suggests studying how NDTJobs are created and executed in particular which sequence they follow.

Proposed Changes

* * * First Change * * * *

# 4	Concepts and background

Editor’s note: This clause provides a description of concepts and background.

# 5	Use cases

## 5.X	Use Case #<X>: Create and Execute NDT Job

### 5.X.1	Description

The MnS Consumer requires clarity on whether the creation and execution of an NDTJob are distinct phases or a single combined procedure.

By having creation of an NDTJob separate to the execution of the NDTJob, it means certain MnS Consumers can define the NDTJob, and other MnS Consumers could execute the NDT jobs. This is beneficial to the MnS Consumers as users with different roles can create the NDTJob such as subject matter experts, then a different set of users can execute the simulation, which creates a clear separation of responsibilities.

In the current specification, the relationship between these phases is inconsistent across clauses:

- Clause 4.4 (NDT Life-cycle Management) describes “NDT job instantiation” as including both creation and execution within a single step, suggesting automatic execution upon creation.

- Clause 6.4 (Procedures for NDT operations) depicts separate interactions between the MnS Consumer and MnS Producer for creation and execution, implying that execution follows a distinct request.

- Clause 7.1 (RESTful HTTP-based solution set) defines only the createMOI operation for NDTJob creation, with no corresponding operation for initiating execution.

This discrepancy makes it unclear how an MnS Consumer can prepare an NDTJob configuration and decide when to initiate its execution. A consistent and state-aware model distinguishing creation from execution would enable deterministic control.

### 5.X.2	Potential requirements

### 5.X.3	Potential solutions

### 5.X.4	Evaluation of potential solutions

# 6	Conclusions and Recommendations

## 6.X	Use case #<X>: <use case title>

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

* * * End of Changes * * * *