3GPP TSG-SA5 Meeting #164	S5-255559

# Dallas, USA, 17 - 21 Oct 2025

Source:	Samsung

Title:	RET Analytics

Document for:	Agreement

Agenda item:	6.20.7

Spec:	3GPP TR 28.886

Version:	0.2.0

Work Item:	FS_eMDAS_Ph4

# 1	Decision/action requested.

The group is asked to endorse the proposal.

# 2	References

None

# 3	Rationale

This contribution provides the solution for RET analytics.

* * * End of Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management Data Analytics (MDA)".

[3]	3GPP TS 28.541: "5G Network Resource Model (NRM); Stage 2 and stage 3".

[4]	3GPP TS 28.567: "Management aspects of closed control loops".

[x]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM)".

[y]	3GPP TS 38.104: "Base Station (BS) radio transmission and reception".

* * * First Change * * * *

### 5.3.4	Use case 4: Remote Electrical Tilt (RET) and Transmission Power analytics (TP)

#### 5.3.4.1	Description

Operators establish their own guidelines for deploying antenna RET, both mechanical and electrical, in cells, primarily based on LTE/NR layers and traffic management strategies. Initial RET and power configurations may not be optimal for all cells, necessitating ongoing fine-tuning. The initial RET may not account for changes in subscriber movement or infrastructure developments. Factors influencing optimal RET and power can evolve over time, including:

Subscriber Movement: Dynamic changes in user behavior, such as daily commutes from office to home or weekend outings to recreational areas, can impact network performance requiring the modification in the RET.

Infrastructure Growth: The addition of new cell sites can introduce interference with neighboring cells. Interference arises when signals from the new site overlap with or disrupt existing signals. This would affect performance and can be mitigated with modification in the RET.

Weather Conditions: Atmospheric phenomena like rain, snow, and fog can affect radio wave propagation, leading to signal attenuation or distortion. Tilt modification may minimize the impact.

It is desirable to use MDA analytics to generate optimal RET and power configuration over time. It is essential to follow initial antenna RET deployment with subsequent fine-tuning using MDA analytics. The producer employs a machine learning-based model that leverages strong correlations between performance, fault and configuration data of the network. This ML model analyzes the impacts of coverage and interference in relation to user traffic behavior, including mobility and weather conditions. It generates recommendations for fine-tuning RET and optimizing transmission power per cell.

#### 5.3.4.2	Potential requirements

REQ-RET-DATA-1: MDA capability for RET analysis shall include providing analytics for indicating the recommended change in RET and TP for a specific cell.

#### 5.3.4.3	Possible solutions

It is proposed to add a new MDA capability for RET and TP analysis. The MDAS Consumer submits an MDARequest, specifying the target cell(s) and the periodic interval at which data should be collected. MDAS Consumer receives the MDA Report for the RET_TPAnalytics. MDAS Producer collects the enabling data and generates the RET_TPAnalytics data. The generated data is returned as MDAReport. After receiving the MDAReport containing the recommended value for RET and TP, the consumer initiates RET and TP configuration.

The managedEntityScope may indicate a geographical area or a specific cell for which the analytics is requested.

The following are the enabling data for this MDA analytics

- Configuration data related with the target cell including latitude, longitude as defined in clause 4.3.2 of [x].

- Configuration data relayed with the cell including azimuth, ARFCN, Antenna height, electrical tilt, minimum electrical tilt, maximum electrical tilt, mechanical tilt and antenna identification as defined in clause 5 of [y].

The MDAReport for this analytics will contain the following information in addition to the common information element of analytics outputs defined in clause 8.3.1 of 3GPP [2].

- Recommended tilt: This specifies the recommended RET value.

- Recommended TP: This specifies the recommended TP value.

Note: the recommendations would apply to the transmission point(s) related with the target cell(s)

* * * End of Change * * * *