3GPP TSG-SA5 Meeting #164	S5-255567

Dallas, USA, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Add use case and requirements on access control on notifications

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.1.0

Work Item:	FS_EnExpo

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.579: "Management services exposure to external consumers through CAPIF".

[4]	3GPP TS 28.319: "Access control for management services".

[Y1]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration   Reference Point (IRP); Information Service (IS)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

# 4	Concepts and background

## 4.X Access control on notifications

Currently, the 3GPP management system supports notification subscription via the NtfSubscriptionControl IOC (see clause 4.3.22 of TS 28.622[Y1]), enabling MnS consumers to subscribe to receive notifications from MnS producers.

The NtfSubscriptionControl IOC (as defined in clause 4.3.22 of TS 28.622[Y1]) includes the following attributes:

Table 4.X-1: NtfSubscriptionControl properties

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationRecipientAddress | M | T | T | F | T |
| notificationTypes | O | T | T | F | T |
| scope | O | T | T | F | T |
| notificationFilter | O | T | T | F | T |
| lastSequenceNo | O | T | F | F | F |
| operationalState | O | T | F | F | T |
| availabilityStatus | O | T | F | F | T |

Further, clause 7.3 of TS 28.319[4] defines the AccessRule class associated to a given MnS consumer identified by their Identity and Role classes. The AccessRule class includes the following attributes (shown in Table 4.X-2) as defined in clause 7.3.3 of TS 28.319[4]):

Table 4.X-2: AccessRule properties

| Attribute Name | S |
| --- | --- |
| ruleName | M |
| dataNodeSelector | M |
| operations | M |
| actions | O |
| componentCData | O |

In clause 7.3.3 of TS 28.319[4], componentCData is defined as an optional attribute which specifies notification types and performance metric names.

* * * Next Change * * * *

## 5.X	Use case #<X>: Access control on notifications

### 5.X.1	Description

Currently, the 3GPP management system supports notification subscription via the NtfSubscriptionControl IOC (see clause 4.3.22 of TS 28.622[Y1]), enabling MnS consumers to subscribe to notifications from MnS producers.

The NtfSubscriptionControl IOC (as defined in clause 4.3.22 of TS 28.622[Y1]) includes the following attributes:

Table 5.X.1-1: NtfSubscriptionControl properties

Further, clause 7.3 of TS 28.319[4] defines the AccessRule class associated to a given MnS consumer identified by their Identity and Role classes. The AccessRule class includes the following attributes as defined in clause 7.3.3 of TS 28.319[4]):

Table 5.X.1-2: AccessRule properties

When an MnS consumer subscribes to receive notifications using the NtfSubscriptionControl IOC (see clause 4.X), the 3GPP management system needs to be able to determine whether a given MnS consumer is authorized to receive notifications on a set of managed object(s). Additionally, an MnS consumer could subscribe to receive notifications by specifying a notification scope that exceeds its authorized notification scope, the 3GPP management system needs to be able to handle such requests.

* * * Next Change * * * *

### 5.X.2	Potential requirements

PREQ-FS_EnExpo-Ntf-01: The 3GPP Management System should provide the capability to configure, per role (as defined in clause 7.3.2 of TS 28.319[4]), the set of notification types that the MnS consumer is authorized to receive.

PREQ-FS_EnExpo-Ntf-02: The 3GPP Management System should provide the capability to enforce the restrictions configured in requirement PREQ-FS_EnExpo-Ntf-01.

PREQ-FS_EnExpo-Ntf-03: The 3GPP Management System should provide the capability to configure, per role, the information content that the MnS consumer is authorized to receive in a notification.

Editor’s note: If the proposed solution is deemed too complicated, then this requirement could be removed.

Editor’s note: The access control rules for reading data via CRUD and access control rules for receiving data via notifications should be common.

Editor’s note: The relationship between the entity that subscribes to receive notifications and the entity that receives the notifications needs to be clarified.

PREQ-FS_EnExpo-Ntf-04: The 3GPP Management System should provide the capability to enforce the restrictions configured in requirement PREQ-FS_EnExpo-Ntf-03.

* * * End of Changes * * * *