3GPP TSG-SA5 Meeting #164	S5-255569

Dallas, USA, 17 - 21 November 2025

Source:	Huawei

Title:	Pseudo-CR on TR 28.887 enhancement of Management data collection

Document for:	Approval

Agenda item:	6.20.8

Spec:	3GPP TR 28.887

Version:	0.2.0

Work Item:	FS_MADCOL_Ph3

Comments

This contribution proposes to enhance the ManagementDataCollection IOC to enable an authorized data consumer to request performance metrics to be produced according to a specified granularity period.

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

* * *Second Change * * * *

4.4 	Clarification of Mechanisms to Discover, Request and Retrieve Management Data

4.4.X	Use Case#X: Enhancement of Management data collection

4.4.X.1      Description

The ManagementDataCollection IOC defined in TS 28.622 [X] represents a management data collection request job for trace metrics and performance metrics. The ManagementDataCollection IOC includes attributes “managementData”, “targetNodeFilter”, “collectionTimeWindow”, “reportingCtrl”, “dataScope”, “condition”, “processMonitor”, “consolidateOutput” and “jobId”.

For the performance metrics (including performance measurements and KPIs), different MnS consumers may have different requirements for the granularity period used to report and produce performance metrics.

4.4.X.2      Problem Statement

In current definition for ManagementDataCollection IOC, it is not clear how the granularity period for data production for the PerfMetricJob (which include the mandatory attribute “granularityPeriod”) can be derived from a ManagementDataCollection.

4.1.X.3      Potential Requirements

TBD

4.1.X.4      Potential Solution

TBD

* * * End of Changes * * * *