3GPP TSG-SA5 Meeting #164 	S5-255588

Dallas, USA, 17th November 2025 – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0539 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 32.422 temporary suspension on TraceJob |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current Situation The TraceJob IOC currently lacks administrativeState and operationalState attributes that exist in the perfMetricJob IOC. This creates operational inefficiencies when users need to temporarily halt trace jobs. Problem Statement - Operators currently must delete and recreate trace jobs to pause/resume them - This causes unnecessary overhead and potential configuration loss - Operators need indicate strong demand for pause/resume functionality Proposed Solution Add state management capabilities to TraceJob IOC Benefits - Preserve job configurations during temporary stoppages - Reduce operational overhead - Align with perfMetricJob functionality - Meet operators’ expectations for job management This proposal was discussed at SA5#161, and an LS was sent to RAN3 and RAN2. In its reply LS (R2-2507745), RAN2 agreed to the addition of these attributes. In its reply LS (S5-255026), RAN3 confirmed that supporting the addition of the attributes described in the SA5 LS to the TraceJob IOC is technically feasible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Trace Session suspension and resumption procedure for Trace, MDT and 5GC UE level measurements collection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Trace Session suspension and resumption procedure in TraceJob |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS28.622 CR 0573 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

## 4.x	Trace Session suspension and resumption for Trace, MDT and 5GC UE level measurements collection

At any time after a trace session is created, the trace session may be temporarily suspended. When a trace session instance is suspended, the trace session(s) shall be deactivated. However, the trace session shall not be terminated in the node. When the trace session is resumed, the coresponding trace session(s) shall be reactivated.

In management-based 5GC trace suspension, the trace session shall be deactivated in the 5GC node, as specified in subclause 4.1.3.9, without terminating the trace session from the 5GC node. When the trace session instance is resumed, the trace session shall be reactivated as specified in subclause 4.1.1.8.

In management-based NG-RAN trace suspension, the trace session shall be deactivated, as specified in subclause 4.1.3.10, without terminating the trace session from the NG-RAN node. When the trace session is resumed, the trace session shall be reactivated as specified in subclause 4.1.1.9.

In management-based NG-RAN MDT suspension, the MDT session shall be deactivated, as specified in subclause 4.1.3.11, without terminating the trace session from the NG-RAN node. When the trace session is resumed, the MDT session shall be reactivated as specified in subclause 4.1.1.9. In addition, for C-MDT suspension, C-MDT reconfiguration shall be stopped. Besides, for logged MDT trace sessions, this Trace Session suspension does not apply to the UE. The logged MDT trace session in the UE is terminated when logging duration expires, or the UE state transforms to RRC-CONNECTED. Any available logged MDT report shall be forwarded to TCE.

In management-based 5GC UE level measurements collection, the 5GC UE level measurements session shall be deactivated, as specified in subclause 4.1.3.12, without terminating the trace session from the 5GC node, e.g. UDM. When the trace session is resumed, the 5GC UE level measurements session shall be reactivated as specified in subclause 4.1.1.10.

In signalling-based trace suspension, the trace session shall be deactivated in the UDM, as specified in subclause 4.1.4.11, without terminating the trace session from the UDM. When the trace session is resumed, the trace session shall be reactivated as specified in subclause 4.1.2.15.

In signalling-based MDT suspension, the MDT session shall be deactivated, as specified in subclause 4.1.4.13, without terminating the trace session from the UDM. When the trace session is resumed, the trace session shall be reactivated as specified in subclause 4.1.2.17. Besides, for logged MDT trace sessions, this Trace Session suspension does not apply to the UE. The logged MDT trace session in the UE is terminated when logging duration expires, or the UE state transforms to RRC-CONNECTED. Any available logged MDT report shall be forwarded to TCE.

In signalling-based 5GC UE level measurements collection, the 5GC UE level measurements session shall be deactivated, as specified in subclause 4.1.4.14, without terminatingthe trace session from the 5GC node. When the trace session is resumed, the 5GC UE level measurements session shall be reactivated as specified in subclause 4.1.2.18.

In RRC reporting measurements collection, the trace session shall be deactivated in NG-RAN, as specified in subclause 4.11.2, without terminatingthe trace session from the NG-RAN node.

In RCEF reporting measurements collection, the trace session shall be deactivated in NG-RAN, as specified in subclause 4.8.4, without terminating the trace session from the NG-RAN node.

In RLF reporting measurements collection, the trace session shall be deactivated in NG-RAN, as specified in subclause 4.3.4, without terminating the trace session from the NG-RAN node.

*** END OF CHANGE ***