3GPP TSG-SA5 Meeting #164	S5-255592

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0747 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR 28.552 Add new measurements related to ADRF storage services when NWDAF hosting ADRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_Ph3-OAM |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | By measuring the number of successful and failed storage and retrieval service requests, the operator can estimate the potential workload on NWDAF hosting ADRF and take management actions for configuration and resource allocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new measurement “Number of successful storage service requests received by ADRF hosted by NWDAF” is introduced. - A new measurement “Number of failed storage service requests received by ADRF hosted by NWDAF” is introduced. - A new measurement “Number of successful retrieval service requests received by ADRF hosted by NWDAF” is introduced. - A new measurement “Number of failed retrieval service requests received by ADRF hosted by NWDAF” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the performance of NWDAF hosting ADRF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.X      Performance measurements related to ADRF

5.X.A    Measurements related to storage services provided by ADRF

5.X.A.x   Number of failed storage service requests received by NWDAF hosting ADRF

a)	This measurement provides the number of storage service requests received and rejected by NWDAF hosting ADRF.

b)  CC

c)	On receiving the storage service request "Nadrf_DataManagement_StorageRequest" by NWDAF hosting ADRF to store data or analytics, if the request is rejected, each rejected request is added to the corresponding counter. This measurement can be split into subcounters per consumer NF. The consumer NF can be NWDAF, DCCF or MFAF. (See TS 23.288 [59]).

d)	It is an integer value

e)	DANS.DataStorageResRejected

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.A.y   Number of successful storage service requests received by NWDAF hosting ADRF

a)	This measurement provides the number of storage service requests received and accepted by NWDAF hosting ADRF.

b)  CC

c)	On receiving the storage service request "Nadrf_DataManagement_StorageRequest" by NWDAF hosting ADRF to store data or analytics, each accepted request is added to the corresponding counter. This measurement can be split into subcounters per consumer NF. The consumer NF can be NWDAF, DCCF or MFAF. (See TS 23.288 [59]).

d)	It is an integer value

e)	DANS.DataStorageResAccepted

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.B    Measurements related to retrieval services provided by ADRF

5.X.B.x   Number of failed retrieval service requests received by NWDAF hosting ADRF

a)	This measurement provides the number of retrieval service requests received and rejected by NWDAF hosting ADRF.

b)  CC

c)	On receiving the retrieval service request "Nadrf_DataManagement_RetrievalRequest" by NWDAF hosting ADRF to retrieve data or analytics, if the request is rejected, each rejected request is added to the corresponding counter. This measurement can be split into subcounters per consumer NF. The consumer NF can be NWDAF, DCCF. (See TS 23.288 [59]).

d)	It is an integer value

e)	DANS.DataRetrievalResRejected

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.B.y   Number of successful retrieval service requests received by NWDAF hosting ADRF

a)	This measurement provides the number of retrieval service requests received and accepted by NWDAF hosting ADRF.

b)  CC

c)	On receiving the retrieval service request "Nadrf_DataManagement_RetrievalRequest" by NWDAF hosting ADRF to retrieve data or analytics, each accepted request is added to the corresponding counter. This measurement can be split into subcounters per consumer NF. The consumer NF can be NWDAF, DCCF. (See TS 23.288 [59]).

d)	It is an integer value

e)	DANS.DataRetrievalResAccepted

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of Change