3GPP TSG-SA5 Meeting #164	S5-255607

Dallas, USA, 17 - 21 November 2025

Source:	Vodafone

Title:	Pseudo-CR on Concepts and background for UMMR

Document for:	Approval

Agenda item:	6.20.15

Spec:	3GPP TR 28.892

Version:	v0.0.1

Work Item:	FS_UMMR_OAM

Comments

Proposed Changes

* * * First Change * * * *

# 4	Concepts and background

## 4.1	IRP and SBMA overview

The 3GPP management architecture has evolved across generations, reflecting a transition from interface-oriented to model-driven and service-based paradigms. Two main frameworks are currently defined in 3GPP specifications: the Integration Reference Point (IRP) framework and the Service-Based Management Architecture (SBMA).

The IRP framework, introduced and widely applied in legacy systems such as 4G/E-UTRAN, defines management through standardized Network Resource Models (NRMs) specified using XML-based schemas. This model provides a mature and reliable foundation for multiple network domains.

With the introduction of 5G, the SBMA model was specified to enable a more flexible, service-oriented, and model-driven approach to network management. SBMA is characterized by the definition of Management Services (MnS), which expose standardized management capabilities through service-based interfaces and NRMs specified using YANG and OpenAPI. This approach enhances automation, interoperability, and alignment with cloud-native network principles.

While 5G management fully adopts the SBMA model with YANG and OpenAPI based NRMs, 4G management remains entirely IRP-based, lacking a corresponding YANG representation. This creates a gap that prevents the consistent use of a unified management framework across generations. To ensure coherent multi-RAT management, it is necessary to extend the SBMA-based framework to include 4G elements.

* * * End of Changes * * * *