3GPP TSG-SA5 Meeting #164	S5-255610

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0370 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Clarify cardinality of IntentReports |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Korea Partners Co Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current description of the IntentReport <IOC> incorrectly states that there is only a single intent report instance, which is automatically deleted when the corresponding intent instance is deleted. This does not reflect the possibility of multiple intent report instances associated with a single intent instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that all IntentReport instances associated with a given intent instance shall be deleted when the intent instance is deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relationship between intent instances and their associated intent report instances may be misinterpreted. As a result, implementations may incorrectly assume a one-to-one relationship and fail to properly delete all intent report instances upon intent instance deletion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 6.2.1.2.2	IntentReport <<InformationObjectClass>

6.2.1.2.2.1	Definition

This IOC represents intent report information from MnS producer to MnS consumer. The IntentReport instance is created by MnS producer as a result of the IntentReportControl (specified in the Intent IOC) that specifies the types and conditions for reporting. When the MnS producer deletes an intent instance based on a request from MnS consumer, the corresponding intent report instances are also deleted by MnS producer automatically.

The IntentReport IOC includes

-	intentFulfilmentReport, which represents the properties of fulfillment information for expectation target, intent expectation, and the whole intent. The fulfilmentReport will be observed from the start of each observation period (specified in the IntentReportControl data type), then at the end of each observation period, the corresponding values will be derived and configured.

-	intentConflictReports, which represents detected conflict information, including conflict type (i.e., intent conflict, expectation conflict and target conflict) and possible solution recommendations to address the conflicts.

-	intentFeasibilityCheckReport, which indicates that the intent is feasible or infeasible. Intent feasibility check information is provided after MnS producer automatically performs feasibility check when receiving the intent creation and modification request from MnS consumer.

-	intentExplorationReport, which represents the properties of the intent exploration result during negotiation in intent pre-evaluation phase. Intent exploration result is provided after MnS producer performs intent exploration pre-evaluation process as requested by MnS consumer.

-	intentFulfilmentNegotiationReport, which represents the properties of intent negotiation information negotiation in intent fulfilment phase. Intent fulfilment negotiation information is provided to MnS Consumer by the MnS producer during intent fulfilment phase.

-	intentUtilityReports, which provides the results of applicable Intent Utility Functions. The results are calculated and reported per utility function applicable to the intent specified in intentReference.

Each instance of IntentReport IOC can contain one or any combination of intentFulfilmentReport, intentConflictReports, intentFeasibilityCheckReport, intentExplorationReport, intentFulfilmentNegotiationReport and intentUtilityReports.

Different MnS consumers can use the "getMOIAttributes" operation to query different attributes of the IntentReport <<IOC>> to obtain corresponding intent report information (including intentFulfilmentReport, intentConflictReports, intentFeasibilityCheckReport, intentExplorationReport, intentFulfilmentNegotiationReport and intentUtilityReports).

Different MnS consumers can subscribe to attribute value change notifications for the IntentReport <<IOC>> to obtain the notifications for different intent report information.

End of  changes