3GPP TSG-SA5 Meeting #164	S5-255114

Dallas, Texas, USA, 17 – 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Intent Interpretation Assistance Information Solution

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR28.881

Version:	0.2.0

Work Item:	Intent driven management services for mobile network phase 4

Comments

This pCR provides the solution to the use case#6 (Section 4.6) in TR28.881.

[1] 	3GPP TR 28.881 “Study on intent driven management services for mobile network phase 4”.

Proposed Changes

* * * First Change * * * *

### 4.6	Use case #6: Intent Interpretation Assistance Information

### 4.6.1 Description

A similar intent can be instantiated on several intent handlers, e.g., on several management systems each responsible for a for a different domain. TS28.312 supports intent negotiation that enables interaction between the MnS producer and MnS consumer to clarify the ability to fulfill intent requirements that may otherwise not be realizable owing to their abstract nature. For the same intents that are sent to several MnS producers, negotiation with the several MnS producers to clarify the intents can be resource wasteful.

The intent driven management service can minimize the number of negotiation exchanges while allowing effective interpretation of intents by reusing the interpretation information  . The MnS consumer compiles prior interactions, e.g. previous IntentNegotiationReports or intentNegotiationConsumerFeedback, and shares those prior interactions as intent interpretation assistance information with the  handler. The Mns consumer should be enabled to provide this intent interpretation assistance information that can be used by the intent handler to support the interpretation of the intent. As an example, if during intent negotiation, a set of options were provided by MnS producer and the MnS consumer selectes one option, the MnS consumer may want the MnS prodcer to resue this information, so the MnS consumer can provide it to the MnS producer for reuse.

Note: The intent interpretation assistance information can be visualized as a “frequently asked questions (FAQ)” section used in many human-oriented service descriptions (e.g. on service providers’ websites).

### 4.6.2 Potential requirements

REQ-Intent_ Interpretation_Assistance_1: The intent driven MnS producer should have the capability enabling the MnS consumer to provide information containing prior intent negotiations to the handler that can then be used by the intent handler to support the interpretation of the intent.

#### 4.6.3 Potential solutions

- 	Add to the  Intent IOCa description of the information about intent interpretation e.g., as:

-	The intent IOC includes intentInterpretationAssistanceInfo which contains prior negotiation interactions from the MnS consumer that are provided to the intent handling function and can be used by the intent handling function to support the interpretation of the intent. The intentInterpretationAssistanceInfo is a hash map that captures the previous MnS producer questions, requests or reports and the corresponding consumer responses. It may include following information related to a timestamp, applicable intent context, and the previous MnS producr to MnS consumer interactions.

-		For example, the hasmap may include a key that is "the set of previous intent fulfillemnt options that were provided by MnS producer" as well as the corresponding value which is " the option that was selected by the MnS consumer".

- 	Introduce a data type and a corresponding attribute on the Intent IOC for intent interpretation assistance information say named intentInterpretationAssistanceInfo. The attribute is readableand writtable.

- 	The intentInterpretationAssistanceInfo is a hash map that captures the previous questions, requests or reports and the coresponding consumer responses. Examples can include the following attributes/ information:

- A timestamp

- Conditions: the conditions under which the intent was previously sent

- Producer report: the request sent by the MnS producer

- Consumer response: the responses provided by the MnS consumer for the specific MnS producer report.

* * * End of Changes * * * *