3GPP TSG-SA5 Meeting #164	S5-255628

Dallas, Texas, USA, 17 – 21 November 2025	revision of S5-255115

Source:	Nokia

Title:	Pseudo-CR on Invariant Guidance in Intent Contexts Solution

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR28.881

Version:	0.2.0

Work Item:	Intent driven management services for mobile network phase 4

Comments

This pCR provides the solution to the use case#5 (Section 4.5) which was approved in the last meeting.

[1] 	3GPP TR 28.881 “Study on intent driven management services for mobile network phase 4”.

Proposed Changes

* * * First Change * * * *

4.5	Use case #5: Invariant Guidance in Intent Contexts

### 4.5.1 Description

TS 28.312 describes a scenario where for an instantiated intent, the intent handler may need to decompose the intent into multiple derivative intents, each to be fulfilled by a separate other intent handler. There may be contexts in the intent which the MnS consumer desires that the other intent handlers understand them exactly as they were provided, i.e., without any modification by the first intent handler in the decomposition process.

As an example, the MNO may, for some decomposition use-cases, want to only use certain certified hardware (e.g., with certain security, privacy, energy consumption, other types of quality guarantees) to be used in the fulfillment of RAN or CN operations. This additional guidance to use this specified hardware or the quality constraints on the resources may be provided as part of the intent context.

The MnS consumer should be enabled to indicate the context which should be transmitted to other intent handlers without modification.

### 4.5.2 Potential requirements

REQ-Intent_InvarGui-1: The intent driven MnS should include a capability enabling the MnS consumer to indicate the requirements, goals and contexts which the MnS consumer recommends to be copied into decomposed intents and transmitted to other intent handlers without modification.

4.5.3 Potential solutions

The “context” dataType, as defined in TS 28.312, can be extended to include information about invariant guidance.

-	add a description for invariant guidance e.g. as: The Context can be used to indicate invariant guidance, e.g., which context should be propagated to all intent handler functions in the case where the intent is decomposed into multiple sub intents. The attribute contextInvariant is a flag to indicate if a specific context should be considered as in invariant guidance or not. The flag applies to context at the expectation level and intent target level.

-	The following attribute would be added to the Context <<dataType>>, i.e.:

Addition to Table 6.2.1.3.4.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| contextInvariant | O | T | F | T | T |

The new attribute can be defined as:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| contextInvariant | It indicates whether the specific context should be considered as invariant guidance to be transferred to subsequent intent handling functions without any modification. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 4.5.4  Evaluation of potential solutions

TBA

* * * End of Changes * * * *