3GPP TSG-SA5 Meeting #164	S5-255637

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | 0049 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.537 Add missing RESTFUL implementation description for management data collection and discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The solution description for follwing management capabilities are descirbed in clause 6.1.4 and 6.5.4, however, the detailed implementation description (RESTful HTTP-based solution set) for each functionality is missing. - Producing and reporting management data. - Discovery of management data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the detailed implementation description (RESTful HTTP-based solution set) for each functionality of the following management capablities: - Producing and reporting management data. - Discovery of management data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The detailed solution description for each functionality of the above management capabilities are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4.2, 6.5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.1.4.2	Stage 3 definition

Following are the stage 3 definition for  producing and reporting management data.

- RESTful HTTP-based solution set

- RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in 3GPP TS 28.532 [2]. OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

- OpenAPI document " TS28623_ManagementDataCollectionNrm.yaml " in clause 4.3 in TS 28.623 [10].

Following are the detailed SS to support management data collection based on Table 12.1.1.1.1-1 and Table 12.1.1.2.1-1 in TS 28.532 [2].

Table 6.1.4.2-1: SS to support management data collection

|  | Functionality | IS operations | HTTP Method | Resource URI |
| --- | --- | --- | --- | --- |
| Management Data Collection | Management data collection request | createMOI operation | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/ ManagementDataCollection |
|  | Management data collection deletion | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/ ManagementDataCollection={id} |
|  | Management data collection status query | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/ ManagementDataCollection={id} |

Note:	The ManagementDataCollection resource is defined in " TS28623_ManagementDataCollectionNrm.yaml ".

- YANG/Netconf-based solution set

- YANG/Netconf-based solution set for generic provisioning management service is defined in clause 12.1.3 in 3GPP TS 28.532 [2].

- YANG model "_3gpp-common-managementdatacollection.yang " in clause 4.4 in TS 28.623 [10].

==============Next change==============

6.5.4.2	Stage 3 definition

Following are the stage 3 definition for discovery of management data:

-	RESTful HTTP-based solution set

-	RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in TS 28.532 [2]. OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

-	OpenAPI document " TS28623_MnSRegistryNrm.yaml" in clause 4.3 in TS 28.623 [10].

Following are the detailed SS to support discovery of management data based on Table 12.1.1.1.1-1 and Table 12.1.1.1.2.1-1 in TS 28.532 [2].

Table 6.1.4.2-2: SS to support management data collection

|  | Functionality | IS operations | HTTP Method | Resource URI |
| --- | --- | --- | --- | --- |
| Management Data Registration | Register the management capability information for a set of management data | createMOI operation | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/MnsRegistry |
|  | Update the management capability information for a set of management data | modifyMOIAttributes operation | PUT PATCH | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/MnsRegistry/MgmtDataInfo={id} |
|  | Query a registered management capability information for a set of management data | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/MnsRegistry/ MgmtDataInfo={id} |
|  | Deregister the management capability information for a set of management data | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/MnsRegistry/ MgmtDataInfo={id} |
| Management Data Discovery | Query available management capabilities for a set of management data | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/MnsRegistry |
|  | Subscribe changes on available management capabilities for a set of management data | createMOI operation | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part} |
|  | Unsubscribe changes on available management capabilities for a set of management data | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/NtfSubscriptionControl={id} |
|  | Notify changes changes on available management capabilities for a set of management data | notifyMOIAttributeValueChanges notification | POST | {notificationTarget} |

Note:	The MnSRegistry and MgmtDataInfo resources are definedin "TS28623_MnSRegistryNrm.yaml", the NtfSubscriptionControl resource is defined in "TS28623_SubscriptionControlNrm.yaml".

==============End of change==============