3GPP TSG-SA5 Meeting #164 	S5-255651

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0542 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.422 Enhancement for continuous MDT on anonymization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The main procedure for Anonymization of MDT data for management based MDT in 5G are as follows: - Attribute “anonymizationOfMdtData” has been defined in TS 28.622 which specifies the level of anonymization of MDT data for management based activation, it will be sent in TraceJob which are specific for MDT or any combination of MDT. - If the anonymization indicates that the IMEI-TAC is required, The gNB should send the CELL TRAFFIC TRACE message to the AMF containing the privacy indicator IE based on Job Type. - AMF shall look up the IMEI-TAC and send to the TCE if the privacy indicator indicates logged MDT or immediate MDT. Continuous management-based MDT for gNB has been introduced in Rel-19, a new job Type named “IMMEDIATE_MDT_AND_ LOGGED_MDT” is added but support for anonymization of MDT data is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As RAN3 give principle said “Solutions for Continuous MDT shall not have an impact on the 5GC”. The solution is proposed to remove the word "only" for gNB "logged MDT" to extend the scope. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing anonymization of MDT data support to continuous management-based MDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE ***

4.7	Anonymization of MDT data for management based MDT

If the Job Type parameter indicates MDT, the anonymization requirements are applicable as described in this clause.

In case of UTRAN/E-UTRAN the anonymization of MDT data depends on the configuration parameter received at the MDT configuration. There are two levels of anonymization:

-	Using IMEI-TAC.

-	Not sending any identity to the TCE.

In case of NG-RAN the anonymization of MDT data depends on the configuration parameter received at the MDT configuration. There are two levels of anonymization:

-	Using IMEI-TAC.

-	Not sending any UE and subscriber identity to the TCE.

If the anonymization parameter value is not known or the anonymization indicates that no identity should be sent to the TCE:

-	For E-UTRAN the eNB should not send the CELL TRAFFIC TRACE message to the MME.

-	For UTRAN the RNC should not send the UPLINK INFORMATION EXCHANGE REQUEST message to the SGSN/MSC Server.

-	For NG-RAN the gNB should not send the CELL TRAFFIC TRACE message to the AMF.

If the anonymization indicates that the IMEI-TAC  is required:

-	For E-UTRAN eNB should send the CELL TRAFFIC TRACE message to the MME and shall put the following in the privacy indicator IE based on Job Type:

-	"immediate MDT" if the Job Type parameter indicates Immediate MDT.

-	"logged MDT" if the Job Type parameter indicates Logged MDT Only.

-	"logged MDT" if the Job Type parameter indicates Logged MBSFN MDT.

-	The gNB should send the CELL TRAFFIC TRACE message to the AMF and shall put the following in the privacy indicator IE based on Job Type:

-	"immediate MDT" if the Job Type parameter indicates Immediate MDT.

-	"logged MDT" if the Job Type parameter indicates Logged MDT .

MME shall look up of the IMEI-TAC and send to the TCE if the privacy indicator indicates logged MDT or Immediate MDT

-	For UTRAN the RNC shall send the UPLINK INFORMATION EXCHANGE REQUEST message to the SGSN/MSC Server with IMSI information. SGSN/MSC Server shall find the corresponding IMEI and send the IMEI-TAC to the TCE.

AMF shall look up the IMEI-TAC and send to the TCE if the privacy indicator indicates logged MDT or immediate MDT.

*** END OF CHANGE ***