3GPP TSG-SA5 Meeting #164	S5-255668

Dallas, USA, 17 - 21 November 2025									(revision of S5-255305)

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add potential solution for Estimation of NG-RAN EC per PLMN-ID granularity

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.1.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a potential solution for the following use case defined in clause 5.4.3 of TR 28.885.

-	Use case #3: Support estimation of EC and EE KPIs at per PLMN-ID granularity in Network sharing scenario

Proposed Changes

* * * First Change * * * *

##### 5.4.3.3.a	Potential solution #<a>: Estimation of NG-RAN EC per PLMN-ID granularity

###### 5.4.3.3.a.1	Introduction

This potential solution describes estimation of energy consumption of a NG-RAN at a per PLMN-ID granularity.

###### 5.4.3.3.a.2	Description

The gNB EC KPI defined in clause 6.7.3.4.2 of TS 28.554 [4] can be enhanced to optionally support reporting at PLMN-ID granularity as follows:

a)	EC_gNB_PLMNID

b)	This KPI describes the energy consumption of an NG-RAN at operator-specific, i.e., at a per PLMN-ID granularity. It is obtained using the of the PNF Energy Consumption measurement.

b-1)	Real, kilowatt-hours (kWh).

b-2)	RATIO

c)	EC_gNB_PLMNID =   PEE.Energy * W

Where,

-	PEE.Energy is PNF Energy consumption as defined in clause 5.1.1.19.3 of TS 28.552 [10].

-	W is the measurement for the weighted average, it is one of the following:

-	proportion of combined DL and UL data volume of the NR Cells of the gNB delivered for the particular PLMN, over the same observation period;

-	a weight defined by the consumer of KPI

Editor’s Note: The equation for W using DL and UL data volume is to be added.

d)	ManagedElement

e)	This KPI is applicable when a ManagedElement is used to represent the combined functionality of 3GPP defined gNB and designed to run on dedicated hardware, and the NG-RAN is shared in network sharing scenarios as described in 32.130 [11].

An example of the weight factor (W) defined by the consumer of the KPI can be based on the proportion of the NG-RAN resources shared between the PLMNs or based on the energy consumption of the gNB without traffic. These examples are not subject to standardization.

* * * End of Changes * * * *