3GPP TSG-SA5 Meeting #165	S5-260292

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0761 | rev | 1 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 enhance MRO related performance measurements to support CHO and LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | According to clause 15.5.2.2.2 In TS 38.300, for intra-system mobility, it states that: In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell. In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell. However, there is no PM of CHO MRO and LTM MRO in the corresponding performance measurement in MRO PM, which makes it impossible to evaluate the performance of MRO caused by CHO and LTM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add corresponding performance measurements to support MRO for CHO and LTM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MRO for CHO and LTM solution can not implement correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.25.x1(new), 5.1.1.25.x2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.1.25.1	Handover failures related to MRO for intra-system mobility

a)	This measurement provides the number of handover failure events related to MRO detected during the intra-system mobility within 5GS, see TS 38.300 [49] clause 15.5.2. The measurement includes separate counters for various handover failure types, classified as "Intra-system too early handover", "Intra-system too late handover" and "Intra-system handover to wrong cell".

b)	CC.

c)	The measurements of too early handovers, too late handovers and handover to wrong cell events are obtained respectively by accumulating the number of failure events detected by gNB during the intra-system mobility within 5GS.

d)	Each measurement is an integer value.

e)	HO.IntraSys.TooEarly

HO.IntraSys.TooLate

HO.IntraSys.ToWrongCell.

f)	NRCellCU

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

5.1.1.25.x1	Handover failures related to CHO MRO for intra-system mobility

a)	This measurement provides the number of handover failure events related to CHO MRO detected during the intra-system mobility within 5GS, see TS 38.300 [49] clause 15.5.2. The measurement includes separate counters for various handover failure types, classified as "Intra-system too early CHO Execution ", "Intra-system too late CHO Execution " and "Intra-system CHO Execution to wrong cell".

b)	CC.

c)	The measurements of too Late CHO Execution, too Early CHO Execution and CHO Execution to Wrong Cell events are obtained respectively by accumulating the number of failure events detected by gNB during the intra-system mobility within 5GS.

d)	Each measurement is an integer value.

e)	HO.IntraSys.CHOTooEarly

HO.IntraSys.CHOTooLate

HO.IntraSys.CHOToWrongCell.

f)	NRCellCU

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

Next Change

5.1.1.25.x2	Handover failures related to LTM MRO for intra-system mobility

a)	This measurement provides the number of handover failure events related to LTM MRO detected during the intra-system mobility within 5GS, see TS 38.300 [49] clause 15.5.2. The measurement includes separate counters for various handover failure types, classified as "Intra-system too early LTM cell switch", "Intra-system too late LTM cell switch" and "Intra-system LTM cell switch to wrong cell".

b)	CC.

c)	The measurements of too Late LTM cell switch, too Early LTM cell switch and LTM cell switch to Wrong Cell events are obtained respectively by accumulating the number of failure events detected by gNB during the intra-system mobility within 5GS.

d)	Each measurement is an integer value.

e)	HO.IntraSys.LTMTooEarly

HO.IntraSys.LTMTooLate

HO.IntraSys.LTMToWrongCell.

f)	NRCellCU

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

End of changes