3GPP TSG-SA5 Meeting #165	S5-260469

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1683 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.541 YANG corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | YANG SS is not matching the approved stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update YANG code to match existing stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 mismatch; interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge only |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2011 at commit ce74b0fd74736ef2c8f9d665283ef70cb775eded |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2011 at commit ce74b0fd74736ef2c8f9d665283ef70cb775eded

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-5g-common-yang-types.yang ***

<CODE BEGINS>

module _3gpp-5g-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5g-common-yang-types";

prefix "types5g3gpp";

import ietf-yang-types { prefix yang; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The model defines common types for 5G networks and

network slicing.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2026-01-25 { reference "CR-0060"; }  // common for R19, R20

revision 2025-11-07 { reference CR-1640 ; }

revision 2025-11-02 { reference CR-1658 ; }

revision 2025-07-25 { reference CR-1558 ; }

revision 2025-03-25 { reference CR-1489 ; }

revision 2024-11-01 { reference CR-1405; }

revision 2024-10-06 { reference CR-1389; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-05-10 { reference CR-0916; }

revision 2021-08-05 { reference S5-214053/CR-0518; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-20 { reference "Initial version."; }

grouping AreaScopeGrp {

description "This <<dataType>> defines an area scope.";

choice AreaScopeChoice {

case eutran-only { // choice 1

leaf-list eutraCellIdList {

type types3gpp:EutraCellId;

min-elements 1;

max-elements 32;

description "List of E-UTRAN cells identified by E-UTRAN-CGI";

}

}

case tac { // choice 3

leaf-list tacList {

type types3gpp:Tac;

min-elements 1;

max-elements 8;

description "Tracking Area Code list";

}

leaf-list cAGIdList {

type types3gpp:CagId;

max-elements 256;

description "It identifies a CAG list containing up to

256 CAG-identifiers per UE or up to 12 CAG-identifiers

per cell, see TS 38.331 [38]. CAG ID is used to combine

with PLMN ID to identify a PNI-NPN.AG ID is a hexadecimal

range with size 32 bit.";

}

}

case ntnGeoAreaList{ // choice 6

list ntnGeoAreaList {

must 'not(../nrCellIdList)';

description "geographical areas for NTN MDT";

key idx;

max-elements 8;

leaf idx { type string; }

uses types3gpp:GeoAreaGrp;

}

}

case else {

list nPNIdentityList { // choice 2, 4 , 5

description "list of NPN IDs of in NR. It is either

a list of PNI-NPNs identified by CAG ID with

associated plmn-Identity or a list of SNPN

identified by Network ID with associated plmn-Identity";

key idx;

min-elements 1;

uses types3gpp:NpnIdGrp;

leaf idx { type string; }

}

leaf-list nrCellIdList {

must 'not(../taiList)';

type types3gpp:NrCellId;

max-elements 32;

description "List of NR cells identified by NG-RAN CGI";

}

list taiList {

must 'not(../nrCellIdList)';

description "Tracking Area Identity list";

key idx;

max-elements 8;

leaf idx { type string; }

uses types3gpp:TaiGrp;

}

}

}

list sliceIdList {

description "Network Slice Id list";

key idx;

uses PLMNInfo;

min-elements 1;

max-elements 16384;

leaf idx { type string; }

}

}

grouping VendorSpecificFeatureGrp {

description "This <<dataType>> represents the information of vendor

specific feature as defined in clause 6.1.6.2.62 of 3GPP TS 29.510";

leaf featureName {

type string;

mandatory true;

config false;

description "It is a string representing a proprietary feature

specific to a given vendor.

It is recommended that the case convention for these strings is the

same as for enumerated data types (i.e. UPPER_WITH_UNDERSCORE;

see 3GPP TS 29.501, clause 5.1.1).";

}

leaf featureVersion {

type string;

mandatory true;

config false;

description "It is a string representing the version of the feature.";

}

}

grouping IpInterfaceGrp {

leaf-list ipv4EndpointAddresses {

description "Available endpoint IPv4 address(es) of

the User Plane interface.";

type inet:ipv4-address;

must '../ipv4EndpointAddresses or ../ipv6EndpointAddresses or ../fqdn';

}

leaf-list ipv6EndpointAddresses {

description "Available endpoint IPv6 address(es) of

the User Plane interface.";

type inet:ipv6-address;

must '../ipv4EndpointAddresses or ../ipv6EndpointAddresses or ../fqdn';

}

leaf fqdn {

description "This parameter defines FQDN of the Network Function

(See TS 23.003).";

type inet:domain-name;

must '../ipv4EndpointAddresses or ../ipv6EndpointAddresses or ../fqdn';

}

}

grouping SdRangeGrp {

leaf start {

type string {

pattern "[A-Fa-f0-9]{6}";

}

mandatory true;

description "First value identifying the start of an SD range.

This string shall be formatted as specified for the sd attribute of the

Snssai data type in clause 5.4.4.2 of TS 29.571.";

}

leaf end {

type string {

pattern "[A-Fa-f0-9]{6}";

}

mandatory true;

description "Last value identifying the end of an SD range.

This string shall be formatted as specified for the sd attribute of the

Snssai data type in clause 5.4.4.2 in TS 29.571";

}

}

grouping SnssaiExtensionGrp {

list sdRanges {

min-elements 1;

key "start end";

description "It shall contain the range(s) of Slice Differentiator values

supported for the Slice/Service Type value indicated in the sst

attribute of the Snssai data type (see clause 5.4.4.2 in TS 29.571).";

uses SdRangeGrp;

}

leaf wildcardSd {

type boolean;

default false;

description "It indicates that all SD values are supported for the

Slice/Service Type value indicated in the sst attribute of the Snssai

data type (see clause 5.4.4.2 in TS 29.571).";

}

}

grouping ExtSnssaiGrp {

list snssai {

description "It represents the S-NSSAI the NetworkSlice managed object

is supporting. The S-NSSAI is defined in TS 23.003 ";

min-elements 1;

max-elements 1;

key "sd sst";

uses SNssai;

}

list snssaiExtension {

description "It represents extensions to the Snssai.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses SnssaiExtensionGrp;

}

}

grouping sNssaiSmfInfoItem {

list sNssai {

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses SNssai;

}

list dnnSmfInfoList {

description "List of parameters supported by the SMF per DNN.

The absence indicates the DNN can be selected for any DNAI.";

min-elements 1;

key dnn;

uses DnnSmfInfoItem;

}

}

grouping MappedCellIdInfoGrp {

description "This data type represents the mapping relationship between

Mapped Cell IDs and geographical areas (see clause 16.14.5 of TS 38.300";

list ntnGeoArea {

description "This attribute indicates a specific geographical location

mapped to Mapped Cell ID(s).";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32; }

uses types3gpp:GeoAreaGrp;

}

list mappedCellId  {

description "This attribute is in format of NCGI to indicate a fixed

geographical area (See subclause 16.14.5 in TS 38.300)";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32; }

uses NcgiGrp;

}

}

grouping NcgiGrp {

description "Represents the Ncgi datatype";

list plmnId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:PLMNId ;

}

leaf nrCellId {

type string;

mandatory true;

description "This attribute represents NR Cell Identity.

It's a 36-bit string identifying an NR Cell Id as specified in

clause 9.3.1.7 of TS 38.413, in hexadecimal representation. Each

character in the string shall take a value of

'0' to '9', 'a' to 'f' or 'A' to 'F' and shall represent 4 bits.

The most significant character representing the 4 most significant

bits of the Cell Id shall appear first in the string, and the

character representing the 4 least significant bit of the

Cell Id shall appear last in the string.

Pattern: '^[A-Fa-f0-9]{9}$'

Example:

An NR Cell Id 0x225BD6007 shall be encoded as '225BD6007'.";

}

leaf nId {

type string;

mandatory true;

description "Network Identity; Shall be present if PlmnIdNid identifies

an SNPN (see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in

3GPP TS 23.501.";

}

}

typedef NRTAC {

type string;

description "This holds the identity of the common Tracking Area Code

for the PLMNs.

allowedValues:

a) It is the TAC or Extended-TAC.

b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300,

subclause 10.1.7 (PLMNID and TAC relation).

c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003

and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473.

d) For a 5G SA (Stand Alone), it has a non-null value.";

}

grouping SNssai {

description

"Single Network Slice Selection Assistance Information(S-NSSAI)";

reference "3GPP TS 23.003";

leaf sd {

description "Slice Differentiator

If not needed, the value can be set to ff:ff:ff.";

type yang:hex-string {

length 8;

}

reference "3GPP TS 23.003";

}

leaf sst {

type uint8;

description "Slice/Service Type.

Values 0 to 127 belong to standardized SST range and are defined in

3GPP TS 23.501. Values 128 to 255 belong to operator-specific range.";

}

}

grouping PlmnIdNid {

description "Represents the SCP domain specific information as defined

in TS 29.510 ";

uses types3gpp:PLMNId;

leaf nid {

type string;

description "This attribute represents network Identity;

Shall be present if PlmnIdNid identifies an SNPN.

(see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in TS 23.501";

}

}

grouping PLMNInfo {

description "The PLMNInfo data type define a S-NSSAI member in a specific

PLMNId, and it have two attributes PLMNId and S-NSSAI (PLMNId, S-NSSAI).

The PLMNId represents a data type that is comprised of mcc

(mobile country code) and mnc (mobile network code), (See TS 23.003

subclause 2.2 and 12.1) and S-NSSAI represents an data type, that is

comprised of an SST (Slice/Service type) and an optional

SD (Slice Differentiator) field";

uses types3gpp:PLMNId;

uses SNssai;

}

typedef CommModelType {

reference "3GPP TS 23501";

type enumeration {

enum DIRECT_COMMUNICATION_WO_NRF {

value 0;

description "Directly communicate to other pre-configured NF service.";

}

enum DIRECT_COMMUNICATION_WITH_NRF {

value 1;

description "Directly communicate to other NF service discovered

by NRF.";

}

enum INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY {

value 2;

description "Communicate to pre-configured other NF service through

SCP as a proxy.";

}

enum INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY {

value 3;

description "Communication to NF service discovered by NRF through SCP

as a proxy.";

}

}

}

grouping CommModel {

leaf groupId {

type uint16;

}

leaf commModelType {

type CommModelType;

}

leaf-list targetNFServiceList {

type types3gpp:DistinguishedName;

}

leaf commModelConfiguration {

type string;

}

}

grouping TaiRangeGrp {

list plmnId {

description "PLMN ID related to the TacRange.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list tacRangeList {

description "The range of the TACs.";

min-elements 1;

key "start end";

uses TacRange;

}

}

grouping TacRange {

leaf start {

description "First value identifying the start of a TAC range,

to be used when the range of TAC's can be represented

as a hexadecimal range (e.g., TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6}$)';

}

}

leaf end {

description "Last value identifying the end of a TAC range,

to be used when the range of TAC's can be represented as

a hexadecimal range (e.g. TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6})$';

}

}

leaf nRTACpattern {

description "Pattern (regular expression according to the ECMA-262)

representing the set of TAC's belonging to this range.

A TAC value is considered part of the range if and only if the

TAC string fully matches the regular expression.";

type string;

}

}

grouping DnnSmfInfoItem {

leaf dnn {

description "Supported DNN.";

mandatory true;

type string;

}

leaf-list dnaiList {

description "List of Data network access identifiers supported by

the SMF for this DNN. The absence of this attribute indicates that

the SMF can be selected for this DNN for any DNAI.";

min-elements 1;

type string;

}

}

grouping SupportedFunc {

leaf function {

type string;

}

leaf policy {

type string;

}

}

typedef EnergySavingLoadThresholdT {

type uint32 {

range 0..10000;

}

units 1/10000;

}

typedef EnergySavingTimeDurationT {

type uint32 {

range 0..900;

}

units seconds;

}

typedef PhysCellID {

type uint32 {

range "0..1007";

}

reference "clause 7.4.2 of TS 38.211";

}

typedef UTC24TimeOfDayT {

description "Time of day in HH:MM or H:MM 24-hour format per UTC

time zone.";

type string {

pattern "(([01]?[0-9])|(2[0-3])):([0-5][0-9])";

}

}

typedef DayOfWeekT {

type enumeration {

enum Monday;

enum Tuesday;

enum Wednesday;

enum Thursday;

enum Friday;

enum Saturday;

enum Sunday;

}

}

grouping CaraConfigurationGrp {

description "This data type represents the configuration used for mobile

NR node (e.g., IAB-node, MWAB-node) to perform certificate enrolment

procedure with Certification Authority server (CA/RA) as specified in

TS 28.315 clause 5.3";

leaf caraAddress {

type inet:host;

description "IP address or FQDN of the CMP (Certificate Management

Protocol) server";

}

leaf portNumber {

type inet:port-number;

description "This parameter specifies the port number used by

CMP (Certificate Management Protocol) server. The port for HTTP/HTTPSs

transfer of CMP messages is not explicitly given in RFC 9811, therefore

this parameter is required. The port number is usually

represented as 2 octets.";

}

leaf path {

type inet:uri;

mandatory true;

description "This parameter specifies the path (in ASCII string) to

the CMP server directory. A CMP server may be located in an

arbitrary path other than root.";

}

leaf subjectName {

type string;

mandatory true;

description "This parameter specifies the subject name (in ASCII

string) of the CA/RA. The use is described in 3GPP TS 33.310

clause 9.5.3.";

}

leaf protocol {

type enumeration {

enum HTTP;

enum HTTPS;

}

description "This parameter specifies the protocol (HTTP or HTTPS)

to be used for certificate enrolment. The use is described in

3GPP TS 33.310 clause 9.6.";

}

}

grouping MnrOamIPConfigGrp {

description "This data type includes the configutation for OAM connectivity

used for mobile NR node (e.g., IAB-node, MWAB-node) to establish

connection with

management system. The configuration attributes include:

Configuration of certification authority (CA/RA) server,

Configuration of security gateway (SeGW), and

Configuration of software configuration server (SCS)";

list caraConfiguration {

description "configuration used for mobile NR node (e.g., IAB-node,

MWAB-node) to perform certificate enrolment procedure as specified

in TS 28.315.";

uses CaraConfigurationGrp;

max-elements 1;

key caraAddress;

}

leaf seGwConfiguration {

description "this parameter specifies IP address or

FQDN of security gateway (SeGW) used for mobile NR

node (e.g., IAB-node, MWAB-node) to establish secure connection

as specified in TS 28.315.";

type inet:host;

}

leaf scsConfiguration {

description "this parameter specifies IP address or FQDN of

configuration server (SCS) used for mobile NR node

(e.g., IAB-node, MWAB-node) to establish connection as

specified in TS 28.315.";

type inet:host;

}

}

grouping LocationInfoGrp {

description "This data type contains location information

of mobile NR node (e.g., IAB-node, MWAB-node).";

leaf gNBId {

type int64 { range "0..4294967295"; }

description "It is either the gNB ID of the IAB-donor-CU that target

IAB-DU

connects to or the gNB Id of the IAB-nonor-CU that serves IAB-MT, or

the gNBId of the gNB that serves MWAB-UE.";

}

list pLMNId {

uses types3gpp:PLMNId;

max-elements 1;

description "The PLMN ID where IAB-MT or MWAB-UE is connected to";

key "mcc mnc";

}

leaf cellLocalId {

type int32 { range "0..16383"; }

description "Identifies an NR cell where IAB-MT or MWAB-UE

is connected to.";

}

leaf nRTAC {

type types5g3gpp:NRTAC;

description "It is TAC pertaining to the cells where IAB-MT or MWAB-UE is

connected.";

}

list tAI {

uses types3gpp:TaiGrp;

key idx;

leaf idx { type string; }

max-elements 1;

description "It is the TAI (see subclause 9.3.3.11 in TS 38.413)

pertaining to the cells where IAB-MT or MWAB-UE is connected";

}

list geoArea {

uses types3gpp:GeoAreaGrp;

key idx;

leaf idx {type string;}

max-elements 1;

description "It specifies geographical area of mobile NR node

(e.g., IAB-node or MWAB-node).";

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** yang-models/_3gpp-5gc-nrm-managed-nfprofile.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-managed-nfprofile {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-managed-nfprofile;

prefix mnfp3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-5gc-nrm-nfservice { prefix nfs3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "NF profile class.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 29.510";

revision 2026-01-25 { reference "CR-0060"; }  // common for R19, R20

revision 2025-07-25 { reference CR-1558 ; }

revision 2025-07-25 { reference "initial revision"; }

grouping SPNInfoIdGrp {

description "This data type represents the SNPN identification

consisting of MCC, MNC, and optionally NID.";

leaf mcc {

type string;

description "Mobile Country Code (MCC) of the SNPN.";

}

leaf mnc {

type string;

description "Mobile Network Code (MNC) of the SNPN.";

}

leaf nId {

type string;

description "Network Identifier (NID) of the SNPN.";

}

}

grouping CollocatedNfInstanceGrp {

description "This data type represents the SNPN identification

consisting of MCC, MNC, and optionally NID.";

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

mandatory true;

type string;

}

leaf nfType {

description "Type of Network Function.";

mandatory true;

type types3gpp:NfType;

}

}

grouping SupiRangeGrp {

leaf start {

description "First value identifying the start ofa SUPI range.To be

used when the range of SUPI's can be represented as a numeric range

(e.g., IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of a SUPI range.To be

used when the range of SUPI's can be represented as a numeric range

(e.g. IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of SUPI's belonging to

this range.A SUPI value is considered part of the range if and

only if the SUPI string fully matches the regular expression.";

type string;

}

}

grouping ConditionItemGrp {

description "This data type represents a single condition item that

shall be evaluated to determine whether a discovered NF (Service)

Instance shall be selected.";

leaf consumerNfTypes {

description "It represents the NF types of the consumers for

which the conditions included in this ConditionItem apply.

If this attribute is absent, the conditions are applicable

to all NF consumer types.";

type types3gpp:NfType;

}

leaf serviceFeature {

description "It represents a feature number of that NF Service

Instance,under CANARY_RELEASE status. This attribute only

applies when the          selectionConditions, where this

ConditionItem is included, is included in a NF Service Instance

This condition is evaluated to <true> when the service requests

froma consumer of this NF Service Instance require the support

of the indicated feature on the NF Service Instance.";

type uint32; // positive integer, 0 excluded by context

}

leaf vsServiceFeature {

description "It represents a Vendor-Specific feature

number of thatNF Service Instance, under CANARY_RELEASE

status. This attribute only applies when the

selectionConditions, where this ConditionItem is included,

is included in a NF Service Instance.This condition is

evaluated to true when the service requests from a

consumer of this NF Service Instance require the support

of the indicated Vendor-Specific feature on the NF

Service Instance.";

type uint32;

}

list supiRangeList {

description "It represents a set of SUPIs for which

the NF (Service) instance under CANARY_RELEASE status shall

be selected.";

key idx;

leaf idx { type uint32; }

uses SupiRangeGrp;

}

list gpsiRangeList {

description "It represents a set of GPSIs for which the NF

(Service)

instance under CANARY_RELEASE status shall be selected";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses IdentityRange;

}

list impuRangeList {

description "It represents a set of IMS Public Identities

for which the NF (Service) instance under CANARY_RELEASE

status shall be selected";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses IdentityRange;

}

list impiRangeList {

description "It represents a set of IMS Private Identities

for which theNF (Service) instance under

CANARY_RELEASE status shall be selected.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses IdentityRange;

}

leaf peiList {

description "It represents a set of PEIs of the UEs for

which the NF(Service) instance under CANARY_RELEASE status

shall be selected";

type string;

}

list taiRangeList {

description "It represents a set of TAIs where the

NF(Service) instance under CANARY_RELEASE status shall

be selected for a certain UE";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses types5g3gpp:TaiRangeGrp;

}

leaf dnnList {

description " It represents a set of TAIs where the NF

(Service) instance under CANARY_RELEASE status shall be

selected for a certain UE";

type string;

}

}

grouping PlmnSnssai {

list plmnId {

description "PLMN ID for which list of supported S-NSSAI(s)

is provided.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssaiList {

description "The specific list of S-NSSAIs supported

by the given PLMN.";

min-elements 1;

max-elements 1;

key "sst sd";

uses types5g3gpp:SNssai;

}

leaf nid {

description "Network Identity; Shall be present if PlmnIdNid

identifies an SNPN ";

reference "see clauses 5.30.2.3, 5.30.2.9, 6.3.4,

and 6.3.8 in 3GPP TS 23.501 ";

type string;

}

}

grouping ConditionGroupGrp {

description "This <<choice>> represents a group of conditions that

shall be evaluated to determine whether a discovered NF (Service)

Instanceshall be selected. (See clause 6.1.6.2.125 TS 29.510 [23]).";

choice LogicalOperator{

case and {

list and {

description "It represents a list of conditions where the

overall evaluation is true only if all the conditions

in the list are evaluated as true.";

key idx;

leaf idx { type uint32; }

uses SelectionConditionsGrp;

}

}

case or{

list or {

description "It represents a list of conditions where the

overall evaluation is true if at least one of the conditions

in the list evaluated as true";

key idx;

leaf idx { type uint32; }

uses SelectionConditionsGrp;

}

}

}

}

grouping SelectionConditionsGrp {

description "This data type represents the list of conditions that

shall be selected by an NF Service Consumer.";

reference "(See clause 6.1.6.2.123 TS 29.510 [23])";

list conditionItem{

key "id";

description "It represent a single condition item that shall be

evaluated Instance shall be selected.";

leaf id{

type string;

}

uses ConditionItemGrp;

}

// circular dependency for grouping "ConditionGroupGrp" introduced in

//stage 2

//    container conditionGroup{

//        description "It represents a group of conditions

//        that shall be evaluated";

//        uses ConditionGroupGrp;

//        }

}

grouping IdentityRange {

leaf start {

description "First value identifying the start of an identity range.

To be used when the range of identities can be represented

as a numeric range (e.g., MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {type string; } // dummy

leaf pattern {type string; } // dummy

}

grouping ManagedNFProfileGrp {

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

mandatory true;

type string;

}

leaf nfType {

description "Type of Network Function.";

mandatory true;

type types3gpp:NfType;

}

leaf nfStatus {

description "Status of the NF Instance.";

mandatory true;

type NFStatus;

}

list collocatedNfInstances {

description "It represents information related to collocated NF

type(s) and corresponding NF Instances when the NF is collocated

with NFs supporting other NF types.";

key "nfInstanceId";

uses  CollocatedNfInstanceGrp;

}

leaf nfInstanceName {

description "It represents human readable name of the NF Instance";

type string;

}

leaf heartBeatTimer {

description "Time in seconds expected between 2 consecutive

heart-beat messages from an NF Instance to the NRF.

It may be included in the registration request.

When present in the request it shall contain the

heartbeat time proposed by the NF service consumer.";

type uint16;

}

list plmnList {

description "PLMN(s) of the Network Function.

This IE shall be present if this information

is available for the NF. If not provided, PLMN ID(s)

of the PLMN of the NRF are assumed for the NF.";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssais {

description "S-NSSAIs of the Network Function. If not

provided, the NF can serve any S-NSSAI. When present

this IE represents the list of S-NSSAIs supported in

all the PLMNs listed in the plmnList IE.";

min-elements 1;

key "sst sd";

uses types5g3gpp:SNssai;

}

leaf fqdn {

description "FQDN of the Network Function. For AMF, the

FQDN registered with the NRF shall be that of the AMF Name.";

type inet:domain-name;

}

leaf hostAddr {

mandatory true;

type inet:host ;

description "Host address of a NF";

}

leaf authzInfo {

description "This parameter defines NF Specific Service authorization

information. It shall include the NF type (s) and NF realms/origins

allowed to consume NF Service(s) of NF Service Producer ";

reference "TS 23.501";

type string;

}

list allowedPlmns {

description "PLMNs allowed to access the NF instance.

If not provided, any PLMN is allowed to access the NF.";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNPNList {

key "mcc mnc nId";

description "SNPN(s) of the Network Function. This attributeIE shall be

present if the NF pertains to one or more SNPNs.";

reference "TS 29.510";

uses SPNInfoIdGrp;

}

list perPlmnSnssaiList {

description "It includes the S-NSSAIs supported by the Network Function

for each PLMN supported by the Network Function. When present, it shall

override sNssais IE.If the perPlmnSnssaiList attribute is provided in

at least one NF Service, the S-NSSAIs supported per PLMN in the

NF Profile shall be the set or a superset of the perPlmnSnssaiList

of the NFService(s).";

key idx;

leaf idx { type uint32; }

uses PlmnSnssai;

}

list allowedSNPNs {

description "SNPNs allowed to access the NF instance.

The absence of this attribute in the NF profile indicates that no SNPN,

other than the SNPN(s) registered in the snpnList attribute of the

NF Profile, is allowed to access the service instance";

key "mCC mNC";

uses nfs3gpp:SNPNIdGrp;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the NF instance.

If not provided, any NF type is allowed to access the NF.";

min-elements 1;

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern (regular expression according to the ECMA-262

dialect [75]) representing the NF domain names within the PLMN of

the NRF allowed to access the NF instance If not provided,

any NF domain is allowed to access the NF.";

type string;

}

list allowedNSSAIs {

description "S-NSSAI of the allowed slices to access the NF instance.

If not provided, any slice is allowed to access the NF.";

key "sst sd";

uses types5g3gpp:SNssai;

}

//    list allowedRuleSet {

//      description "S-NSSAI of the allowed slices to access the NF instance.

//        If not provided, any slice is allowed to access the NF.";

//      key "sst sd";

//      uses RuleSet;?

//    }

leaf locality {

description "Operator defined information about the location

of the NF instance (e.g. geographic location, data center).";

type string;

}

leaf extLocality {

description "It indicates the operator defined information about the

location of the NF instance. The key of the map shall be a (unique)

valid JSON string per clause 7 of IETF RFC 8259 [92], with a maximum

of 32 characters, representing a type of locality as defined in

clause 6.1.6.3.18.";

type string;

}

leaf capacity {

description "Static capacity information in the range of 0-65535,

expressed as a weight relative to other NF instances of

the same type; if capacity is also present in the nfServiceList

parameters, those will have precedence over this value.";

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100,

indicates the current load percentage of the NF.";

type types3gpp:Load;

}

leaf loadTimeStamp {

description "It indicates the point in time in which the latest load

information (sent by the NF in the load attribute of the NF Profile)

was generated at the NF Instance.If the NF did not provide a timestamp,

the NRF should set it to the instant when the NRF received the message

where the NF provided the latest load information.";

type yang:date-and-time ;

}

leaf-list recoveryTime {

description "Timestamp when the NF was (re)started. The NRF shall notify

NFs subscribed to receiving notifications of changes of the NF profile,

if the NF recoveryTime is changed";

type yang:date-and-time;

}

leaf nfServicePersistence {

description "This parameter indicates whether the different service

instances of a same NF Service in the NF instance, supporting a same

API version, are capable to persist their resource state in shared

storage and therefore these resources are available after a new NF

service instance supporting the same API version is selected by a NF

Service Consumer ";

reference "TS 29.510";

type boolean;

}

leaf-list nfSetIdList {

type string ;

min-elements 1;

description "A NF Set Identifier is a globally unique identifier of

a set of equivalent and interchangeable CP NFs from a given network

that provide distribution, redundancy and scalability

(see clause 5.21.3 of 3GPP TS 23.501 [2]).An NF Set Identifier shall

be constructed from the MCC, MNC, NID (for SNPN), NF type and a Set ID.

A NF Set Identifier shall be formatted as the following string:

set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC> for a NF Set in a PLMN, or

set<Set ID>.<nftype>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC> for a NF Set

in a SNPN.At most one NF Set ID shall be indicated per PLMN-ID or SNPN

of the NF.";

}

leaf-list nfProfileChangesSupportInd {

description "This parameter indicates if the NF Service Consumer supports

or does not support receiving NF Profile Changes. It may be present in

the NFRegister or NFUpdate (NF Profile Complete Replacement) request and

shall be absent in the response (see Annex B 3GPP TS 29.510 [23]).  ";

max-elements 1;

type boolean;

}

leaf-list nfProfilePartialUpdateChangesSupportInd {

description "It represents NF Profile Partial Update Changes Support

Indicator. TRUE: the NF Service Consumer supports receiving NF Profile

Changes in the response to an NF Profile Partial Update operation.

FALSE (default): the NF Service Consumer does not support receiving

NF Profile Changes in the response to an NF Profile Partial Update

operation";

max-elements 1;

type boolean;

}

leaf-list nfProfileChangesInd {

description "NF Profile Changes Indicator. This IE shall be absent

in the request to the NRF and may be included by the NRF

in NFRegister or NFUpdate (NF Profile Complete Replacement) response.

true: the NF Profile contains NF Profile changes.

false (default): complete NF Profile.";

type boolean;

max-elements 1;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.

This attribute may contain multiple default subscriptions for a same

notification type; in that case, those default subscriptions are used

as alternative notification endpoints.";

key notificationType;

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

leaf servingScope {

description "This parameter indicates the served geographical areas

of a NF instance.";

type string;

}

leaf-list lcHSupportInd {

description "This parameter indicates whether the NF supports or does

not support Load Control based on LCI Header.";

reference "clause 6.3 of 3gpp TS 29.500";

max-elements 1;

type boolean;

}

leaf-list olcHSupportInd {

description "This parameter indicates whether the NF supports or does

not support Overload Control based on OCI Header .";

reference "clause 6.4 of 3gpp TS 29.500";

max-elements 1;

type boolean;

}

leaf-list nfSetRecoveryTimeList {

description "This parameter contains the recovery time of NF Set(s)

indicated by the NfSetId, where the NF instance belongs.";

type yang:date-and-time ;

max-elements 1;

}

leaf-list serviceSetRecoveryTimeList {

description "This parameter contains the recovery time of NF Service

Set(s) configured in the NF instance, which are indicated by the

NfServiceSetId";

type yang:date-and-time ;

max-elements 1;

}

leaf scpDomains {

description "This parameter shall carry the list of SCP domains the

SCP belongs to, or the SCP domain the NF (other than SCP) or the SEPP

belongs to..";

type string;

}

leaf vendorId {

description "Vendor ID of the NF instance, according to the

IANA-assigned SMI Network Management Private Enterprise Codes [77].

allowedValues: 6 decimal digits; if the SMI code has less than

6 digits, it shall be padded with leading digits 0 to complete

a 6-digit string value.";

type string;

}

leaf hniList {

description "Identifications of Credentials Holder or Default Credentials

Server. It is an array of FQDN.";

type string;

}

leaf-list interPlmnFqdn {

description "If the NF needs to be discoverable by other

NFs in a different PLMN, then an FQDN that is used

for inter-PLMN routing is specified.";

type inet:domain-name;

}

list nfServices {

description "List of NF Service Instances. It shall include

the services produced by the NF that can be

discovered by other NFs.";

key serviceInstanceID;

min-elements 1;

uses nfs3gpp:NFServiceGrp;

}

list supportedVendorSpecificFeatures {

description "It indicates a map of Vendor-Specific features, where the

key of the map is the IANA-assigned SMI Network Management Private

Enterprise Codes and the value of each entry of the map shall be

a list (array) of VendorSpecificFeature objects as defined

in the clause 5.3.247.";

key "enterpriseCode";

leaf enterpriseCode {

type uint32;

description "IANA-assigned SMI Network Management Private

Enterprise Code";

}

list vendorSpecificFeature {

key "featureName";

config false;

description "VendorSpecificFeature objects as defined

in the clause 5.3.247.";

uses types5g3gpp:VendorSpecificFeatureGrp;

}

}

list selectionConditions {

description "Identifications of Credentials Holder or Default Credentials

Server. It is an array of FQDN.";

leaf idx { type uint32; }

key "idx";

uses SelectionConditionsGrp;

}

leaf-list canaryRelease {

description "This attribute indicates whether an NF instance whose

nfStatus is set to REGISTERED is in Canary Release condition,

i.e. it should only be selected by NF Service Consumers under the

conditions indicated by the selectionConditions attribute.

True: the NF is under Canary Release condition, even if the

nfStatus is set to REGISTERED

False: the NF instance indicates its Canary Release condition

via the nfStatus attribute";

max-elements 1;

type boolean;

}

leaf-list exclusiveCanaryReleaseSelection {

description "This attribute indicates whether an NF Service

Consumer should only select an NF Service Producer in Canary

Release condition.

True: the consumer shall only select producers in Canary Release

condition

False: the consumer may select producers not in Canary Release

condition";

max-elements 1;

type boolean;

}

leaf sharedProfileDataId {

description "This attribute indicates a string uniquely identifying

Shared Profile Data. The format of the sharedProfileDataId shall be

a Universally Unique Identifier (UUID) version 4, as described in

IETF RFC 4122 [44]. The hexadecimal letters should be formatted as

lower-case characters by the sender, and they shall be handled as

case-insensitive by the receiver.

Example: 4ace9d34-2c69-4f99-92d5-a73a3fe8e23b";

type string;

}

leaf-list shutdownTime {

description "It indicates the timestamp when the NF Instance is

planned to be shut down. This attribute may be present if the

nfStatus is set to UNDISCOVERABLE due to scheduled shutdown";

type yang:date-and-time ;

max-elements 1;

}

leaf supportedRcfs {

description "It represents a list of Resource Content Filter IDs";

type string;

}

leaf canaryPrecedenceOverPreferred {

description "ManagedNFProfile.canaryPrecedenceOverPreferred  This

attribute indicates whether the NRF shall prioritize the NF Service

Producer in Canary Release condition over the preferences

(preferred-xxx, ext-preferred-xxx) present in NF discovery requests.

allowedValues:

True NRF shall prioritize NF Service Producers in Canary Release

condition at NF discovery requests, i.e. NF Service Producers

determined according to preferred-xxx and/or ext-preferred-xxx

shall be prioritized after the NF Service Producers in Canary

Release condition. The associated NF (service) priorities for Service

Producers in Canary Release condition shall not be modified by NRF.

False NRF shall prioritize the NF Service Producers according to

preferred-xxx and/or ext-preferred-xxx (i.e. Canary Release condition

in NF Service Producers shall not be prioritized over NF Service

Consumer preferences at NF discovery requests)";

type boolean;

}

typedef NFStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 2 ***

*** START OF CHANGE 3 ***

*** yang-models/_3gpp-5gc-nrm-nfservice.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nfservice {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfservice;

prefix nfs3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-yang-types { prefix yang; }

import ietf-inet-types { prefix inet; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "NF service class.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 29.510";

revision 2026-01-25 { reference "CR-0060"; }  // common for R19, R20

revision 2025-01-25 { reference CR-1442; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2021-01-25 { reference CR-0454 ; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-06-17 { reference "initial revision"; }

grouping NFServiceGrp {

description "Represents the NFService IOC";

leaf serviceInstanceID {

description

"Unique ID of the service instance within a given NF Instance.";

mandatory true;

type string;

}

leaf serviceName {

description "Name of the service instance (e.g. 'nudm-sdm').";

mandatory true;

type ServiceName;

}

list versions { //check in review if key is ok (unique)

description "API versions supported by the NF Service and if available,

the corresponding retirement date of the NF Service.";

min-elements 1;

key "apiVersionInUri apiFullVersion";

uses NFServiceVersion;

}

leaf scheme {

description "URI scheme (e.g. 'http', 'https').";

mandatory true;

type UriScheme;

}

leaf nfServiceStatus {

description "Status of the NF Service Instance.";

mandatory true;

type NFServiceStatus;

}

leaf fqdn {

description "FQDN of the NF Service Instance.";

//optional support

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF service needs to be discoverable by other NFs in a

different PLMN, then an FQDN that is used for inter PLMN routing.";

//optional support

type inet:domain-name;

}

list ipEndPoints {

description "IP address(es) and port information of the Network Function

(including IPv4 and/or IPv6 address)where the service is listening

for incoming service requests.";

//optional support

key idx;

leaf idx {

type string;

}

min-elements 1;

uses ipEndPoint;

}

leaf apiPrefix {

description "Optional path segment(s) used to construct the {apiRoot}

variable of the different API URIs.";

//optional support

type string;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

//optional support

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

list allowedPLMNs {

description "PLMNs allowed to access the service instance.

The absence of this attribute indicates that any PLMN is allowed to

access the service instance.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

list allowedSnpns {

description "PLMNs allowed to access the service instance.

The absence of this attribute indicates that any PLMN is allowed to

access the service instance.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "SNPNs allowed to access the NF instance";

min-elements 1;

//optional support

type  types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed to

access the service instance.";

//optional support

min-elements 1;

type string;

}

list allowedNSSAI {

description "S-NSSAI of the allowed slices to access the service

instance. The absence of this attribute indicates that any slice

is allowed to access the service instance.";

min-elements 1;

//optional support

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf-list allowedOperationsPerNfType {

description "It indicates the allowed operations on resources for

each type of NF; the key of the map is the NF Type, and the value

is an array of scopes";

//optional support

type string;

min-elements 1;

}

leaf-list allowedOperationsPerNfInstance {

description "It indicates the allowed operations on resources for

a given NF Instance; the key of the map is the NF Instance Id, and

the value is an array of scopes";

//optional support

type string;

min-elements 1;

}

leaf allowedOperationsPerNfInstanceOverrides {

type boolean;

default false;

description "When it is present and set to true, indicates that

the scopes defined in attribute allowedOperationsPerNfInstance

for a given NF Instance ID take precedence over the scopes defined

in attribute allowedOperationsPerNfType for the corresponding

NF type of the NF Instance associated to such NF Instance ID..";

}

leaf-list oauth2Required {

type boolean;

max-elements 1;

description "It indicates whether the NF Service Instance requires

Oauth2-based authorization";

}

leaf-list sharedServiceDataId {

type string;

max-elements 1;

description "String uniquely identifying SharedServiceData. The format

of the sharedServiceDataId shall be a Universally Unique Identifier

(UUID) version 4, as described in IETF RFC 4122 [44]. The hexadecimal

letters should be formatted as lower-case characters by the sender,

and they shall be handled as case-insensitive by the receiver.";

}

leaf priority {

description "Priority (relative to other services of the same type)

in the range of 0-65535, to be used for NF Service selection; lower

values indicate a higher priority.";

//optional support

type uint16 { range "0..65535"; }

}

leaf capacity {

description "Static capacity information in the range of 0-65535,

expressed as a weight relative to other services of the same type.";

//optional support

type uint16;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

//optional support

type yang:date-and-time;

}

leaf vendorId {

description "Vendor ID of the NF instance";

//optional support

type string;

}

}

typedef SupportedFeatures {

type string {

pattern '[A-Fa-f0-9]*';

}

}

list supportedVendorSpecificFeatures {

description "It indicates a map of Vendor-Specific features, where the

key of the map is the IANA-assigned SMI Network Management Private

Enterprise Codes and the value of each entry of the map shall be

a list (array) of VendorSpecificFeature objects as defined

in the clause 5.3.247.";

key "enterpriseCode";

leaf enterpriseCode {

type uint32;

description "IANA-assigned SMI Network Management Private

Enterprise Code";

}

list vendorSpecificFeature {

key "featureName";

config false;

description "VendorSpecificFeature objects as defined

in the clause 5.3.247.";

uses types5g3gpp:VendorSpecificFeatureGrp;

}

}

grouping ipEndPoint {

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

}

leaf transport {

type TransportProtocol;

}

leaf port {

type uint16;

}

}

typedef TransportProtocol {

type enumeration {

enum TCP;

enum STCP;

enum UDP;

}

}

grouping NFServiceVersion {

leaf apiVersionInUri {

mandatory true;

type string;

}

leaf apiFullVersion {

mandatory true;

type string;

}

leaf expiry {

//optional to support

type yang:date-and-time;

}

}

typedef ServiceName {

type enumeration {

enum NNRF_NFM;

enum NNRF_DISC;

enum NUDM_SDM;

enum NUDM_UECM;

enum NUDM_UEAU;

enum NUDM_EE;

enum NUDM_PP;

enum NAMF_COMM;

enum NAMF_EVTS;

enum NAMF_MT;

enum NAMF_LOC;

enum NSMF_PDUSESSION;

enum NSMF_EVENT-EXPOSURE;

enum NAUSF_AUTH;

enum NAUSF_SORPROTECTION;

enum NNEF_PFDMANAGEMENT;

enum NPCF_AM-POLICY-CONTROL;

enum NPCF_SMPOLICYCONTROL;

enum NPCF_POLICYAUTHORIZATION;

enum NPCF_BDTPOLICYCONTROL;

enum NPCF_EVENTEXPOSURE;

enum NPCF_UE_POLICY_CONTROL;

enum NSMSF_SMS;

enum NNSSF_NSSELECTION;

enum NNSSF_NSSAIAVAILABILITY;

enum NUDR_DR;

enum NLMF_LOC;

enum N5G_EIR_EIC;

enum NBSF_MANAGEMENT;

enum NCHF_SPENDINGLIMITCONTROL;

enum NCHF_CONVERGEDCHARGING;

enum NNWDAF_EVENTSSUBSCRIPTION;

enum NNWDAF_ANALYTICSINFO;

}

}

typedef UriScheme {

type enumeration {

enum HTTP;

enum HTTPS;

}

}

typedef NFServiceStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

enum UNDISCOVERABLE;

}

}

grouping ChfServiceInfo {

leaf primaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a

secondary CHF instance of another primary CHF service instance.";

//conditional to support

type string;

}

leaf secondaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a

primary CHF instance of another secondary CHF service instance.";

//conditional to support

type string;

}

}

grouping SNPNIdGrp {

description "This <<dataType>> represents the information of a SNPN

identification";

leaf mCC {

description "This is the Mobile Country Code (MCC) of the PLMN

identifier. See TS 23.003 [13] subclause 2.2 and 12.1";

type string;

}

leaf mNC {

description "This is the Mobile Network Code (MNC) of the PLMN

identifier. See TS 23.003 [13] subclause 2.2 and 12.1.";

type string;

}

leaf nId {

description "Network Identity; Shall be present if PlmnIdNid

identifies an SNPN";

type string;

}

}

}

<CODE ENDS>

*** END OF CHANGE 3 ***

*** START OF CHANGE 5 ***

*** yang-models/_3gpp-nr-nrm-ntnfunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-ntnfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-nr-nrm-ntnfunction;

prefix ntn3gpp;

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-yang-types { prefix yang; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Implements support the C-SON function of Capacity and

Coverage optimization .

Copyright 2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2026-02-02 { reference "CR-1682 CR-1683"; } //Common in r19, r20

revision 2025-03-27 { reference CR-1512; }

revision 2025-02-17 { reference CR-1479 ; }

revision 2024-05-24 { reference CR-1273 ; }

feature NTNFunctionUnderSubNetwork {

description "The NTNFunction shall be contained under SubNetwork";

}

feature NTNFunctionUnderManagedElement {

description "The NTNFunction shall be contained under ManagedElement";

}

grouping PositionVelocityGrp {

description "This data type defines configuration parameters to support

satellite position and velocity state";

leaf positionX {

type uint32 {

range 0..604800;

}

default 0;

units meter;

description "X, Y, Z coordinate of satellite position state vector

in ECEF. Unit is meter.

Step of 1.3 m. Actual value = field value * 1.3.";

}

leaf positionY {

type uint32 {

range 0..604800;

}

default 0;

units meter;

description "X, Y, Z coordinate of satellite position state vector

in ECEF. Unit is meter.

Step of 1.3 m. Actual value = field value * 1.3.";

}

leaf positionZ {

type uint32 {

range 0..604800;

}

default 0;

units meter;

description "X, Y, Z coordinate of satellite position state vector

in ECEF. Unit is meter.

Step of 1.3 m. Actual value = field value * 1.3.";

}

leaf velocityVX {

type int32 {

range -131072..131071;

}

default 0;

units meter/second;

description "X, Y, Z coordinate of satellite velocity state vector

in ECEF.

Step of 0.06 m/s. Actual value = field value * 0.06.";

}

leaf velocityVY {

type int32 {

range -131072..131071;

}

default 0;

units meter/second;

description "X, Y, Z coordinate of satellite velocity state vector

in ECEF.

Step of 0.06 m/s. Actual value = field value * 0.06.";

}

leaf velocityVZ {

type int32 {

range -131072..131071;

}

default 0;

units meter/second;

description "X, Y, Z coordinate of satellite velocity state vector

in ECEF.

Step of 0.06 m/s. Actual value = field value * 0.06.";

}

}

grouping OrbitalGrp {

description "This data type defines configuration parameters of orbital

trajectory information to support satellite access.";

leaf semiMajorAxis {

type uint64 {

range 0..8589934591;

}

default 0;

units meter;

description "Satellite orbital parameter: semi major axis alpha,

see NIMA TR 8350.2.

Step of 4.249 * 10**-3 m.

Actual value = 6500000 + field value * (4.249 * 10**-3).";

}

leaf eccentricity {

type int32 {

range -524288..524287;

}

default 0;

description "Satellite orbital parameter: eccentricity e,

see NIMA TR 8350.2.

Step 1.431 * 10**-8.

Actual value = field value * (1.431 * 10**-8).";

}

leaf periapsis {

type uint32 {

range 0..16777215;

}

default 0;

units radian;

description "Satellite orbital parameter: argument of periapsis omega,

see NIMA TR 8350.2.

Step of 2.341* 10**-8 rad.

Actual value = field value * (2.341* 10**-8).";

}

leaf longitude {

type uint32 {

range 0..2097151;

}

default 0;

units radian;

description "Satellite orbital parameter: longitude of ascending node

OMEGA, see NIMA TR 8350.2.

Step of 2.341* 10**-8 rad.

Actual value = field value * (2.341* 10**-8).";

}

leaf inclination {

type int32 {

range -524288..524287;

}

default 0;

units radian;

description "Satellite orbital parameter: inclination i,

see NIMA TR 8350.2.

Step of 2.341* 10**-8 rad.

Actual value = field value * (2.341* 10**-8).";

}

leaf meanAnomaly {

type uint32 {

range 0..16777215;

}

default 0;

units radian;

description "Satellite orbital parameter: Mean anomaly M at epoch time,

see NIMA TR 8350.2.

Step of 2.341* 10**-8 rad.

Actual value = field value * (2.341* 10**-8).";

}

}

grouping EphemerisGrp {

description "This data type represents the satellite ephemeris related

information. The ephemeris data format may be expressed either in

format of position and velocity state vector or in format of orbital

parameters.";

leaf satelliteId {

type string {

pattern "00([01][0-9][0-9])|(2[0-4][0-9]|(25[0-5]))";

}

description "This attribute indicates satellite Id.number.

It shall be formatted as a fixed 5-digit string, padding with

leading digits '0' to complete a 5-digit length.

allowedValues: 00000..00255";

}

leaf epochTime {

type yang:date-and-time;

description "It defines the ephemeris reference time.";

}

choice positionVelocity-or-orbital {

list positionVelocity {

description "It indicates ephemeris is in format of NTN payload

position and velocity state vectors.";

max-elements 1;

key idx;

leaf idx { type uint32; }

uses PositionVelocityGrp;

}

list orbital {

description "It indicates ephemeris is in orbital parameter ephemeris

format, as specified in NIMA TR 8350.2";

max-elements 1;

key idx;

leaf idx { type uint32; }

uses OrbitalGrp;

}

}

}

grouping EphemerisInfoSetGrp {

description "Represents the EphemerisInfoSet IOC";

list EphemerisInfos {

description "This is the list of Ephemeris related information.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses EphemerisGrp;

}

}

grouping NTNEntityConfigGrp {

description "The attribute nTNConfEntity specifies an NTN related MOI.

The attribute nTNConfList optionally specifies configuration

updates to be applied to the nTNConfEntity.  If no updates are provided

the value of nTNConfEntity is used without modification.";

leaf nTNConfEntity {

type types3gpp:DistinguishedName;

description "Specifies the DN of a specific NTN related MOI.

allowedValues:  DN of of the following MOIs:

EP_NgC, NRCellCU, NRCellDU, NRSectorCarrier,

SectorEquipmentFunction, NRCellRelation.";

}

list nTNConfigList {

uses types3gpp:nameValuePair;

key "name";

description "Specifies the list of configuration parameters and values.

The content of the attribute is a list of attributeName- attributeValue

pairs.  AttributeValues may be complex types.";

}

}

grouping NTNTimeBasedConfigGrp {

description "Define configuration to be applied during a specified time

window.

The attribute timeWindow specifies the time window during which the

specified configuration is applied.

The attribute nTNEntityConfigList specifies configuration updates applied

during the specified timeWindow.";

list timeWindow {

key idx;

leaf idx { type uint32 ; }

min-elements 1;

max-elements 1;

uses types3gpp:TimeWindowGrp;

description "It defines the time window.";

}

list nTNEntityConfigList {

key idx;

leaf idx { type uint32 ; }

uses NTNEntityConfigGrp;

description "It contains a list of configuration

updates to be applied to specified NTN entities.";

}

}

grouping NTNFunctionGrp {

description "Represents the NTNFunction IOC";

list nTNpLMNInfoList {

description "It defines which PLMNs that can be served by the NR NTN cell,

and which S-NSSAIs can be supported by the NR NTN cell for

corresponding PLMN in case of network slicing feature is supported.

The pLMNId of the first entry of the list is the PLMNId used to

construct the nCGI for the NR cell.";

min-elements 1;

ordered-by user;

key idx;

leaf idx { type uint32; }

uses types5g3gpp:PLMNInfo;

}

leaf-list nTNTAClist {

type types5g3gpp:NRTAC;

min-elements 1;

description "It is the list of Tracking Area Codes (either legacy TAC or

extended TAC) for NR NTN.

Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of

TS 38.413.";

}

}

grouping NTNFunctionSubtreeGrp {

list NTNFunction {

description "This IOC contains attributes to support the non-terrestrial

NR access.  This IOC contains instances of NTNTimeBasedConfig to support

time-based configuration of the following NTN related entities:

EP_NgC, NRCellCU, NRCellDU, NRSectorCarrier, SectorEquipmentFunction,

and NRCellRelation.  The IP Configuration of the feeder links may be

configured via IP Autoconfiguration services [x] and/or configured via

instances of NTNTimeBasedConfig in anticipation of feeder link

switchovers.";

key id;

max-elements 1;

uses top3gpp:Top_Grp;

container attributes {

uses NTNFunctionGrp;

}

list EphemerisInfoSet {

description "This IOC represents the satellite ephemeris information

describing the orbital trajectory information or coordinates for the

NTN vehicles.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EphemerisInfoSetGrp;

}

}

list NTNTimeBasedConfig {

description "This IOC represents the NTN time-based configuration.

";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NTNTimeBasedConfigGrp;

}

}

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature NTNFunctionUnderSubNetwork;

uses NTNFunctionSubtreeGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature NTNFunctionUnderManagedElement;

uses NTNFunctionSubtreeGrp;

}

}

<CODE ENDS>

*** END OF CHANGE 5 ***