3GPP TSG-SA5 Meeting #165	S5-260643

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0607 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.623 Correct several issues for the IOCs related to MADCOL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2026-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | - ExternalDataType is defined as string with several allowed values speciifed, however, such allowed vlaues are missing in the YAML definition. - Note 7 in clause 4.4.1 in TS 28.622 described that allowed value for mgtDataCategory can be further extended by the implementations, however, such extention mechanism is missing in the YAML definition. - Figure 4.2.1-8: MnS Registry NRM fragment InTS 28.622 desribed that SubNetwork can contain zero, one or multiple MnSRegistry, however, in TS28623_MnSRegistryNrm.yaml, SubNetwork only contain zero or one MnSRegistry. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the YAML definition for ExternalDataType to support both string with allowed values and string. - Update the YAML definition for mgtDataCategory to support both string with allowed values and string. - Update the YAML definition for SubNetwork-ncO-MnSRegistryNrm to support the possibility to contain multiple MnSRegistry. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The YAML deifnition for ExternalDataType, mgtDataCategory and SubNetwork-ncO-MnSRegistryNrm is not aligned with stage2 deifnition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Following YAML files normatively defined in the forge are updated: - TS28623_ExternalDataMgmtNrm.yaml - TS28623_ManagementDataCollectionNrm.yaml - TS28623_MnSRegistryNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2024 at commit 02f823320d69d06b025ded5bf2644dcfd42cc4a3 |  |  |  |  |  |  |  |  |

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2024 at commit 02f823320d69d06b025ded5bf2644dcfd42cc4a3

*** START OF CHANGE 1 ***

*** OpenAPI/TS28623_ExternalDataMgmtNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: External data management NRM

version: 19.5.0

description: >-

OAS 3.0.1 definition of the External data type NRM fragment

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM, External data type NRM fragment

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-ExternalDataMgmtNrm:

type: object

properties:

ExternalDataType:

$ref: '#/components/schemas/ExternalDataType-Multiple'

#-------- Definition of types-----------------------------------------------------

ExternalDataScope:

type: object

properties:

geoAreas:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

objectInstancesIncluded:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

objectInstancesExcluded:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#-------- Definition of concrete IOCs --------------------------------------------

ExternalDataType-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

externalDataType:

type: string

example:

- ELECTRONIC_MAP

- CAMARA_DATA

- UE_PATH

- CAMERA_PHOTO

- EVENT_SCHEDULE

- WEATHER_FORECASTS

description: >-

Type of external management data as defined by the implementation.

mediaLocation:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

externalDataTypeSchema:

type: string

externalDataScope:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/ExternalDataScope'

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

ExternalDataType-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalDataType-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-externalDataMgmtNrm:

oneOf:

- $ref: '#/components/schemas/ExternalDataType-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** OpenAPI/TS28623_ManagementDataCollectionNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Management Data Collection NRM

version: 20.0.0

description: >-

OAS 3.0.1 definition of the Management Data Collection NRM fragment

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM, Management Data Collection NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-ManagementDataCollectionNrm:

type: object

properties:

ManagementDataCollection:

$ref: '#/components/schemas/ManagementDataCollection-Multiple'

#-------Definition of generic IOCs ----------#

#-------- Definition of types-----------------------------------------------------

ManagementData:

oneOf:

- type: array

items:

$ref: '#/components/schemas/mgtDataCategory'

uniqueItems: true

- type: array

items:

$ref: '#/components/schemas/mgtDataName'

uniqueItems: true

mgtDataCategory:

oneOf:

- type: string

enum:

- COVERAGE

- CAPACITY

- SERVICE_EXPERIENCE

- TRACE

- ENERGY_EFFICIENCY

- MOBILITY

- ACCESSIBILITY

- type: string

mgtDataName:

type: string

NodeFilter:

type: object

properties:

areaOfInterest:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AreaOfInterest'

networkDomain:

type: string

enum:

- CN

- RAN

cpUpType:

type: string

enum:

- CP

- UP

sst:

type: integer

objectInstances:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#-------- Definition of concrete IOCs --------------------------------------------

ManagementDataCollection-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

managementData:

$ref: '#/components/schemas/ManagementData'

targetNodeFilter:

type: array

items:

$ref: '#/components/schemas/NodeFilter'

uniqueItems: true

collectionTimeWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

reportingCtrl:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ReportingCtrl'

dataScope:

type: string

enum:

- SNSSAI

- 5QI

- PLMN

condition:

type: string

processMonitor:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ProcessMonitor'

consolidateOutput:

type: boolean

jobId:

type: string

required:

- managementData

- targetNodeFilter

- collectionTimeWindow

- reportingCtrl

- jobId

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

ManagementDataCollection-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagementDataCollection-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-mgmtDataCollectionNrm:

oneOf:

- $ref: '#/components/schemas/ManagementDataCollection-Single'

<CODE ENDS>

*** END OF CHANGE 2 ***

*** START OF CHANGE 3 ***

*** OpenAPI/TS28623_MnSRegistryNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: MnS Registry NRM

version: 19.6.0

description: >-

OAS 3.0.1 definition of the MnS Registry NRM fragment

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM, MnS Registry NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-MnSRegistryNrm:

type: object

properties:

MnsRegistry:

$ref: '#/components/schemas/MnsRegistry-Multiple'

#-------Definition of generic IOCs ----------#

#-------- Definition of types-----------------------------------------------------

MnsCapability:

type: string

enum:

- NR_PROVISIONING

- 5GC_PROVISIONING

- NETWORK_SLICING_PROVISIONING

- EDGE_COMPUTING_PROVISIONING

- PERFORMANCE_METRIC_COLLECTION_CONTROL

- PERFORMANCE_METRIC_DATA_REPORT

- PERFORMANCE_METRIC_THRESHOLD_MONITOR_CONTROL

- PERFORMANCE_METRIC_THRESHOLD_NOTIFICATION

- FAULT_CONTROL

- FAULT_NOTIFICATION

- TRACE_MDT_DATA_COLLECTION_CONTROL

- TRACE_MDT_DATA_REPORT

- STM_PROVISIONING

- STM_STREAMING

- QOE_DATA_COLLECTION_CONTROL

- QOE_DATA_REPORT

- FILE_RETRIEVAL

- FILE_DOWNLOAD

- CONFIGURATION_PLAN_MGMT

- SUBSCRIPTION_CONTROL

- HEARTBEAT_CONTROL

- HEARTBEAT_NOTIFICATION

- MANAGEMENT_DATA_ANALYTIC

- RANSC_MANAGEMENT

- SON_POLICY

- COMMUNICATION_SERVICE_ASSURANCE_CONTROL

- CLOSED_CONTROL_LOOP_MANAGEMENT

- INTENT_DRIVEN_MANAGEMENT

- ML_MODEL_MANAGEMENT

- NDT_LCM

- MNS_REGISTRY_AND_DISCOVERY

- MGMTDATA_REGISTRY_AND_DISCOVERY

- MNS_ACCESS_CONTROL_MANAGEMENT

- DSO_RAPID_RECOVERY_AND_THRESHOLD_MONITORING

- EXTERNALDATA_DISCOVERY_AND_REQUEST

#-------- Definition of concrete IOCs --------------------------------------------

MnsRegistry-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

MnsInfo:

$ref: '#/components/schemas/MnsInfo-Multiple'

MgmtDataInfo:

$ref: '#/components/schemas/MgmtDataInfo-Multiple'

MnsInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

mnsLabel:

type: string

mnsType:

type: string

enum:

- PROV_MNS

- FAULT_SUPERVISION_MNS

- STREAMING_DATA_REPORTING_MNS

- FILE_DATA_REPORTING_MNS

mnsVersion:

type: string

mnsAddress:

description: Resource URI as defined in the relevant Technical Specification

$ref: 'TS28623_ComDefs.yaml#/components/schemas/UriRo'

mnsCapability:

description: >-

It describes the types of management capabilities of the MnS instance provided by the MnS producer.

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MnsCapability'

mnsScope:

description: >-

It is used to provide information about the management scope of the Management Service. The management scope is

used to represent the set of managed object instances that can be accessed using the Management Service.

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MnsScope'

minItems: 1

mgmtDataInfoRef:

description: >-

List of DN of MgmtDataInfo instance(s) which are associated the MnSInfo which represent a

management service instance

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

uniqueItems: true

minItems: 1

MnsScope:

description: ->

It defines the information about the management scope of the Management Service.

oneOf:

- type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

- type: array

uniqueItems: true

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tai'

MgmtDataInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

supportedManagementData:

description: >-

It defines the list of management data that can be supported.

type: array

uniqueItems: true

items:

$ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/ManagementData'

supportedGranularityPeriods:

description: >-

Granularity periods supported for the production of associated management data.

The period is defined in seconds.

type: array

items:

type: integer

uniqueItems: true

supportedReportingPeriods:

description: >-

Reporting periods supported for the associated management data.

The period is defined in seconds.

type: array

items:

type: integer

uniqueItems: true

historicalDataPeriod:

description: >-

This attribute describes the maximum period of the requested

historical data.

The period is defined in seconds.

When the value of this attribute is NULL, which means the

capability of querying historical data is not supported.

type: integer

supportedReportingMethod:

description: >-

List of supported reporting methods for the associated

management data.

type: array

uniqueItems: true

items:

type: string

enum:

- FILE

- STREAM

minItems: 1

supportedDataScope:

description: >-

List of supported sub counter capabilities for the associated management data.

type: array

uniqueItems: true

items:

type: string

enum:

- SNSSAI

- 5QI

- PLMN

minItems: 1

supportedDataRequestMnSRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

supportedDataReportingMnSRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

MnsRegistry-Multiple:

type: array

items:

$ref: '#/components/schemas/MnsRegistry-Single'

MnsInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/MnsInfo-Single'

MgmtDataInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/MgmtDataInfo-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-mnSRegistryNrm:

oneOf:

- $ref: '#/components/schemas/MnsInfo-Single'

- $ref: '#/components/schemas/MnsRegistry-Single'

- $ref: '#/components/schemas/MgmtDataInfo-Single'

<CODE ENDS>

*** END OF CHANGE 3 ***