3GPP TSG-SA5 Meeting #165	S5-260645

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0608 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.623 Correct definition for GeoArea related attributes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2026-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | In TS 28.622, MnsScope <<Choice>> contain one of the attribute “objectInstanceList”, “geoAreasList” and “taiList”, however, such attribute name is missing in the YAML definition for MnsScope in TS28623_MnSRegistryNrm.yaml. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the YAML definition for MnsScope to align with corresponding stage 2 deifnition in TS 28.622. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The YAML definition for MnsScope is not aligned with stage 2 deifnition in TS 28.622. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Following YAML files normatively defined in the forge are updated: - TS28623_MnSRegistryNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2025 at commit 0a03aa8a249ecf9ce20658d39c48f62018a871e7 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** OpenAPI/TS28623_MnSRegistryNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: MnS Registry NRM

version: 19.6.0

description: >-

OAS 3.0.1 definition of the MnS Registry NRM fragment

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM, MnS Registry NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-MnSRegistryNrm:

type: object

properties:

MnsRegistry:

$ref: '#/components/schemas/MnsRegistry-Single'

#-------Definition of generic IOCs ----------#

#-------- Definition of types-----------------------------------------------------

MnsCapability:

type: string

enum:

- NR_PROVISIONING

- 5GC_PROVISIONING

- NETWORK_SLICING_PROVISIONING

- EDGE_COMPUTING_PROVISIONING

- PERFORMANCE_METRIC_COLLECTION_CONTROL

- PERFORMANCE_METRIC_DATA_REPORT

- PERFORMANCE_METRIC_THRESHOLD_MONITOR_CONTROL

- PERFORMANCE_METRIC_THRESHOLD_NOTIFICATION

- FAULT_CONTROL

- FAULT_NOTIFICATION

- TRACE_MDT_DATA_COLLECTION_CONTROL

- TRACE_MDT_DATA_REPORT

- STM_PROVISIONING

- STM_STREAMING

- QOE_DATA_COLLECTION_CONTROL

- QOE_DATA_REPORT

- FILE_RETRIEVAL

- FILE_DOWNLOAD

- CONFIGURATION_PLAN_MGMT

- SUBSCRIPTION_CONTROL

- HEARTBEAT_CONTROL

- HEARTBEAT_NOTIFICATION

- MANAGEMENT_DATA_ANALYTIC

- RANSC_MANAGEMENT

- SON_POLICY

- COMMUNICATION_SERVICE_ASSURANCE_CONTROL

- CLOSED_CONTROL_LOOP_MANAGEMENT

- INTENT_DRIVEN_MANAGEMENT

- ML_MODEL_MANAGEMENT

- NDT_LCM

- MNS_REGISTRY_AND_DISCOVERY

- MGMTDATA_REGISTRY_AND_DISCOVERY

- MNS_ACCESS_CONTROL_MANAGEMENT

- DSO_RAPID_RECOVERY_AND_THRESHOLD_MONITORING

- EXTERNALDATA_DISCOVERY_AND_REQUEST

#-------- Definition of concrete IOCs --------------------------------------------

MnsRegistry-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

MnsInfo:

$ref: '#/components/schemas/MnsInfo-Multiple'

MgmtDataInfo:

$ref: '#/components/schemas/MgmtDataInfo-Multiple'

MnsInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

mnsLabel:

type: string

mnsType:

type: string

enum:

- PROV_MNS

- FAULT_SUPERVISION_MNS

- STREAMING_DATA_REPORTING_MNS

- FILE_DATA_REPORTING_MNS

mnsVersion:

type: string

mnsAddress:

description: Resource URI as defined in the relevant Technical Specification

$ref: 'TS28623_ComDefs.yaml#/components/schemas/UriRo'

mnsCapability:

description: >-

It describes the types of management capabilities of the MnS instance provided by the MnS producer.

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MnsCapability'

mnsScope:

description: >-

It is used to provide information about the management scope of the Management Service. The management scope is

used to represent the set of managed object instances that can be accessed using the Management Service.

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/MnsScope'

minItems: 1

mgmtDataInfoRef:

description: >-

List of DN of MgmtDataInfo instance(s) which are associated the MnSInfo which represent a

management service instance

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

uniqueItems: true

minItems: 1

MnsScope:

description: ->

It defines the information about the management scope of the Management Service.

oneOf:

- type: object

properties:

objectInstanceList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- type: object

properties:

geoAreasList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

- type: object

properties:

taiList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tai'

MgmtDataInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

supportedManagementData:

description: >-

It defines the list of management data that can be supported.

type: array

uniqueItems: true

items:

$ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/ManagementData'

supportedGranularityPeriods:

description: >-

Granularity periods supported for the production of associated management data.

The period is defined in seconds.

type: array

items:

type: integer

uniqueItems: true

supportedReportingPeriods:

description: >-

Reporting periods supported for the associated management data.

The period is defined in seconds.

type: array

items:

type: integer

uniqueItems: true

historicalDataPeriod:

description: >-

This attribute describes the maximum period of the requested

historical data.

The period is defined in seconds.

When the value of this attribute is NULL, which means the

capability of querying historical data is not supported.

type: integer

supportedReportingMethod:

description: >-

List of supported reporting methods for the associated

management data.

type: array

uniqueItems: true

items:

type: string

enum:

- FILE

- STREAM

minItems: 1

supportedDataScope:

description: >-

List of supported sub counter capabilities for the associated management data.

type: array

uniqueItems: true

items:

type: string

enum:

- SNSSAI

- 5QI

- PLMN

minItems: 1

supportedDataRequestMnSRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

supportedDataReportingMnSRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

MnsInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/MnsInfo-Single'

MgmtDataInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/MgmtDataInfo-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-mnSRegistryNrm:

oneOf:

- $ref: '#/components/schemas/MnsInfo-Single'

- $ref: '#/components/schemas/MnsRegistry-Single'

- $ref: '#/components/schemas/MgmtDataInfo-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***