3GPP TSG-SA5 Meeting #165	S5-260663d1

# Goa, India, 09 – 13 February 2026												Revision of S5-260433

Source:	Ericsson

Title:	Pseudo-CR TR 28.882 Add solution to two-sided ML model training use case

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.2.0

Work Item:	FS_AIML_MGT_Ph3

Comments

This pCR proposes a solution for two-sided ML model training use case reported in clause 5.1.1.4.1

Proposed Changes

* * * First change * * * *

#### 5.1.1.4		Management support to data collection for two-sided model training

##### 5.1.1.4.1	Management support to CSI compression

##### 5.1.1.4.1.1	Description

To support CSI compression defined in TR 38.843 [3], the operator can deliver relevant data for two-sided model training (see NOTE 1) to a UE-side model training entity (e.g. a server deployed by an MNO or by an OTT service provider). The UE-side model training entity uses the received data to perform UE-part model training for CSI compression.

For this use case, the following approach is considered: gNB -> OAM -> UE-side training entity (a server inside MNO or an OTT server), where the gNB is the data-collection entity for relevant data for two-sided model training.

The operator uses the 3GPP management system to control and supervise how relevant data for CSI compression is delivered to the UE-side training entity (see NOTE 2).

Figure 5.1.1.4.1.1-1 Illustration of data collection for two-side CSI model training

The UE-side training entity sends a subscription request (see NOTE 3) to the 3GPP management system, expressing its interest to receive the data for CSI compression. The UE-side training entity can also include some conditions related to locations (e.g., geographic area) or time (e.g., time windows) for when it expects this data. The 3GPP management system registers this request, which includes the identifier of the UE-side training entity (see NOTE 5). If the subscribed data is not available at the 3GPP management system, the 3GPP management system then decides from which gNB(s) this data needs to be collected and proceeds with their configuration. Each selected gNB (see NOTE 4) is configured with information that specifies what data this gNB needs to produce for CSI compression, and how this data when available needs to be reported to the 3GPP management system . Based on this configuration and the network status, the gNB reports the requested data to the 3GPP management system.

Upon collecting the data from the different gNBs, the management system delivers it to the UE-side training entity based on access control information specified by the operator (see NOTE 2).

NOTE 1:	The relevant data for two-side model (see arrows in Figure 5.1.1.4.1.1-1) is subject to further discussion, pending ongoing correspondence and confirmation by RAN2.

NOTE 2:	Access control information is for further discussion.

NOTE 3:	The term ‘subscription request’ does not specifically imply adoption of a subscribe/notify mechanism in the solution. It is used to capture the requirement that the UE-side training entity can declare its interest in data delivery.

NOTE 4:	The selected gNB(s) support the AI/ML feature for CSI compression use case.

NOTE 5:	The information of the UE-side training entity is managed by the operator within the 3GPP management system. The 3GPP management system will use this information to select appropriate gNB(s).

##### 5.1.1.4.1.2	Potential requirements

REQ-ML_TWOSIDE-01: The 3GPP management system should have a capability allowing a UE-side training entity to subscribe for receiving relevant data for CSI compression.

REQ-ML_TWOSIDE-02: The 3GPP management system should have a capability to configure one or more gNBs to produce and report relevant data for CSI compression.

REQ-ML_TWOSIDE-03: The 3GPP management system should have a capability to deliver relevant data to a subscribed UE-side training entity.

##### 5.1.1.4.1.3	Possible solutions

The current data collection framework specified in 3GPP management system allows configuring data collection jobs on data producers. This configuration is done over ProvMnS using any of the following IOCs defined in 3GPP TS 28.622: ManagementDataCollection, TraceJob, PerfMetricJob and QMCJob. These IOCs allows handling with data for observability purposes. This data, referred to as management data, is restricted to the following data categories: trace/MDT/QoE data, performance measurements and analytics/KPIs.

The problem is that the “relevant data for two-sided ML model” in Figure 2 does not fall under any of these data categories, and therefore does not qualify as management data.; the reason is that “relevant data for two-sided ML model” contains data not used for observability purposes, but for training an ML model in a specific use case, i.e. CSI compression. Therefore, none of the IOCs currently defined in the data collection framework can be reused to support this use case.

The proposed solution defines brand-new IOCs that allow 3GPP management system to support receiving and configuring requests scoping data for CSI compression use case. These brand-new IOCs are two: CSIDataConfiguration IOC and CSIDataJob IOC. For the proposed solution,

CSIDataConfiguration IOC is used on the management service interface between UE-side training entity and the 3GPP management system (see Figure 5.1.1.4.1.1-1). This IOC is used by the UE-side training entity (external management service consumer) to configure how it expects to receive “relevant data for two-sided ML model”, so that the 3GPP management system (management service producer) can report this “relevant data for two-sided ML model” accordingly. This IOC includes attributes specifying the following information:

- The data that is requested to be reported to the UE-side training entity, i.e. the “relevant data for two-sided ML model”. The requested data can be specified using the identifier/label that 3GPP management system defines to represent this data.

- Context information  for the requested data. This context specifies conditions that UE-side training entity can issue as part of the request, including e.g. area-based conditions (geographic area(s) where the data is expected to be collected), time-based conditions (time window(s) expressing when the collected data is expected to be delivered) or frequency-based conditions.

- Information of the NW vendor(s), e.g. identifier, name, etc., with which the UE-side training entity has tested the use case before. The 3GPP management system will use this information to select appropriate gNB(s).

NOTE: Without prior testing, the two-sided ML model would not work, due to dependencies.

-  Reporting control information, to report requested data to UE-side training entity. This includes selected reporting method (e.g., file-based reporting, streaming-based reporting) and associated control parameters.

-  Information for the UE-side training entity to monitor the progress of the request fulfilment, e.g. status, percentage completion, etc, given the async nature of the request.

CSIDataJob IOC is used on the management service interface between the 3GPP management system and each gNB. This IOC represents a data production job request sent by the 3GPP management system (management service consumer) and fulfilled by the gNB (management service producer). This IOC includes attributes specifying the following information:

- The data that is requested to be produced. The requested data can be specified using the identifiers/labels that gNB defines to represent this data.

- Configuration parameters associated to the requested data. These parameters include settings that the gNB needs to know in order to produce the data for the CSI compression use case.

NOTE: These parameters will be used by gNB on the radio interface to tweak data collection from UEs. The interaction between gNB and UEs over this interface is outside the scope of SA5 discussion.

- Reporting control information to deliver requested data to 3GPP management systemThis includes selected reporting method (e.g., file-based reporting, streaming-based reporting) and associated control parameters.

- Identifier of the data production job that gets created on the gNB. This identifier can be used for correlation purposes, e.g. if the 3GPP management system triggers multiple data production job requests to fulfil a given data collection job request, then all these data production jobs can have the same identifier.

The reason to use different IOCs over the two interfaces is that the information contained in each IOC is different. Additionally, please note that there exist dependencies between both IOCs, i.e. CSIDataConfiguration IOC can only be instantiated based on CSIDataConfiguration IOC. These dependencies will be modelled in normative stage.

Following these IOC definitions, the solution flow might look like as follows:

Step 1a. The UE-side training entity sends createMOI request (see createMOI operation defined in TS 28.532 )to the 3GPP management system, for CSIDataConfiguration IOC.  The UE-side training entity is external MnS consumer, and 3GPP management system is MnS producer.

Step 1b. The management system creates a CSIDataConfiguration MOI and configures it with the information contained in the request.

Step 1c. The management system sends a createMOI response (see createMOI operation defined in TS 28.53) to the UE-side training entity. This response includes the Distinguished Name (DN) of the created CSIDataConfiguration MOI.

Step 2. The 3GPP management system processes the information received in step 1a, checks access rights for UE-side training entity (NOTE) and checks whether the requested data i.e. “relevant data for two-sided ML model” is already available at the 3GPP management system. If available, then go to step 8.  Otherwise, go to step 3.

NOTE: access rights are configured by the operator. Their definition is for FFS.

Step 3. The 3GPP management system processes the information received in step 1a and select one or more or more gNBs to produce data (see NOTE).  For each gNB, the 3GPP management system performs steps 4, 5 and 6.

Step 4a. The 3GPP management system sends createMOI request (see createMOI operation defined in TS 28.532) to a gNB, for CSIDataJob IOC.  The 3GPP management system is MnS consumer, and gNB is MnS producer.

Step 4b. The gNB creates a CSIDataJob MOI and configures it with the information contained in the request.

Step 4c. The management system sends a createMOI response (see createMOI operation defined in TS 28.532) to the UE-side training entity. This response includes the Distinguished Name (DN) of the created CSIDataJob MOI.

Step 5. Based on information received in step 4a, the gNB interacts with UEs over the air interface, configuring them to measure and log relevant and report them back to the gNB using RRC framework.

NOTE: Step 5 is outside SA5 scope.

Step 6. The gNB reports produced data to the 3GPP management system, using the reporting method that the management system had previously configured in step 4a.

Step 8. Once the 3GPP management system receives data produced by all the configured gNBs, it checks whether the received data can be sent to UE-side training entity as-is (i.e. the gNB generates “relevant data for two-sided ML model training”) or further processing is needed on the 3GPP management system (i.e. the gNB sends data that 3GPP management system will use to generate “relevant data for two-sided ML model training”).

NOTE: Step 8 will depend on RAN discussion progress.

Step 9.  The 3GPP management system reports “relevant data for two-sided ML model” using the reporting method that the  management system had previously configured in step 1a.

##### 5.1.1.4.1.4	Possible solutions evaluation

The solution proposes to enhance the data collection framework in the context of 5G, by defining brand-new IOCs: CSIDataConfiguration IOC (used for UE-side training entity to send configuration request to 3GPP management system to receive data for CSI compression, see step 1) and CSIDataJob IOC (used for 3GPP management system to send data production request job to selected gNBs, see step 4).The definition of these two IOCs allows supporting the AI/ML feature CSI compression use case that RAN has defined in Rel-20 in the context of 5G, while ensuring that the existing mechanisms for data collection remain untouched on their scope, which is observability.

The solution proposes that how data is reported (see steps 6 and step 9) relies on existing reporting solutions. The reason is that current RAN information does not hint any issue that prevents reusing existing delivery reporting methods (file and/or streaming based) and associated formats.

Since this use case will be discussed again in 6G timeframe, as per SA guidance, the proposed solution should be taken as possible in the context of 5G only.

Annex A (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA5#162 |  |  |  |  | Initial skeleton | 0.0.0 |
| 2025-10 | SA5#163 | S5-254664 S5-254666 S5-254667 S5-254744 | pCR pCR pCR pCR |  |  | pCR TR 28.882 initial ToC Pseudo-CR on TR 28.882 add Management support to UE-Side model training use case Pseudo-CR on TR 28.882 add Management support to NW-Side model training use case Pseudo-CR on TR 28.882 Add New Use Case on Management of Vertical Federated Learning | 0.1.0 |
| 2025-11 | SA5#164 | S5-255505 S5-255506 S5-255515 S5-255518 S5-255519 S5-255687 | pCR pCR pCR pCR pCR pCR |  |  | Pseudo-CR TR 28.882 Use case on ML model transfer delivery to UE Pseudo-CR on TR 28.882 add Management support use case and requirement for two-sided model training Pseudo-CR TR 28.882 Add Solution for Management of Vertical Federated Learning pCR on Rel-20 TR 28.882 Adding Enhancements on LCM of Federated Learning Pseudo-CR TR 28.882 Sustainability aspects of ML model training and inference Rel-20 Pseudo-CR TR 28.882 Use case on enhancing Reinforcement Learning with performance targets | 0.2.0 |