3GPP TSG SA5 Meeting #165	S5-260725d1

Goa, India, 9-13 February 2026

Source:	Samsung, AsiaInfo, Nokia

Title:	Rel-20 pCR 32.801-01 Use case for management exposure to agents external to 3GPP system

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	v0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution is a consolidation of the following documents and has been prepared to assist with efficiency of the discussions:

- S5-260294 Rel-20 pCR 32.801-01 Use case of Agent Enablement, Samsung

- S5-260308 Rel-20 pCR TR 32.801-1 Add use case of agents in 6G network, AsiaInfo

- S5-260351 Rel-20 pCR TR 32.801-01 Add Management Scenarios on Generative AI and LLMs in 6G System, Nokia

Proposed Changes

* * * First Change * * * *

### 6.1.X <Management Scenario Category#1>

#### 6.1.X.1 Management Scenario #<1>: Use case for management exposure to agents external to 3GPP system

##### 6.1.X.1.1 Description

The 3GPP SA1 Rel-20 study (TR 22.870) has systematically identified and defined a series of AI Agent use cases for 6G. These use cases reveal the application scenarios, roles, service requirements, and associated management needs of AI Agents across various dimensions of 6G network. They thus provide a basis for SA5 addressing support for AI Agents that are external to the 3GPP management system.

Such Agents may require to access the management services provided by the 3gpp management system to effectively perform their task. For example, an Agent aiming to optimize handover in the RAN node may need to access performance assurance management service to collect available network data (e.g., resource utilization, energy saving state, number of registered subscriber) from the neighbor RAN node and then take intelligent decision to optimize handover.

In the context of existing 3GPP management services exposure, there are different exposure approaches defined in TS 28.533

In the new 6G scenarios, in order to facilitate the adoption and consumption of network management services by such network external AI agentic ecosystem, there are two alternative approaches that can be considered:

(i) the exposure of management services is adapted to the way the external AI agents discover and consume external services (e.g. tools, resources discovery and consumption.

(ii) the external AI agents implement an integration layer to enable consumption of existing management service exposure.

##### The implications of both approaches for this management scenario should be studied.

##### 6.1.X.1.2 Potential Requirements

<Moderator´s suggestion, consider either of the following alternatives below as the single requirement to start the investigation>

(Alternative 1) REQ-Agents-Exp-1: The 6G management system shall enable exposure of management services towards 3rd party MnS consumers that leverage AI agents. <0351>

(Alternative 2) REQ-Agents-Exp-1: The 6G management system shall support the capability to enable AI Agent to access management services. <0294>

<Requirements below are the original requirements (to be removed)>

[REQ-AGENTS_IN_6G_NET-1] The 6G management system shall provide a management service which allows to assign a unique identifier to each Agent in 6G network. <0308>

<We are discussing the external Agents, and the 3GPP management system does not assign the identifier of such Agents>

[REQ-AGENTS_IN_6G_NET-2] The 6G management system shall provide a registration service that allows an Agent to register its metadata, including its unique identifier, capabilities, status, and access endpoint(s). <0308>

<We are discussing the external Agents, the registration should be only for Agents within the management system>

[REQ-AGENTS_IN_6G_NET-3] The 6G management system shall provide a discovery service that allows authorized entities (e.g., orchestrators, other Agents) to query registered Agents based on criteria such as capabilities or operational status. <0308>

Reqt-GenAI-01: The 6G 3GPP management system should enable exposure of management services towards 3rd party MnS consumers that leverage AI agents as an alternative to already existing exposure options.  <0351>

<We should have a common exposure mechanism regardless if external MnS consumer is an AI Agent or any other technology>

Reqt-GenAI-02: The 6G 3GPP management system should enable management on Inference MnS enabled by LLM. Including capabilities, e.g. on  configuration of external knowledge usage for producing inference. <0351>

<This is implementation-specific>

REQ-SNM-KG-1: The 3GPP management system shall support the capability to enable AI Agent to access management services. <0294>

* * * End of Changes * * * *