3GPP TSG-SA5 Meeting #165	S5-260726d1

Goa, India, 09 - 13 February 2026

Source:	China Mobile, ZTE Corporation, Samsung, China Unicom

Title:	pCR on TR 32.801-01 Consolidated-Category 2: Autonomous agent within 3GPP management system

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution is a consolidation of the documents

on Autonomous agents within the management system

Proposed Changes

* * * First Change * * * *

# 6	6G Management Scenarios

## 6.1 Identified 6G management scenarios

### 6.1.X Autonomous Agent

#### 6.1.X.Y Autonomous agent within 3GPP management system

##### 6.1.X.Y.1 Description

1. General description

6G networks introduce new capabilities that significantly broaden the scope of network including services such as ISAC, which introduce more diverse managed entities and more complex operating environments. These scenarios greatly increase the heterogeneity and operational complexity of 6G networks and place significantly higher demands on the management system.

Automation capabilities in 5G (e.g., AI/ML, IDM, MDA, CCL, NDT) are specified as relatively discrete enablers with no capability supporting their dynamic orchestration and collaborative invocation to solve complex, cross-domain operational problems. It should be explored how to integrate disparate automation capabilities to optimize and support the above scenarios  through Autonomous Agents.

An Autonomous Agents is a continuously operating intelligent entity capable of perception, reasoning & planning, action execution, and iterative learning. It can dynamically adapt to environmental changes, generalize to unfamiliar situations, and continuously refine its behavior based on feedback.

##### 6.1.x.1.2 Potential Requirements

* * * End of Changes * * * *