3GPP TSG SA5 Meeting #165	S5-260727

Goa, India, 9-13 February 2026

Source:	Huawei, CMCC, Ericsson

Title:	Rel-20 pCR TR 32.801-01 Terminology on autonomous agent for management

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	V0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution proposes the concept of Autonomous Network Management Agent (ANM Agent) for 6G network management and orchestration.

3GPP SA1, TM Forum and ETSI ZSM have provided agent related concepts.

2.1 3GPP SA1 TR 22.870 “Study on 6G Use Cases and Service Requirements” defines AI Agent as following:

AI Agent: an automated intelligent entity that achieves a specific goal (autonomously or not) on behalf of another entity, by e.g. interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (independently or in collaboration with other AI Agents)

2.2 TMF IG1258 “Autonomous Networks Glossary” defines Autonomous Agent as following:

Autonomous Agent: An autonomous agent is a system or entity that operates independently, making decisions and taking actions based on its own perceptions and goals without direct human intervention. It acts to fulfill its own agenda or goals and must be adaptive, continuously learning and improving from its experiences and interactions with its environment.

2.3	ETSI ZSM GR ZSM-020 “Study on the Utilization of Agents in Autonomous Networks” defines Autonomous Agent as following:

Autonomous Agent: intent-driven, autonomous AI entity designed to perceive its environment, analyse information using the agent’s existing knowledge, make decisions also based on this knowledge and execute actions to achieve predefined goals described in the intent with minimal to no human intervention, as well as continuously learning and adapting its behaviours based on the changing conditions in the environment.

2.4 3GPP TS 28.100 “Management and orchestration; Levels of autonomous network” defines Autonomous Network as following:

Autonomous Network: telecommunication system (including management system and network) with autonomy capabilities which is able to be governed by itself with minimal to no human intervention.

Observation:

1.	Among the use cases described in 3GPP SA1 TR 22.870, AI Agent is widely used for user equipment (e.g., Drone, Vehicle, Robot, etc), third party applications (e.g., AI assistant, etc) and network.

2.	Term Autonomous Agent provided by TMF and ZSM focuses on a general system or entity that operates autonomously without direct human intervention.

3.	There is no specific term to describe autonomous network management agent

Intent driven, operate autonomously are important features to be adopted in management domain. SA5 should study the potential usage of autonomous network management agent for 3GPP network, which is different from the AI Agent addressed in SA1. It’s proposed to add concept of Autonomous Networks Management Agent concept in SA5.

Proposed Changes

* * * First Change * * * *

7	Key Issues

7.x Autonomous management agent

7.x.1 Overview

With adopting the new technologies, network operators and verticals’ express intents through natural language. Autonomous network management applies agent concept to use existing knowledge, make decisions based on this knowledge and execute actions to achieve predefined goals. For example, in a large-scale event (e.g., music festivals, sports games) online experience assurance scenario, Autonomous Network Management Agent has the capability of understanding event organizer’s natural language service assurance request and makes customized capacity planning and network configuration in the complex 6G system for the event. Furthermore, the intelligence and flexibility of Autonomous Network Management Agent could also optimize network swiftly to avoid potential risk beforehand.

There are three types of agent:

1. Agent external to 3GPP system, e.g., 3rd party application Agent

2. Management Agent within 3GPP management system

3. Agent in 3GPP network

Clause 7.x.2 focuses on the concept of management agent within 3GPP management system.

Note1: Use of “Management Agent” in bullet 2 to be revisited.

Note2: Use of “Agent” in bullet 3 to be aligned with SA2.

7.x.2. Concept of agent in management domain

Management Agent: an entity that operates autonomously to achieve a specific goal for service and network management and orchestration. Management Agent may be intent driven, task driven, adaptive, continuously learning, reasoning, decision-making and 1s based on the changing conditions in the environment. The autonomy capabilities carried by Management Agent are able to be governed by itself based on tasks or with minimal to no human intervention. Management Agent may use rules, logic, learned models, and tools to generate outputs (actions or tasks) but must remain restricted from altering their assigned tasks.

Editor's Note; The name of the management agent may need to be revisited.

7.x.3. Relationship with MnF

An IHF receives intents and an CCL receives goals where both intents and goals, describe the desired outcomes. Management agents can dynamically integrate and collaboratively invoke the management capabilities, e.g., IHF, AI/ML, NDT, CCL, etc.

NOTE: Interaction between agents and whether any existing functions will become agents is FFS.

* * * End of Changes * * * *