3GPP TSG-SA5 Meeting #165	S5-260728d2

Goa, India, 09 - 13 February 2026

Source:	Rapporteurs

Title:	pCR on TR 32.801-01 consolidated data management contributions

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Proposed Changes

* * * First Change * * * *

# 5.	6G Management Architecture Principles

Editor’s note: add to the list of principles in clause 5:

- The data management framework in the management system to support all the domains (e.g., RAN, Core).

* * * 2nd Change * * * *

## 6.1 New 6G management scenarios

Editor's note 1: This clause will contain new 6G management scenarios and the corresponding requirements.

### 6.1.y Data Management Framework

* * * 3rd Change * * * *

#### 6.1.y.1 Management Scenario #1: Data Management Framework in the Management System

##### 6.1.y.1.1 Description

With the evolution of 6G mobile networks, operators are required to manage a highly distributed and complex network. This environment spans core network , RAN , multiple layers of edge computing, and a wide range of network slices. These domains continually generate large volumes of heterogeneous data with differing semantics, granularity, and temporal characteristics.

As operators progress toward higher levels of autonomy in network operation, management data becomes increasingly central to supporting intelligent network management capabilities. AI‑assisted network optimization, proactive analysis, and real‑time synchronization with network digital twins rely on timely, accurate, and semantically consistent data. However, existing data collection, storage, and exposure mechanisms are fragmented and insufficient to support the needs of AI‑driven and autonomous network management.

There are some differences today in how data is produced and managed and the aim is to provide consistent management of all data used to fulfil 6G use cases for operators.

Today’s data management is too use case specific and does not account for data characteristics, e.g., latency, volume, etc.

- .-

To address these limitations, a Data Management Framework (DMFK) is needed as a key enabler for the intelligent evolution of network management in 6G. The DMF provides standardized mechanisms across the lifecycle of data for identified 6G use cases and existing networks,

By ensuring consistent, timely, and high‑quality management data availability, the DMF enables the transition from reactive monitoring to proactive optimization and supports the realization of advanced levels of autonomy in 6G network management.

##### 6.1.y.1.2 Potential Requirements

REQ-DMF-1: The 3GPP management system shall support the data management framework to empower intelligent and autonomous network operation and maintenance.

REQ-DMF-2: The 3GPP management system shall support the data management framework that accommodates diverse latency, granularity, and consumption demands associated with management data.

REQ-DMF-3: The 3GPP management system shall support data management framework for standardized lifecycle management of management data.

REQ-DMF-4: 3GPP management system should multiple QoS requirements for different types of data and deployment scenarios.

REQ-DMF-5: The 3GPP management system shall support data quality assurance for management data.

REQ-DMF-6: The 3GPP management system shall be able to guarantee the data quality (e.g., freshness, completeness, accuracy, completeness, consistency) and data delivery QoS.

REQ-DMF-7: The 3GPP management system shall ensure effective interoperability and coexistence between the 6G and 5G data management systems.

REQ-DMF-8: The 3GPP management system shall provide a unified capability for discovering and interpreting management data based on its attributes and semantics.

REQ-DMF-9: The 3GPP management system shall support secure access control and exposure of management data.

REQ-DMF-10: The 3GPP management system shall enable coordinated operation between the data management framework and autonomous management services (e.g., NDT, IDMS, CCL, etc.).

* * * 4th Change * * * *

# 7	Key Issues

Editor's note: This clause will contain the potential Key Issues related to the 6G management scenarios in clause 6. The Mapping table in Annex A will reflect the relation between the management scenarios and Key Issues.

## 7.1 Data Management Framework (DMFK)

### 7.1.1 Key Issue #1: DMF Capabilities

The DMF should include capabilities to enable the following functionalities:

- Data Lifecycle Management

- Data Collection: capability for data collection. The data which are related to data collection includes:

- Existing Management Data: Performance Measurements (PM), Key Performance Indicators (KPI), Fault/Alarm data, Configuration Management (CM) data, Trace and MDT data, Quality of Experience (QoE), system logs, external management data etc.

- New 6G Management Data: the new data for network and service management. The integration of relevant external management data sources may also be considered.

- Data Reporting:

- Data Processing: capability such as data normalization, validation, quality assessment (e.g., freshness, accuracy, completeness, consistency), aggregation, correlation, semantic enrichment,).

- Data discovery

- Data registration

- Data delivery

- Data storage

- Data repository

- Job management:   The framework should support unified transport protocols (i.e., data request, data subscription and notifications)  and data models to ensure consistent management control and data reporting, meeting data delivery QoS (e.g., latency, throughput), avoiding duplicate collection.

- Access Control and Data Exposure: The framework shall implement secure data access and data lineage and exposure. Additionally it shall allow MnS consumer to consume the data from multiple instances of DMFK and share data across multiple instances.  To supply authorized consumers with validated management data in the required timeliness and format,

- Editor’s note: Coordination with SA3 may be needed for data access control and exposure

- Integration with Autonomous Management Services: A critical success factor for the DMF is its seamless integration with key autonomous functions. data needs to be published and consumed so services share the same enriched, consistent data and context across domains and layers.  The study shall therefore specify the data support and interaction mechanisms between the DMF and autonomous MnS(s) for, e.g.:

    - Network Digital Twin (NDT) (providing real-time and historical data feeds),

    - Intent-Driven Management (supporting with semantic/knowledge data),

    - Closed Control Loop (CCL) (providing timely data support for the monitor-analyze-decision-execute cycle).

- Data contracts and versioning

  - There is a need to define schemas and semantic contracts with backward/forward compatibility; support streaming, file and batch access, and formalize change control.

- Performance

  - Where applicable, the SLOs should be defied for end-to-end data handling, covering latency, throughput, streaming metrics, storage read/write latency and durability, burst tolerance, and data quality metrics so automation loops remain timely.

- Management of 6G Data Framework

  - In TR 22.870 [2], use case and potential requirements for data framework are described. The management aspects of the data framework itself (e.g., configuration of data collection requirements and tasks, performance monitoring of the data framework) shall be an integral part of this study.

Editor’s note: Coordination with SA2 and other working groups are needed according to progress of data framework study in SA2.

- Interoperability with 5G Management Systems

A smooth transition is imperative. The study shall include:

  - Backward Compatibility: Defining how to adapt and reuse interfaces between the 6G DMF and 5G management systems/NFs, evolving 5G data models where necessary.

  - Coexistence and Interworking: Establishing architectural patterns for the concurrent operation of 5G and 6G data management framework during migration.

### 7.1.2 Key Issue #2: DMF Architecture

Figure X shows a high-level data-centric 6G management architecture including DMF:

Figure x: High level Data-Centric 6G Management Architecture

The 6G DMF should be realized through new management services (separated or integrated) within the Service-Based Management Architecture (SBMA), enabling coordinated data management across domains (cross-domain, RAN-domain, and CN-domain). As depicted in Figure Y, the framework contains components of Data Repository, Data Management Functions and Data Providers, providing data MnS(s) to internal and external MnS consumers.

Figure Y: An example of a Data Management Framework

* * * End of Changes * * * *