3GPP TSG SA5 Meeting #165	S5-260735

Goa, India, 9-13 February 2026

Source:	Nokia

Title:	Pseudo-CR on Add potential solution and evaluation on access control on notifications

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.2.0

Work Item:	FS_EnExpo

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

Proposed Changes

* * * First Change * * * *

### 5.2.3	Potential solutions

Editor's note:	This clause provides one or more solutions. Further (sub-)clause(s) may be added to capture details.

#### 5.2.3.X Configuration of access control on notifications

In this proposed solution, a new NotificationAccessRule is proposed as another access rule type to be added to the class diagram in clause 7.2 of of TS 28.319[4].  The NotificationAccessRule provides access control on a specific notification type or all notification types. As illustrated in Figure 5.2.3.X-1, each NotificationAccessRule is associated to one or more Role(s). The attributes of the NotificationAccessRule are presented in Table 5.2.3.X-1.

Table 5.2.3.X-1: Attributes of the NotificationAccessRule

| Attribute Name | S |
| --- | --- |
| ruleName | M |
| dataNodeSelector | O |
| notificationTypes | M |
| actions | O |

In the NotificationAccessRule, the notificationType attribute represents the notification type event for which the access rule applies, whereas the ruleName, dataNodeSelector and actions attribute definitions are as defined in clause 7.4.1 of TS 28.319[4]. Accordingly, for an MnS consumer to be able to receive a given notification type event from the MnS producer, one of the Role(s) associated to the MnS consumer should be configured with the corresponding NotificationAccessRule and the action of the NotificationAccessRule should be permitted (i.e., by setting the value of the action attribute to “ALLOW”). Also, for an MnS consumer to receive notifications from one or more managed object instances (represented by the “dataNodeSelector” attribute of the DataNodeAccessRule), the Role associated to the MnS consumer needs to be configured to have “read” operation permitted on those managed object instances. If provided, the NotificationAccessRule.dataNodeSelector attribute can be used to explicitly set the notification access control rules on one or more MOIs.

Figure 5.2.3.X-1: Classes for role-based access control including the notification access rule (See Figure 7.2-1 of TS 28.319[4])

The following definition for the notificationType attribute for the NotificationAccessRule is provided:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| notificationType | It describes the notification type that the NotificationAccessRule applies to. The absence of a value for this attribute means that the NotificationAccessRule applies for all notification types. | type: SupportedNotifications.notificationTypes. (see clause 4.3.61 of TS 28.622[6]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

#### 5.2.3.Y Enforcement of the access control on notifications

In the following clauses, it is explained how the proposed solution in clause 5.2.3.X can be used by the MnS producer when it comes to enforcement on notification subscriptions and sending considering different scenarios. The proposed solution in this clause assumes that the MnS consumer is authorized to create the NtfSubscriptionCtrl MOI as per the access control model described in clause 7 of TS 28.319[4].

##### 5.2.3.Y.1 Notification subscription handling

This potential solution considers a scenario when an MnS consumer subscribes to receive notifications using the NtfSubscriptionControl IOC(see clause 4.3.22 of TS 28.622[6]). To this end,  the MnS producer evaluates the subscription request by checking if there are existing NotificationAccessRule(s)where:

- the Role associated to the NotificationAccessRule is the same as the Role associated to the MnS consumer

- the NotificationAccessRule.notificationTypes includes a subset of all the notification types specified in the NtfSubscriptionCtrl.notificationTypes attribute.

- If the NotificationAccessRule.DataNodeSelector attribute is present, the values in NtfSubscriptionCtrl.scope and NtfSubscriptionCtrl.notificationFilter attributes in the request are a subset or correspond exactly to the values of the NotificationAccessRule.DataNodeSelector.

- If the NotificationAccessRule.DataNodeSelector attribute is absent, checks if the value of the values in NtfSubscriptionCtrl.scope and NtfSubscriptionCtrl.notificationFilter attributes in the request are a subset or correspond exactly to the values of the DataNodeAccessRule.DataNodeSelector and the “read” operation on the DataNodeAccessRule.DataNodeSelector is allowed

- the value of the NotificationAccessRule.action is “ALLOW”

If all the checks (i) to (v) are true, the notification subscription request succeeds. If no such rule is found, (as a default behaviour) the creation/modification of the NtfSubscriptionControl MOI is denied. However, if checks (ii), (iii) and (iv) are false and checks (i) and (v) are true, then two different behaviours of the MnS producer are proposed, i.e., the strict scope and partial scope enforcement models:

- Strict scope enforcement model: here the MnS producer informs the MnS consumer that the notification subscription request wasn’t successful and provides the appropriate reason.In this case, the possible reasons could include: access denied to the NtfSubscriptionControl MOI, not authorized to receive “XYZ” notification types, not authorized to receive any notification types, not authorized to receive notifications for “XYZ” part of the scope, not authorized to receive any notifications on this whole scope. Accordingly, in the strict scope enforcement model, the MnS consumer is not allowed to subscribe to receive notification types beyond the configured NotificationAccessRule.notificationTypes, NotificationAccessRule.dataNodeSelector and DataNodeAccessRule. dataNodeSelector authorized for the Role(s) associated to the MnS consumer.

- Partial scope enforcement model: here the MnS producer informs the MnS consumer that the notification subscription request has been successfully created. The MnS producer can indicate to the MnS consumer that the subscription is accepted but the notifications will be delivered only for the authorized partial scope “XYZ” or subset of the notification types of NotificationAccessRule.notificationTypes.

NOTE: It’s upto implementation which enforcement model is applied by the MnS producer.

##### 5.2.3.Y.2 Notification sending

In this scenario, when a notification event occurs, the MnS producer does similar checks as described in clause 5.2.3.Y.1.

If statements (i) to (iv) in clause 5.2.3.Y.1 are true, then the notification event will be sent to the notificationRecipeintAddress (this attribute was provided by the MnS consumer during the NtfSubcriptionCtrl MOI creation request). However if any of the above statements is false, no notification will be sent out.

Further, in case some notifications that were earlier sucessfully subcribed to by the MnS consumer can nolonger be sent, then the MnS producer will inform the MnS consumer of such notification subcription changes including the reason. One possible reason for the change in the notification events received by the MnS consumer could be due to the modification(s) of the NotificationAccessRule(s) associated to the Role(s)of the MnS consumer after the notification subscription

NOTE:  The proposed solution assumes that the MnS producer’s responsibility is limited to ensuring that the MnS consumer is authorized to subscribe and receive given notification type(s). So, it’s upto the MnS consumer to ensure that the entity that receives the notifications (identified by the notificationRecipientAddress attribute of the NtfSubscriptionControl IOC) is authorized to receive such notification type(s).

* * * Next Change * * * *

### 5.2.4	Evaluation of potential solutions

Editor's note:	This clause provides evaluation of potential solutions.

The proposed solution extends the existing access control model defined in TS 28.319[4] by adding a new access control rule for notifications called NotificationAccessRule. Further, the proposed solution specifies how the defined NotificationAccessRule(s) and the dataNodeAccessRule(s)can be used to provide access control for  notification subscription handling and sending.

* * * End of Changes * * * *