3GPP TSG-SA5 Meeting #165	S5-260741

Goa, India, 9 - 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.315 | CR | 0003 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | - x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.315 Add procedure for WAB-node connecting to management system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph4-OAM |  |  |  |  |  | Date: |  |  | 2026-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A mobile NR node can implement different network features, including IAB and WAB. The management of a mobile NR node includes the node connectivity to management system (specified in TS 28.314/28.315) and node configuration (specified in TS 28.531). In TS 28.315, the mobile NR node connecting to management system only covers IAB feature; WAB feature is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for WAB-node connecting to management system, so both IAB and WAB features are covered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure of node-OAM connectivity for WAB feature is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6, 5.x (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.6	IAB-node connects to management system

IAB-node connects to management system at power up and during mobility following PnC steps as described in clause 6.1.2 of TS 28.314 [2]. The management system that IAB-node connects to may change due to IAB-node mobility behaviours. The procedures for IAB-node connecting to management system based on its locations are illustrated in Figure 5.6.1, this includes step 2 of obtaining IP configuration via notification mechanism instead of initial IP autoconfiguration procedure.

In step 1, IAB-node has established a connection with its management system via PnC

In step 2, it reuses the existing notifications and operations as defined in TS 28.532 [4] to obtain IP configuration for OAM connectivity:

The provisioning MnS producer of IAB-node should provide IAB-node location information via any of the following notifications:

- notifyMOICreation

- notifyMOIAttributeValueChanges

- notififyMOIChanges

The provisioning MnS consumer (management system) should provide IP configuration for OAM connectivity via any of the following operations upon the reception of notifications:

- createMOI

- modifyMOIAttributes

In step 3, IAB-node connects to new management system by following subsequent PnC steps including connection to CA/RA, SeGW and SCS.

Figure 5.6.1: Procedure flows for IAB-node connects to management system

Next Change

## 5.X	WAB-node connects to management system

WAB-node connects to management system at power up and during mobility following PnC steps as described in clause 6.2.2 of TS 28.314 [2]. The management system that WAB-node connects to may change due to WAB-node mobility behaviours. The procedures for WAB-node connecting to management system based on its locations are illustrated in Figure 5.X.1, this includes step 2 of obtaining IP configuration via notification mechanism instead of initial IP autoconfiguration procedure.

Description of step 1, step 2 and step 3 are the same as in clause 5.6, with WAB-node instead of IAB-node.

Figure 5.X.1: Procedure flows for WAB-node connects to management system

Next Change

# A.X	IAB-node connects to management system

The following PlantUML source code is used to describe the procedure flows for WAB-node connecting to management system, as depicted by Figure 5.X.1:

@startuml

actor “WAB-node” as WAB

participant "management system 1" as OAM_1

participant "management system 2" as OAM_2

Ref over WAB, OAM_1: 1. Precondition: WAB-node has established connection with its management system (1) via PnC

Ref over WAB, OAM_1: 2. Obtain IP configuration for new management system via notifications based on WAB-node location

Ref over WAB, OAM_2: 3. Connect to new management system (2) by following subsequent PnC procedures including connection to CA/RA, SeGW and SCS

hide footbox

skinparam DefaultFontSize 11

skinparam sequenceActorBackgroundColor #FFFFFF

skinparam sequenceParticipantBackgroundColor #FFFFFF

skinparam noteBackgroundColor #FFFFFF

autonumber "#'.'"

skinparam monochrome true

skinparam shadowing false

@enduml

End of Changes