3GPP TSG-SA5 Meeting #165 	S5-260753

Goa, INDIA 9 - 13 Febrary 2026

Title:	Reply LS on geographical area scope MDT

Response to:	LS on geographical area scope MDT (R3-258823)

Release:	REL-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	SA5

To:	RAN3, RAN2

Cc:	-

Contact person:	Min Shu

shumin@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

SA5 thanks RAN3 for the incoming LS on geographical area scope MDT (R3-258823), with following question:

- “In RAN3 signaling, two different PLMN list IEs are used for the user consent and for geographical area, as defined in attached CRs. The content of the PLMN list for the user consent may be different than the content of the PLMN list for geographical area. Since SA5 has mentioned that they reuse existing PLMN list IE for the geographical area, RAN3 would like to ask SA5 to verify if the SA5 solution addresses RAN3’s case.”

SA5 would like to provide the following clarification in response to the questions raised:

•	SA5 notes that the MDT PLMN List (defined in TS 32.422, subclause 5.10.24) is used by the management domain to indicate the PLMN(s) where MDT measurement collection, status indication and log reporting are administratively allowed. It is an optional attribute provided by OAM as part of the MDT configurations in the Trace Job message.

•	SA5 further notes that, as specified in TS 32.422, subclause 4.9, for user consent in Management based MDT, the   Management-Based MDT PLMN List IE is handled by the AMF and provided to the RAN via AMF-RAN signaling messages. In particular, this information is included in relevant AMF messages (e.g. INITIAL CONTEXT SETUP REQUEST, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE). The details of the message and the IE are specified in TS 38.413.

•	SA5 observes that, although a similar naming is used, the above IE/attribute are associated with different procedures and message contexts. From the SA5 perspective, no immediate issues are identified with this usage.

SA5 confirm that reuse existing PLMN list defined in TS 28.622 and TS 32.422 for the geographical area can addresses RAN3’s case for NTN MDT area scope.

SA5 observes that TS 38.413 defines multiple IEs for PLMN list.  SA5 would like to ask RAN3 to clarify any potential relationships between these IEs, the Signalling Based MDT PLMN List and NTN PLMN list.

SA5 is open to further discussion should additional clarification be needed.

2	Actions

To RAN3

ACTION: 	SA5 kindly asks RAN3 to take the above into consideration.

3	Dates of next TSG SA WG 5 meetings

SA5#166		3 – 17 April 2026 		       Malta

SA5#167		18 - 22 May 2026			       China