3GPP TSG-SA5 Meeting #165 	S5-260754d3

Goa, India, 9th February 2026 – 13th February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0546 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.422 corrections on C-MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2026-1-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the incoming LS (R3-258848), RAN3 has agreed the corresponding procedures for Continuous MDT. Additional alignment is needed in our TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the Continuous MDT deactivation procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | misalignments with RAN specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

##### 4.1.1.9.4	NG-RAN activation mechanisms for C-MDT data collections for non-split RAN architecture

For non-split RAN architecture, the C-MDT data collection procedure shall be same as specified in subclause 4.1.1.9.2 with following additions.

1)	The management system shall either send two Trace Session activation requests (one for Immediate MDT, another for Logged MDT) with same TR to the gNB, or send one Trace Session activation request with Job Type (IMMEDIATE_MDT_AND_ LOGGED_MDT) to the gNB. In additional of the trace control and configuration parameters specified in subclause 4.1.1.9.2 step 1, the Trace Session activation request(s) shall contain a TRSR Prefix Configuration parameter, which reserves a TRSR range(s) for the gNB during TRSR assignment. This TRSR Prefix Configuration parameter is used to identify this is a C-MDT activation request. Furthermore, the management system shall ensure that Trace Session activation requests with the same TR are sent to all participating NR-RAN nodes involved in the same C-MDT job.

2)	Upon receiving the Trace Session activation request(s), the gNB shall identify the Trace Session activation request(s) as a C-MDT job, start a C-MDT session, and store the associated configuration parameters.

3)	The gNB and UE shall follow the steps specified in subclause 4.1.1.9.2, step 3 to 8, with following additions:

-	The gNB shall configure the selected UE with an Immediate MDT session and a Logged MDT session using the configurations from the Trace Session activation request(s). A unique TRSR is assigned to the UE for Immediate MDT and Logged MDT. The TRSR is allocated based on the received TRSR Prefix Configuration parameter.

-	Upon transitioning into RRC_CONNECTED, the UE may report Logged MDT measurements, including TR and TRSR, to the gNB, as specified in subclause 4.1.1.9.2 step 7 and 8. If the reported TR is same as the Trace Reference received in the Trace Session Activation request(s), the gNB shall identify the UE as part of the same C-MDT job. If the area-based selection and user consent conditions are satisfied, the gNB shall reconfigure the UE with an Immediate MDT and the gNB shall reconfigure the UE with a Logged MDT if the gNB determines that the Logged MDT duration is about to expire.

-	Upon receiving a UE via a connected-mode inter-gNB mobility procedure, if the TR and TRSR are received and the TR matches an entry in the Trace Reference List received in the Trace Session Activation request(s), the gNB shall identify the UE as part of the same C-MDT job. If the area-based selection and user consent conditions are satisfied, the gNB shall configure this UE with an Immediate MDT and a Logged MDT.

-	In any case when UE moves to new gNB and a new TRSR is assigned, the correlation between the old TRSR and the new TRSR shall be reported by gNB to the TCE.

*** END OF CHANGE ***