3GPP TSG-SA5 Meeting #165	S5-260768

Goa, India, 09 - 13 February 2026

Source:		China Mobile, Orange

Title:	Pseudo-CR on TR 32.801 Add cloud native management scenario for 6G resiliency

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

Based on the clause 5.6.3 in TR 22.870, this contribution proposes to add cloud native Management scenario for 6G resiliency.

Proposed Changes

* * * First Change * * * *

2	References

[X] 	TR 22.870 Study on 6G Use Cases and Service Requirements.

* * * Next Change * * * *

6	6G Management Scenarios

6.2 Management scenarios for support of 6G services

6.2.1 System and Operation Aspects

6.2.1.X Management Scenario #<X>: <Cloud native management scenario>

6.2.1.X.1 Description

To achieve objective of 6G resiliency use case in clause 5.6.3 of TR 22.870 [X], the 3GPP management system should follow cloud native principles to design and achieve management capabilities, for instance, adapt the decoupling characteristics of cloud-nativeness, and support elastic scaling and independent upgrading of network functions themselves. The use case assumes that 6G capabilities (e.g., core network functions, radio access functions, and related services) are designed as a collection of workloads (e.g., NF Deployment) on top of the cloud native infrastructure.

The 3GPP management system can collect real-time resource status information (including compute, memory, storage, and network resources) from the cloud-native infrastructure, and combined with AIML capabilities to realize prediction and detection of network anomalies.

In the various disruption scenarios, such as physical damage, cyber-attacks, and natural disasters, the 3GPP management system supports elastic management of cloud-native workloads, such as coordinating with cloud orchestration to implement dynamic scheduling of  6G NF workloads and infrastructure resources, activating redundant workloads, triggering service traffic rerouting, and automatically ensuring service continuity, thereby improving network resilience and OAM efficiency.

This scenario focuses on cloud-native management capabilities within the scope of SA5, and achieves synergy with 6G system architecture in SA2 and use case requirements in SA1.

6.2.1.1.2 Potential Requirements

[PR 6.2.1.X-1] The 6G 3GPP management system should be able to interact with cloud infrastructure to collect resource status information.

[PR 6.2.1.X-2] The 6G 3GPP management system should be able to configure and request to the external orchestration and management systems to automatically reallocate workloads to maintain service continuity and performance.

[PR 6.2.1.X-3] The 6G 3GPP management system should be able to model NF deployments as part of the NRM.

* * * End of Changes * * * *