3GPP TSG-SA WG6 Meeting #70	S6-255149

Dallas, USA 17th – 21st November 2025

Source:	Nokia

Title:	New Solution for KI#4

Spec:	3GPP TR 23.700-51 V1.1.0

Agenda item:	9.5

Document for:	Approval

Contact:	rajesh_babu.natarajan@nokia.com

1. Introduction

This paper introduces a solution for the KI#4 to support of QoS differentiation for non-3GPP devices

2. Reason for Change

This solution addresses on application enabler layer enhancement to support SEALDD synchronization and connection management among tethering device and multiple tethered devices.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 V1.1.0.

* * * First Change * * * *

# 5	Solutions

## 5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 |
| --- | --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |  |
| Sol #2 |  | x |  |  |  |
| Sol #3 |  |  | x |  |  |
| Sol #4 |  |  |  | x |  |
| Sol #5 | x |  |  |  |  |
| Sol #6 | x |  |  |  |  |
| Sol #X |  |  |  | x |  |

* * * Next Change * * * *

## 5.x	Solution #x: Enhancement to support SEALDD synchronization and connection management among tethering device and multiple tethered devices

### 5.x.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [x].

### 5.x.2	Solution description

This solution addresses on application enabler layer enhancement to support SEALDD synchronization and connection management among tethering device and multiple tethered devices.

Current SEALDD specifications (TS 23.433) define only a single tethered device delay measurement, derived via simple ICMP-based RTT/2, which is then appended to the network-side timing reference. This approach assumes there is only one tethered device and applies its delay as a single fixed adjustment across all flows. However, in multimodal XR/VAL sessions, multiple tethered devices may be simultaneously active — such as a headset for video, earbuds for audio, and a haptic glove or controller for tactile feedback. Each tethered link exhibits different latency, jitter, and reliability characteristics, and thus a single ICMP-based RTT measurement from one device is insufficient to characterize the delay conditions across all modalities. While current specifications recognize tether delay measurement as a requirement, they do not specify mechanisms for per-device (or per-flow) tethered delay modelling, nor do they provide means for integrating such measurements into network timing and QoS control.

This solution addresses the above requirements.

##### 5.x.2.1	SEALDD enabled Tethering link measurement and provisioning for XR application

##### 9.12.2.3.x SEALDD measurement for Multimodal flows synchronization with multiple tethered devices Figure 9.12.2.3.x-1 illustrates the procedures of SEALDD measurement for Multimodal flows synchronization with multiple tethered devices. Pre-conditions: - The VAL server has discovered and selected the SEALDD server by CAPIF functions. - The SEALDD server has been provisioned with a multi-modal SEALDD policy including the synchronization threshold, as specified in clause 9.10.3.1. - The SEALDD client(s) has been provisioned with a multi-modal SEALDD policy, as specified in clause 9.10.2.4. Figure 9.12.2.3.3-1: SEALDD measurement based for Multimodal flows synchronization with multiple tethered devices 1. An on-going multi-modal data transmission connection is established according to the steps 1-8 of clause 9.12.2.1.2. 2. The regular data transmission connection is established, with the information received in step1. The connection between the SEALDD server and 3GPP UE is established as defined in clause 9.2.2.2. 3. The SEALDD transmission quality measurement is defined in clause 9.7.2.3. The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server. 4. The SEALDD client measures the tethering link quality measurements with different tethering devices. 5. The SEALDD client sends the report to the SEALDD server using the transmission quality measurement notification that additionally includes for each of the tethered device the SyncMeasurementReport and SyncOffsetReport. This information is used for further synchronization between the SEALDD client and tethered devices by the SEALDD server as defined in clause 9.12.2.2. 6. Based on the data transmission quality measurement results obtained for multiple associated flow over SEALDD-UU interface in step 5, and the synchronization threshold for multi-modal application as described in pre-condition, the SEALDD server determines the service flow(s) (i.e. address/port for SEALDD-UU flow) that needs to be adjusted among the multiple associated flows in SEALDD-UU interface, and the corresponding required QoS information (i.e. transmission delay). E.g., the SEALDD server evaluates whether the difference of the packet delay measurements for the flows that need to be in synchrony is above the synchronization threshold, and if it is so, the SEALDD server takes the corresponding corrective actions. The SEALDD server may advance or delay the transmission of the DL packets according to the flow assistance information to maintain the synchronization of DL flows. The DL transmission timing advance period correction mode indicates SEALDD server to advance the DL packet transmission by advance period. The DL transmission timing delay period correction mode indicates SEALDD server to delay the transmission of the DL packet by DL delay period units. The SEALDD server may further update the flow alignment assistance information like DL/UL transmission timing advance or delay period to fine-tune as per the data transmission quality measurement results. 7. The SEALDD server sends the AF request to 5GC via N33/N5 with the SEALDD traffic descriptor of the adjusted flow(s) (i.e. address/port for the adjusted SEALDD-UU flow) and the corresponding required QoS information determined in step 5, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [6]. The SEALDD traffic descriptor of the adjusted flow(s) contains the address or port in SEALDD server side, and/or SEALDD client side. After requesting the transmission quality optimization on 5G network with the required QoS for the adjusted flow(s), the multi-flow synchronization of multi-modal application is satisfied. #### 9.7.3.3 SEALDD enabled data transmission quality measurement notification Table 9.7.3.3-1 describes the information flow from the SEALDD server to the VAL server for notifying the transmission quality measurement reports. Table 9.7.3.3-1: SEALDD transmission quality measurement notification Information element; Status; Description. Subscription ID; M; Subscription identifier corresponding to the subscription.. Transmission quality measurement reports list; M; The generated transmission quality results in SEALDD server or SEALDD client, as specified in Table 9.7.3.3-2.. Table 9.7.3.3-2 describes the information elements for the transmission quality measurement reports list, provided by the SEALDD server after performing transmission quality measurement. Table 9.7.3.3-2: SEALDD transmission quality measurement reports list Information element; Status; Description. > Measurement ID; M; Measurement identifiers, (e.g. UL/DL/E2E latency, UL/DL bitrate, UL/DL/E2E packet loss rate, UL/DL/E2E jitter) and crossflow identifiers (e.g., crossflow latency, crossflow bitrate, crossflow packet loss rate, crossflow jitter).. > VAL UE/user ID(s) or flow(s)traffic descriptor(s); O (see NOTE); It indicates the VAL UE(s) or VAL user(s) under SEALDD measurement. For a single VAL UE/user or flow(s)traffic descriptor(s), it can be omitted and the associated measurement values are for the single VAL UE/user. For multiple VAL UEs/users or flow(s)traffic descriptor(s) with reporting granularity set to individual UE/user or flow traffic descriptor, the associated measurement values are for individual VAL UE/user or flow traffic descriptor as indicated in this IE. For multiple VAL UEs/users with reporting granularity set to VAL UE/user group/list or all VAL UEs/users, the associated measurement values are aggregation for all VAL UEs/users or the VAL UE/user group/list and this IE includes the measured VAL UEs/users.. > Crossflow measurement information; O (see NOTE); Represents the crossflow measurement information, e.g., list of pairs traffic descriptor with traffic direction (UL or DL). Example of the crossflow measurement information: {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}.. > Average measurement value; O; The average measurement value of measurement results. > Minimum measurement value; O; The minimum measurement value of measurement results. > maximum measurement value; O; The maximum measurement value of measurement results. > Standard deviation measurement value; O; Standard deviation measurement value of measurement results. > kPercentile measurement value; O; Indicates the kpercentile measurement value of measurement results. > Measurement period; O; Indicates the measurement period. > Timestamp; O; Indicates the timestamp of measurement results. > SyncMeasurementReport; O; Indicates the tethered devices measurement report.. >> SEALDD-UU flow ID; O; Identifier of the SEALDD-UU flow.. >> Delay; O; Indicates the measured delay.. >> Jitter; O; Indicates the measured jitter.. >> Packet loss ratio; O; Indicates the measured packet loss ratio.. >> Clock drift; O; Indicates the measured clock drift.. > SyncOffsetReport; O; Indicates the thethered devices synchorinzation offset report.. >> SEALDD-UU flow IDs; O; Identifier of the SEALDD-UU flows.. >> Sync error; O; Indicates the synchronization error, expressed in msec.. >> Requested Offset; O; Indicates the requested offset, expressed in msec.. >> Validity period; O; Indicates the validity period of the report.. NOTE: These IEs are mutually exclusive..

### 5.x.3	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *