3GPP TSG-SA WG6 Meeting #70	S6-255154

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0096 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the SM data source subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently with the SM data source subscription procedure, the consumer can detect if any changes to the SM data source profiles of its interest by passing the list of SM data source IDs. Notifications will be triggered by the SM server if any changes to the contents of the SM data source profile (e.g. SM data type, availability, update interval etc). But there is no way for the consumer to get to know about the availability of new SM data sources in the area of interest or if the existing SM data sources becomes unavailable (due to de-registration) unleass the consumer does the discovery procedure on a periodic basis which is not efficient. Hence as part of the subscription there sould be an option for the consumet to know about the availability of new SM data sources or unavailability of existing SM data sources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposes to add a event type and triggering crtieria to handle the case described in the reason for change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existing solution is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.6.2.1, 9.5.6.2.2, 9.5.6.3.1, 9.5.6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 9.5.6.2.1	Subscribe SM data source

Figure 9.5.6.2.1-1 depicts the procedure for the authorized VAL server or SEAL server to subscribe to the SM server for the the SM data source specific event. VAL server or SEAL server can get notification information when the the SM server detects subscribed event.

Pre-conditions:

1.	The VAL server or SEAL server has received information (e.g. URI, IP address) related to the SEAL SM server;

2.	The VAL server or SEAL server has received security credentials authorizing it to communicate with the SEAL SM server.

3.	The VAL server or SEAL server has already discovered the SM data sources and obtained the SM data source information.

Figure 9.5.6.2.1-1: SM data source subscription

1.	The requestor (e.g., VAL server or SEAL server) sends a SM data source subscription request to the SM server. The request includes requestor ID, security credentials, a notification endpoint, event to subscribe which triggers the notification etc as specified in Table 9.5.6.3.1-1.

2.	Upon receiving the request from the requestor, the SM server validates if the requestor is authorized for the request. If the requestor is authorized, the SM server creates the subscription and stores the subscription information.

a.	If the subscribed event is for "changes in SM data source(s) profiles", the SM server monitors for updates to the elements of SM data source profile(s) described in Table 7.3.3.2-1 that are associated with the target SM data source(s) provided in the subscription request. The SM server also subscribes to the SEAL Location Management Server services in 3GPP TS 23.434 or NEF services specified in 3GPP TS 23.502 to monitor the location of the SM data source. The SM server also monitors updates to the SM data source profile done via the procedure defined in clause 9.5.3.

b. 	If the subscribed event is for "change of SM Data source(s)", the SM server starts monitoring for the registration and de-registration of the SM data source(s) in the given area of interest provided in the subscription request.

3.	The SM server sends a SM data source subscription response to the requestor with the indication of success or failure as specified in Table 9.5.6.3.2-1.

# * * * * Next change * * * *

##### 9.5.6.2.2	Notify SM data source event

Figure 9.5.6.2.2-1 depicts the notify spatial map event operation between the authorized SM data source consumer (VAL server or SEAL server) and SEAL SM server.

Figure 9.5.6.2.2-1: SM data source notification

1.	The SEAL SM server detects an event that triggers the notification to the subscribers for the corresponding event. SM server determines whether the detected event matches triggering criteria and notifies subscribers accordingly.

a)	If the subscribed event is "changes in SM data source(s) profiles", the SM server detects changes to the SM data source like changes in to the SM data source profile, SM data type, SM data format, SM data area, changes in location, availability state, data source update interval.

b)	If the subscribed event is "changes of SM data source(s)", the SM server detects if any SM data source(s) becomes available or unavailable based on the area of interest provided in the subscription request.

2.	The SEAL SM server notifies the VAL servers or SEAL servers which subscribed the corresponding event. The notification includes a subscription ID and event-specific information.

a)	If the event is "changes in SM data source(s) profiles", the notification message includes the list of all SM data sources identifiers that are updated and the updated elements (e.g. changes in SM data type, SM data format, SM data area, location, availability state, data source update interval).

b)	If the event is "changes of SM data source(s)", SM data source notification notification message includes the information on the detected triggering event, i.e. SM data source(s) IDs and if it becomes available or unavailable.

# * * * * Next change * * * *

##### 9.5.6.3.1	Subscribe SM data source request

Table 9.5.6.3.1-1 shows the subscribe SM data source request sent by VAL server or SEAL server to SEAL SM server to subscribe for SM data source event(s).

Table 9.5.6.3.1-1: Subscribe SM data source request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL server) |
| Security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| Expiry time | O | Proposed subscription expiry time |
| List of SM data source IDs | O (NOTE 1) | ID(s) of the SM data source(s) to subscribe |
| Area of interest | O (NOTE 1) | Three-dimensional area information to discover SM data source(s) to subscribe |
| List of events | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O | List of events to subscribed. Possible values are: - Change of SM Data source(s) |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred and to notify the requestor |
| >> Changes in SM data source profiles | O (NOTE 2) | Triggers notification if any or specified element described in Table 7.3.3.2-1 change is detected. |
| >> Changes in SM data source(s) | O (NOTE 2) | Triggers notification if any or specified SM data source(s) becomes available or unavailable. |
| > Notification intervals | O | Specifies when to notify the detected event, Possible values are: - Event based (e.g., promptly); Or - Periodically (with minimum time between consecutive notifications) - intervals (i.e. minimum time between consecutive notifications) |
| > interval time | O | Time between consecutive notifications (applicable only if notification intervals is set to intervals. |
| NOTE 1: At least one of these elements shall be provided. NOTE 2: At least one of the sub-elements shall be provided. |  |  |

# * * * * Next change * * * *

##### 9.5.6.3.3	Notify SM data source event

Table 9.5.6.3.3-1 shows the SM data source notification sent by SEAL SM server to VAL server or SEAL server to notify about SM data source event(s).

Table 9.5.6.3.3-1: SM data source event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the subscription request |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |
| Event type | M | Type of event detected. One of the event as specified in Table 9.5.6.3.1-1. |
| > Changes of SM data source(s) profile | O (NOTE) | Indicates list of SM data source ID and information about changes in the SM data source. |
| >> SM data source ID | O | Identifier of the SM data source |
| >> SM Data source profile | O | Updated data source profile of the SM data source as specified in clause 7.3.3.2. |
| > Changes of SM data source(s) | O (NOTE) | Information on the SM data source which triggered the notification |
| >> SM data source ID | O | ID of the SM data source |
| >> Type of change | O | Specifies the detected type of change (whether it became available or unavailable) |
| NOTE: This IE includes the list of SM data source ID and the corresponding information if the changes are detected for more than one SM data source. |  |  |