3GPP TSG-SA WG6 Meeting #71	S6-260016

Goa, India 9th – 13th February 2026	(revision of S6-26xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0264 | rev | - | Current version: | 20.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on server-server information flows (MCVideo) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2026-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To ensure architectural consistency and functional parity with other MCX services (MCPTT in TS 23.379 and MCData in TS 23.282), it is necessary to define multi-server procedures for MCVideo ad hoc group calls. This CR introduces missing information flows to support signaling, enabling ad hoc group call across multiple MCVideo system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 6 new clauses, correct 2 clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing definitions of MC multiple system scenarios in MCVideo |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.19.2.3x (new), 7.19.2.6x (new), 7.19.2.8x (new), 7.19.2.9, 7.19.2.9a, 7.19.2.9x (new), 7.19.2.13x (new), 7.19.2.14x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 7.19.2.3x	Ad hoc group call request (MCVideo server – MCVideo server)

Table 7.19.2.3x-1 describes the information flow ad hoc group call request between the MCVideo servers.

Table 7.19.2.3x-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID | M | The MCVideo group ID to be associated with the ad hoc group call |
| MCVideo ID list (see NOTE 1, NOTE 3) | O | MCVideo IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Media parameters of MCVideo server |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCVideo imminent peril call |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCVideo emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: This element is included only when the originating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. |  |  |

* * * Next Change * * *

#### 7.19.2.6x	Ad hoc group call response (MCVideo server – MCVideo server)

Table 7.19.2.6x-1 describes the information flow ad hoc group call response between the MCVideo servers.

Table 7.19.2.6x-1: Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the target MCVideo user |
| Functional alias | O | The functional alias of the target MCVideo user |
| MCVideo ad hoc group ID | M | The MCVideo group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group call request (success or failure) |

* * * Next Change * * *

#### 7.19.2.8x	Ad hoc group call release request (MCVideo server – MCVideo server)

Table 7.19.2.8x-1 describes the information flow ad hoc group call release request between MCVideo servers.

Table 7.19.2.8x-1: Ad hoc group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the ad hoc group call participant |
| Functional alias | O | The functional alias of the ad hoc group call participant |
| MCVideo ad hoc group ID | M | The MCVideo group ID of the ad hoc group call on which call is released |

* * * Next Change * * *

#### 7.19.2.9	Ad hoc group call release response (MCVideo client – MCVideo server)

Table 7.19.2.9-1 describes the information flow ad hoc group call release response from the MCVideo client to the MCVideo server.

Table 7.19.2.9-1: Ad hoc group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the ad hoc group call participant |
| Functional alias | O | The functional alias of the ad hoc group call participant |
| MCVideo ad hoc group ID | M | The MCVideo group ID of the ad hoc group call on which call is released |

* * * Next Change * * *

#### 7.19.2.9a	Ad hoc group call release response (MCVideo server – MCVideo client)

Table 7.19.2.9a-1 describes the information flow ad hoc group call release response from the MCVideo server to the MCVideo client.

Table 7.19.2.9a-1: Ad hoc group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the user authorized to release the ad hoc group call |
| Functional alias | O | The functional alias of the authorized user requesting to release the ad hoc group call |
| MCVideo ad hoc group ID | M | The MCVideo group ID of the ad hoc group call on which call is released |

* * * Next Change * * *

#### 7.19.2.9x	Ad hoc group call release response (MCVideo server – MCVideo server)

Table 7.19.2.9x-1 describes the information flow ad hoc group call release response between MCVideo servers.

Table 7.19.2.9x-1: Ad hoc group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the user authorized to release the ad hoc group call |
| Functional alias | O | The functional alias of the authorized user requesting to release the ad hoc group call |
| MCVideo ad hoc group ID | M | The MCVideo group ID of the ad hoc group call on which call is released |

* * * Next Change * * *

#### 7.19.2.13x	Ad hoc group call leave request (MCVideo server – MCVideo server)

Table 7.19.2.13x-1 describes the information flow ad hoc group call leave request between MCVideo servers.

Table 7.19.2.13x-1: Ad hoc group call leave request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the ad hoc group call participant leaving the call |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call on which call is on-going |
| Reason to leave (See NOTE) | O | Carries the reason of why the MCVideo client is leaving ongoing ad hoc group call (e.g. Due to migration). |
| NOTE: This element must be included when originally sent from client to server (see clause 7.19.2.13a). This element may be included when originally sent from server to client (see clause 7.19.2.13) |  |  |

* * * Next Change * * *

#### 7.19.2.14x	Ad hoc group call leave response (MCVideo server – MCVideo server)

Table 7.19.2.14x-1 describes the information flow Ad hoc group call leave response between MCVideo servers.

Table 7.19.2.14x-1: Ad hoc group call leave response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the ad hoc group call participant leaving the call |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call on which call is on-going |
| Result (See NOTE) | O | Result of the ad hoc group call leave request – success or failure. |
| NOTE: This element must be included when originally sent from server to client (see clause 7.19.2.14a). This element must not be included when originally sent from client to server (see clause 7.19.2.14) |  |  |

* * * End of Changes * * *