3GPP TSG-SA WG6 Meeting #70	S6-255337

Dallas, United States 17th – 21st November 2025	(revision of S6-255074)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0503 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving Editor’s Notes on server-to-server messages (MCPTT) information flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Deleting the editor´s note in clauses 10.19.2.6 Editor's Note: It is FFS if the server to server message is needed in a call request or response message. - Adding information flows Ad hoc group call leave request/response and Ad hoc group call release request/response between MCPTT servers, to be consitent through the document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleting Editor´s notes, adding 5 new information flows between MCPTT servers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguities in specification. Notes remaining in specification and missing server to server IF may mislead of server to server communication process. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.3x (new), 10.19.2.6, 10.19.2.8x (new), 10.19.2.9x (new),10.19.2.13x (new), 10.19.2.14x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes in the corresponding procedures (10.19.3.2) are discussed in CR0504 (S6-255075). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 10.19.2.3x	Ad hoc group call request (MCPTT server – MCPTT server)

Table 10.19.2.3x-1 describes the information flow ad hoc group call request between the MCPTT servers.

Table 10.19.2.3x-1: Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| MCPTT ID list (see NOTE 1, NOTE 3) | O | MCPTT IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Offered Media parameters of MCPTT server |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCPTT imminent peril ad hoc group call |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCPTT emergency ad hoc group call |
| Preconfigured MCPTT group ID | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: This element is included only when the originating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. |  |  |

* * * Second Change * * *

#### 10.19.2.6	Ad hoc group call response (MCPTT server – MCPTT server)

Table 10.19.2.6-1 describes the information flow ad hoc group call response between the MCPTT servers.

Table 10.19.2.6-1: Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MCPTT user |
| Functional alias | O | The functional alias of the target MCPTT user |
| MCPTT ad hoc group ID | M | The MCPTT group ID associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group call request (success or failure) |

* * * Third Change * * *

#### 10.19.2.8x	Ad hoc group call release request (MCPTT server – MCPTT server)

Table 10.19.2.8x-1 describes the information flow ad hoc group call release request between MCPTT servers.

Table 10.19.2.8x-1: Ad hoc group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the user authorized to release the ad hoc group call |
| Functional alias | O | The functional alias of the authorized user requesting to release the ad hoc group call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of the ad hoc group call on which call is released |

* * * Fourth Change * * *

#### 10.19.2.9x	Ad hoc group call release response (MCPTT server – MCPTT server)

Table 10.19.2.9x-1 describes the information flow ad hoc group call release response between MCPTT servers.

Table 10.19.2.9x-1: Ad hoc group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the user authorized to release the ad hoc group call |
| Functional alias | O | The functional alias of the authorized user requesting to release the ad hoc group call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of the ad hoc group call that is released |

* * * Fifth Change * * *

#### 10.19.2.13x	Ad hoc group call leave request (MCPTT server – MCPTT server)

Table 10.19.2.13x-1 describes the information flow ad hoc group call leave request between MCPTT servers.

Table 10.19.2.13x-1: Ad hoc group call leave request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant leaving the call |
| Functional alias (see NOTE 1) | O | The functional alias of the ad hoc group call participant leaving the call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call on which call is on-going |
| Reason to leave (see NOTE 2) | O | Carries the reason of why the MCPTT client is leaving ongoing ad hoc group call (e.g. due to migration). |
| NOTE 1: This element may only be included when originally sent from client to server (see clause 10.19.2.13a). NOTE 2: This element must be included when originally sent from client to server (see clause 10.19.2.13a). This element may be included when originally sent from server to client (see clause 10.19.2.13). |  |  |

* * * Sixth Change * * *

#### 10.19.2.14x	Ad hoc group call leave response (MCPTT server – MCPTT server)

Table 10.19.2.14x-1 describes the information flow Ad hoc group call leave response between MCPTT servers.

Table 10.19.2.14x-1: Ad hoc group call leave response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant leaving the call |
| Functional alias (see NOTE 1) | O | The functional alias of the ad hoc group call participant leaving the call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call on which call is on-going |
| Result (see NOTE 2) | O | Result of the ad hoc group call leave request – success or failure. |
| NOTE 1: This element may only be included when originally sent from server to client (see clause 10.19.2.14a). NOTE 2: This element must be included when originally sent from server to client (see clause 10.19.2.14a). This element must not be included when originally sent from client to server (see 10.19.2.14). |  |  |

* * * End of changes * * *