3GPP TSG-SA WG6 Meeting #71	S6-260420

Goa, India 9th – 13th February 2026	(revision of S6-260349)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0406 | rev | 1 | Current version: | 19.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the EN related to use of Application ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2026-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Below EN exist in clause 7.17.7.1.1 and this CR proposes to delete the EN. Editor's note: Use of Application ID to target the FD to a specific application use is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal is to delete the editors notes and replace with NOTE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 7.17.7.1.1	Ad hoc group standalone FD request (MCData client – MCData server)

Table 7.17.7.1.1-1 describes the information flow for the Ad hoc group standalone FD request sent from the MCData client to the MCData server.

Table 7.17.7.1.1-1: Ad hoc group standalone FD request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the FD request |
| Functional alias | O | The associated functional alias of the MCData user sending the FD request |
| MCData ad hoc group ID (see NOTE 1) | M | The MCData group ID of the ad hoc group to which the file is to be sent |
| Preconfigured MCData group ID (see NOTE 2) | O | The identity of the MCData group whose configuration (e.g. security related information) is to be applied for ad hoc group file distribution. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Emergency indicator (see NOTE 3) | O | Indicates that the standalone file distribution request is for emergency ad hoc group standalone file distribution service |
| Imminent peril indicator (see NOTE 3) | O | Indicates that the standalone file distribution request is for imminent peril ad hoc group standalone file distribution service |
| Disposition indication | O | Indicates whether file download completed report is expected or not |
| Download indication | O | Indicates mandatory download |
| Location | O | Location of the Originating MCData user sending the file |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Application identifier (see NOTE 4) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| Content reference | M | URL reference to the content and file metadata information |
| Deposit file indication | O | Indicates whether the file to be stored into the MCData message store account of the MCData user |
| NOTE 1: The MCData ad hoc group ID is determined prior to by using the Determine ad hoc group request. NOTE 2: If end-to-end encryption is required, then this element is included and the value is determined prior to by using the Determine ad hoc group request. NOTE 3: If used, only one of these information elements shall be present. NOTE 4: The application identifier shall be included only if the payload destination type indicates that the FD message is for application consumption. Use of Application ID to target the FD to a specific application use is out of scope of the present document. |  |  |

* * * End Change * * *