3GPP TSG-SA WG6 Meeting #70	S6-255338

Dallas, United States 17th – 21st November 2025	(revision of S6-255252)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0505 | rev | 1 | Current version: | 19.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 10.19.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrections to informtion flow table in 10.19.2.3. This information flow defines the message IEs in the AHGC request message between the primary MCPTT server of the AHGC originator and the the group host MCPTT server. There is no procedure defined in the TS on how this message is used. But the definition of “Group host MCPTT server” in clause 3.1 and mentioned in the TS for non AHGC procedures may give clues. In clause 3.1 the following definition is defined: Group host MCPTT server: The MCPTT server within an MCPTT system that provides centralised support for MCPTT services of an MCPTT group defined in a group home MCPTT system. The following information flow tables mention “group host MCPTT server” saying it is from MCPTT server to the group host MCPTT server: - 10.6.2.2.1 MCPTT emergency group call request - 10.6.2.2.5 MCPTT imminent peril group call request - 10.6.2.2.8 Group call request (MCPTT server – group host MCPTT server) - 10.6.2.2.20 Group join request - 10.6.2.2.21 Group join response - 10.19.2.3 Ad hoc group call request (MCPTT server – group host MCPTT server) The following procedures mention “group host MCPTT server”: - Clause 10.6.2.3.1.1.2, The group host MCPTT server is the “primary” MCPTT server in step 5. - Clause 10.6.2.4.1.2 , The group host MCPTT server is the “primary” MCPTT server in step 1. - Clause 10.12, the group host MCPTT server is the Partner MCPTT system floor control server. In summary, there is NO procedure figure shows “group host MCPTT server” and from the mentions in existing procedures we understand that the group host MCPTT server in single MC MCPTT system it is the primary MCPTT server. Based on the above analysis, the information flow in 10.19.2.3 need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the information flow table to correct the error accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect normative text will misguide stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.19.2.3	Ad hoc group call request (MCPTT server – group host MCPTT server)

Table 10.19.2.3-1 describes the information flow ad hoc group call request between the MCPTT server and the group host MCPTT server.

Table 10.19.2.3-1: Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| MCPTT ID list | M | MCPTT IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Offered Media parameters of MCPTT server |
| Implicit floor request (see NOTE 2) | O | Indicates that the originating client requests the floor |
| Broadcast indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is an MCPTT imminent peril ad hoc group call |
| Emergency Indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is an MCPTT emergency ad hoc group call |
| Preconfigured MCPTT group ID | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Criteria for determining the participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: If used, only one of these information elements is present. NOTE 2: This information element shall be included only when the originating client requests the floor. |  |  |