3GPP TSG-SA WG6 Meeting #70	S6-255415

Dallas, United States 17th – 21st November 2025	(revision of S6-255080)

Source:	InterDigital, KPN N.V.

Title:	KI #7 evaluation and conclusions

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Quang Ly, <quang.ly@interdigital.com>

1. Introduction

This contribution proposes an overall evaluation and conclusions for key issue #7.

2. Reason for Change

KI #7 evaluation is required for overall KI #7 conclusions. Conclusions for KI #7 are also provided.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

## 10.8	Key Issue #7: VFL sample alignment across VAL servers

Key Issue #7 studied VFL sample alignment performed by AIMLE servers to enable VFL operation. The open issues studied in KI #7 are as follows:

1.	Whether and how to enhance VFL operation considering also VAL servers, AIMLE clients and AIMLE servers as VFL participants.

2.	Whether and how to support/enhance sample alignment also considering VAL servers / AIMLE servers / AIMLE clients as VFL participants.

There are two solutions that address KI #7. Solutions #15 addesses VFL sample alignment involving AIMLE clients while solution #22 addresses VFL sample alignment involving VAL servers serving as VFL clients.

Solution #15 addresses open issues 1 by proposing to enhance existing VFL procedure described in clause 8.18 of 3GPP TR 23.482 [2]. The enhancements include client feature selection and sample binding requests sent from an AIMLE server to AIMLE clients as part of VFL alignment procedure. The client feature selection request is used by the AIMLE server to activate or de-activate feature ID(s)/feature list maintained by AIMLE clients for each data domain. The sample binding request is used by the AIMLE server to provide sample binding criteria and sample pool for AIMLE clients to bind features in available datasets to the sample pool as part of VFL sample alignment.

Solution #22 addresses open issues 1 and 2 by proposing procedures in support of sample alignment by VAL servers. Enhancements to FL registration procedure described in clause 8.4 of 3GPP TR 23.482 [2] for VFL client registration are proposed to provide VFL related metadata to discover and select VFL clients and to assist with VFL sample alignment. A new procedure for VFL training and inferencing is proposed for AIMLE servers to assist with VFL operations involving multiple VAL servers.

* * * Next Change * * * *

## 11.x	Conclusions of key issue #7

The following solutions will be considered for normative work for KI #7:

- Solution #15 provides support for VFL sample alignment involving AIMLE clients.

- Solution #22 provides support for VFL sample alignment involving VAL servers serving as VFL clients.

NOTE:	Consideration for merging solutions #15 and #22 will be made during normative phase.

* * * End of Change * * * *