3GPP TSG-SA WG6 Meeting #70	S6-255539

Dallas, USA, 17th – 21st November 2025	(revision of S6-255171)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on update to solution#7

Spec:	3GPP TR 23.700-26 0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR propose to add the information flow table into solution#7.

2. Reason for Change

The information flow table is missing from solution#7.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.2.0.

* * * First Change * * * *

7.7	Solution #7: Application AIoT Task execution

7.7.1	Solution description

7.7.1.1	General

This solution address key issue#2, 3 and 4 to introduce a solution for application AIoT task execution. The AIoT tasks from the application layer cannot be directly aligned with the AIoT NEF interface. It is necessary to translate AIoT tasks into AIoT inventory and/or command towards the 3GPP core network.

7.7.1.2	Procedures

The figure 7.7.1.2-1 shows a high-level description of the AIoT task execution initiated by AIoT application layer entity (e.g., VAL server) towards the AIoT enabler server.

Pre-conditions:

-	The VAL consumer (e.g., VAL server) has provisioned the application layer related information as described in sol#6.

Figure 7.7.1.2-1: AIoT Task initiation

1.	AIoT task is generated in the AIoT application layer according to the service requirements. Usually, the AIoT task is described with AIoT service scenario with the information of application object and AIoT application service region as described in table 7.7.1.2-1. The AIoT task is initiated and sent to AIoT enabler server with AIoT task information.

Table 7.7.1.2-1: Example of Application AIoT task description

Application AIoT Tasks information

Application Object ID(s)

AIoT application region ID

AIoT service scenario ID

2.	The AIoT enabler server determines to invoke the network Inventory and/or Command service API towards the 3GPP core network. The value of input parameters of the Inventory and/or Command service API is determined based on the received AIoT task information and the provisioned service data as described in sol#6 in clause 7.6.

For example, for the inbound task in warehouse A at check point X, the AIoT enable layer needs to determine the target area information, which is associated to “warehouse A check point X” from table 7.6.1.3.1-4. Due to the AIoT service scenario ID indicating it is the “inbound” scenario, then a rapid periodic AIoT inventory service should be performed with a time interval of result aggregation being set Xms. The AIoT inventory towards the 3GPP network may be performance multiple times as instructed by the AIoT network services requirements as described in table 7.6.1.3.1-5.

In another example, for the monitor task in warehouse A, the AIoT enable server needs determines the corresponding AIoT network services requirements and the Interested AIoT service events as described in table 7.6.1.3.1-5. Then the AIoT enabler server performs periodically inventory towards target service area corresponding to the service region area of warehouse A (the mapping is provisioned in table 7.6.1.3.1-4) with a time interval of result aggregation being set Yms.

3.	The final AIoT service API invocation(s) is initiated towards the 3GPP core network.

4.	The 3GPP core network starts the AIoT services operations

5.	The AIoT enabler server sends a AIoT task initiate response information to the application layer to notify whether the AIoT task is initiate successfully or not, then the application layer decides whether to wait for the result or republish the AIoT task.

NOTE 1:	The step 5 can be performed before step 4.

NOTE 2:	The step 1 and step 5 can use the subscribe/notification model.

7.7.1.3	Information flows

7.7.1.3.1	AIoT task initiate request

Table 7.7.3.1-1 describes the AIoT task initiate request from the AIoT application layer consumer (e.g., VAL server) towards the AIoT enabler server.

Table 7.7.1.3.1-1: AIoT task initiate request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identifier | M | The identifier of the requester. |
| Notification endpoint | M | The endpoint to receive the reported AIoT task execution result. |
| AIoT Service scenario ID | M | Represent the AIoT service scenario that the AIoT task initiate request is targeting for. |
| Request target Information | M | The information associated with the AIoT request. |
| > Application Object ID(s) | M | The target object of the AIoT task initiate request is targeting for. |
| AIoT application region ID | O | The application region where AIoT task initiate request is targeting for. |
| Reporting Interval | M | Time interval for AIoT enabler server to report results to AIoT application layer. |

7.7.1.3.2	AIoT task initiate response

Table 7.7.1.3.2-1 describes the AIoT task initiate response from the AIoT enabler server to the AIoT application layer consumer (e.g., VAL server).

Table 7.7.1.3.2-1: AIoT task initiate response

| Information element | Status | Description |
| --- | --- | --- |
| Success | M (NOTE) | Indicate the request is successfully processed. |
| CorrelationID | M | The identifier of the AIoT task execution which will be used further to correlate the notifications sent from the AIoT enabler server. |
| Failed | M (NOTE) | The AIoT task initiate request execution failed. |
| > Failure Cause | O | The Reasons for AIoT task initiate request execution failure. |
| NOTE: Only one of them shall present. |  |  |

7.7.2	Architecture Impacts

This solution introduces the AIoT task transformation capability to the AIoT enabler server.

7.7.3	Corresponding APIs

This solution introduces a new AIoT task execution service.

7.7.4	Solution evaluation