3GPP TSG-SA WG6 Meeting #70	S6-255545

Dallas, USA, 17th – 21th Nov 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	Overall evaluation for KI#1

Spec:	3GPP TR 23.700-26 v0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes to provide the overall evaluation for KI#1.

2. Reason for Change

To complete the overall evaluation for the KI#1.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-26 v0.2.0.

* * * First Change * * * *

8	Overall evaluation

This clause will provide evaluation of different solutions.

8.1	General

This clause will provide a general description of the overall evaluation.

8.2	Overall evaluation of key issue #1

The KI#1 mainly studies the following open issue:

-	Whether and how to enhance the application enablement architecture /functional model or new application enabler to support the Ambient IoT services (i.e., AIoT inventory and command services).

To address the open issues aforementioned, there are 3 solutions proposed as following:

Solution #1 introduces a new architecture and functional model in the application enablement layer to support the AIoT services. Specifically, this solution introduces the on-network functional model, the relevant functional entities (including the AIoT management server, the AIoT management client, the VAL server, the VAL client, etc.) and the reference points included in the new architecture for the AIoT services.

To align with the system architecture for AIoT services specified in clause 4.2 of 3GPP TS 23.369 [3], the AIoT management client in the new architecture for the application enablement layer only acts as a service consumer which could request the AIoT services to the AIoT management server over AM-UU reference point, but it will not act as a UE reader to obtain and report the AIoT device information. And the off-network functional model for AIoT application enablement architecture is not supported for Sol#1.

Solution #2 also introduces a new architecture and functional model in the application enablement layer to support the AIoT services. The main difference between Sol#1 and Sol#2 is there is neither VAL client nor AIoT client in Sol#2 to provide the application enabler layer support for AIoT services from the client side. And the application enablement architecture layer can only be enhanced for AIoT service at the server side which acts as a SEAL layer function entity. Besides, the off-network functional model for AIoT application enablement architecture is not supported for Sol#2.

Solution #3 also introduces a new architecture and functional model in the application enablement layer to support the AIoT services. The solution highlights the AIoT services entities should be part of Application enabler layer and not the services layer. And it also introduces the on-network functional model, the relevant functional entities (including the AIoTE server, the AIoTE client, the VAL/SEAL server, the VAL/SEAL client, etc.) and the reference points included in the new architecture for the AIoT services.

Editor’s note: The overall evaluation for the solutions may be updated when the relevant solution updates.

8.3	Overall evaluation of key issue #2

This clause will provide an overall evaluation for KI#2.

8.4	Overall evaluation of key issue #3

This clause will provide an overall evaluation for KI#3.

8.5	Overall evaluation of key issue #4

This clause will provide an overall evaluation for KI#4.

* * * End of Change * * * *