3GPP TSG-SA WG6 Meeting #70	S6-255129

Dallas, USA, 17th – 21th Nov 2025	(revision of S6-25xxxx)

Source:	CATT, Ericsson

Title:	Update Sol#12 to fix the EN

Spec:	3GPP TR 23.700-26 v0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes to update the Sol#12 to resolve the EN in TR 23.700-26.

2. Reason for Change

This paper is proposed to fix the following EN for Sol#12.

Editor’s note: Whether and how the procedures of AIoT Inventory subscription and AIoT monitoring subscription can be combined or not is FFS.

The AIoT Inventory subscription procedure in Sol#9 focuses on how to support the periodic and event-triggered AIoT inventory service request. However, the AIoT monitoring subscription procedure in Sol#12 focuses on how to support the monitoring request for AIoT devices. These are two different procedures for different services which should not be combind toghether.

What’s more, in the existing approved solutions, how to define the AIoT service requests from the VAL server is still FFS. Some solutions propose to define one AIoT service request with different service type (e.g. inventory, command). Some solutions propose to define the separated AIoT inventory service request, AIoT command service request and AIoT monitoring service request. And some solutions propose to add new service request like AIoT discovery requests instead of AIoT inventory/command requests. Thus, I propose to discuss the definition for the AIoT service requests in the normative phase, include whether to combine the AIoT Inventory subscription procedure and the AIoT monitoring subscription procedure or not.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-26 v0.2.0.

* * * First Change * * * *

7.12	Solution #12: Support of monitoring requests for AIoT devices

7.12.1	Solution description

7.12.1.1	General

This solution addresses the KI#2 and KI#3. Some application servers (e.g. a logistics company) may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations, and then send warnings to the AIoT devices in case they deviate the planned  trajectory/direction.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

- 	Monitoring the location of requested AIoT devices;

- 	Report the location information for the target AIoT devices;

- 	Report to indicate the target AIoT device is deviating or will deviate the planned  location trajectory.

7.12.1.2	Procedure of Monitoring requests for AIoT devices

Figure 7.12.1.2-1 illustrates the high-level procedure of monitoring requests for AIoT devices.

Pre-condition:

-	The AIoT enabler is the new application enabler in the application enablement layer which supports the AIoT related operations and services.

Figure 7.12.1.2-1: Procedure of Monitoring requests for AIoT devices

1.	The VAL server sends an AIoT monitoring subscription request to the AIoT enabler, including the monitoring service type (e.g., location), service ID, target area information, target AIoT device information, triggering conditions (e.g., periodic or event triggered reporting), time intervals for periodic reporting, monitoring time duration, events for event-triggered reporting, AIoT device group member deviation indicator, the group ID for a group of AIoT devices and the planned location trajectory if the AIoT device group member deviation indicator is included, etc.

The AIoT device group member deviation indicator indicates whether and which AIoT device (or type of AIoT devices) is deviating (or will deviate) from other similar devices or the planned location trajectory in a group.

The information about the target AIoT Device(s) may include Filtering Information as described in clause 5.8 of 3GPP TS 23.369[3].

2.	The AIoT enabler checks whether the VAL server is authorized to request the AIoT monitoring subscription request, may be based on e.g. the pre-configurations or operator policies.

3.	If the request is authorized, the AIoT enabler sends an AIoT monitoring subscription response to the VAL server.

4.	Then, the AIoT enabler invokes Nnef_AIoT_Command request to the 3GPP CN periodically as specified in clause 6.2.3 of 3GPP TS 23.369 [3] with location information requested IE to obtain the requested AIoT device data including the location information of AIoT devices. The Command type is Read.

5.	Upon received the AIoT device data and related location information, the AIoT enabler stores  them based on the AIoT monitoring subscription request. If the subscription request includes the AIoT device group member deviation analytics indication, the AIoT enabler may interact with ADAES to obtain the predicted location information and the deviation analytics for the requested AIoT devices as specified in clause 8.15 of 3GPP TS 23.436 [7].

The AIoT enabler will send the obtained AIoT device data (including related location information) to the ADAES who doesn’t need to sends a data collection subscription request to the Data Producer (e.g. 5GC). Based on the received AIoT device data, the ADAE server analyses and sends location-related UE group analytics notifications to the AIoT enabler which may include the location information, the predicted location information the deviation analytics for the group of AIoT devices, etc. The deviation analytics mainly indicate which AIoT device (or type of AIoT devices) is deviating (or will deviate) from other similar devices or the planned location trajectory  in a group.

6. 	The AIoT enabler checks if the received location-related analytics in step 5 could meet the service requirements or not. And if not, the step 4 and step 5 may need to be operated repeatedly.

7.	The AIoT enabler reports the monitoring results for the requested AIoT device to the VAL server via sending AIoT monitoring notification message. The report may include the AIoT device ID, the location information for the AIoT device,  the predicted location information for the AIoT device, the deviation analytics, the device availability status, etc.

Editor’s note: Whether and how the procedures of AIoT Inventory subscription and AIoT monitoring subscription can be combined or not is FFS.

Editor’s note: Whether and how the ADAE analytics specified in clause 8.15 of 3GPP TS 23.436 can be used or enhanced for the direction and the deviation predictions of the requested AIoT devices is FFS.

7.12.2	Architecture Impacts

This solution proposes a new architecture (i.e., new AIoT enabler) to support AIoT services.

For the new architecture, please check the Sol#1 for KI#1 for more details.

7.12.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

7.12.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *