3GPP TSG-SA WG6 Meeting #70	S6-255579

Dallas, USA, 17th – 21th Nov 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	Update Sol#4 to fix the EN

Spec:	3GPP TR 23.700-15 v0.2.0

Agenda item:	9.13

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes to update the Sol#4 to fix the EN.

2. Reason for Change

The contribution mainly proposes to update the Sol#4 to fix the following EN and also makes some clarification and correction for the whole procedure.

Editor's note:	How the LMS tracks the detected UAVs that without UE modules is FFS.

First, the step 5 in the procedure of 6.5.1.1 is optional and it only applies when the 3GPP CN doesn’t expose the location or the location trajectory for the detected UAVs. Secondly, if the step 5 operates, the LMS cannot monitor and track the location of the UAVs without UE modules (e.g. USIM). The LMS only can monitor and track the location of the UAVs with UE modules and UAV IDs. The UAV IDs come from the service requests initiated from the VAL server or the exposure from 3GPP CN for the detected UAVs. So it’s better to add the following NOTEs to explain and resolve the above EN.

NOTE 1:	The LMS in the SEAL cannot monitor and track the location of the UAVs without UE modules (e.g. USIM).

NOTE 2:	The LMS in the SEAL can monitor and track the location of the UAVs with UE modules using the UAV IDs which included in the service requests in step 1 or the detected UAV IDs exposed from 3GPP CN described in step 4.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-15 v0.2.0.

* * * First Change * * * *

6.5	Solution #4: Enhancements of UAV services utilizing the sensing results

6.5.1	Solution description

This solution addresses the KI#2.

The following high-level solution principles apply to this solution:

-	A high-level procedure for sensing request and exposure is proposed with a new Sensing enabler in the application enablement layer.

-	The solution assumes the 3GPP core network will expose the sensing results related to UAVs to the AF (e.g., VAL server).

6.5.1.1	Procedure of detecting UAVs utilizing sensing results

Figure 6.5.1.1-1 illustrates the high-level procedure of detecting UAVs utilizing sensing results.

Pre-condition:

-	The following Sensing enabler is a new enabler that implements the sensing functions in the application enablement layer.

Figure 6.5.1.1-1: Procedure of detecting UAVs utilizing sensing results

1.	The VAL server sends a UAV detect request to the UAE to detect the UAVs in a certain area, including the service type (i.e., UAV detection, UAV tracking), service ID, sensing service requirements for the detected UAVs (e.g., location accuracy, velocity, latency, sensing resolution, etc.), the target area, the reporting conditions (e.g., periodic reporting or event-triggered reporting), the reporting time intervals if the request is periodic, time duration, optional tracked object information (e.g., UAV ID, normal flight trajectory) if the object needs to be tracked, etc.

2.	The UAE checks whether the VAL server is authorized to invoke such request based on e.g. the pre-configurations or the operator policies.

3.	If the request is authorized, the UAE sends the UAV related sensing results request to the sensing enabler to ask for the UAV related sensing results.

4.	The sensing enabler may invoke the sensing related service request to the 3GPP CN based on the service requests received in step 1and obtain the sensing results from the 3GPP CN via NEF if the SM server is an untrusted AF or interact with the SF entity in 5GC directly as a trusted AF.

Editor's note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

5.	Upon received the exposure from network layer, the sensing enabler may determine the detected UAVs that entering the target area and then the sensing enabler may interact with LMS to track the detected UAVs with UE modules and obtain the location trajectory (including the velocity, direction, altitude, etc.) for them in the requested time duration as specified in clause 9.3.5 of 3GPP TS 23.434 [2].

6.	The sensing enabler aggregates and combines the sensing results obtained in step 4 and step 5 to determine all of UAVs entering the target area as well as their flight trajectories, and then the sensing enabler compares the detected UAV flight trajectory with the normal flight trajectory included in step 1 to identify if the UAVs entering the target area are authorized  or not.

7.	The sensing enabler notifies the UAE for the UAV related sensing results, including all UAVs entering the target area and the unauthorized UAVs that not allowed entering the target area.

8.	The UAE sends the analysed sensing results to the VAL server via UAV detect response message, including all UAVs entering the target area and the unauthorized UAVs that not allowed entering the target area.

6.5.2	Architecture Impacts

This solution proposes a new sensing enabler to support the sensing service.

For the new architecture for the sensing enabler, please check the solution#1 for the KI#1.

6.5.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *