3GPP TSG-SA WG6 Meeting #70	S6-255406

Dallas, United States 17th – 21st November 2025	(revision of S6-255037s)

Source:	KPN N.V, InterDigital

Title:	Completion and Evaluation of Solution #22 Sample Alignment Enablement for VAL Servers in Vertical Federated Learning

Spec:	3GPP TR 23.700-83 1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

This contribution completes Solution #22 by correcting and finalizing the procedure, and by adding the necessary API updates in accordance with the proposed approach. It also provides a comprehensive evaluation of the procedure.

2. Reason for Change

Solution #22 still contains several unresolved Editor Notes, and the procedure need to be ammended to fit with defined API(s) in this contribution. This contribution addresses these gaps by correcting and completing the procedure, incorporating the required API updates, and providing a full evaluation of the procedure.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 V1.1.0.

* * * First Change * * * *

#### 7.22.1.2	Procedures

##### 7.22.1.2.1	VFL client registration

FL member registration is specified in clause 8.4 of 3GPP TS 23.482 [2]. The FL members can be VAL servers that perform the registration via VAL servers. To support VFL registration the procedure needs to be further updated. For this purpose, the following changes are needed in clause 8.4.2 of 3GPP TS 23.482 [2] in the corresponding steps.

- For FL registration request, when VAL servers acting as VFL clients register as candidate FL members, they provide additional metadata including supported feature sets, and local UE identifiers in their sample space. The FL member endpoint information is also provided in the registration for communication with the FL member such as initiating VFL training/inference as in step 4 of clause 7.22.1.2.2.

- The AIMLE server before storing the VFL client registration, first maps each local UE identifier in the sample space to a corresponding unified UE identifier (e.g., GPSI such as MSISDN) via interaction with the Core Network functions and stores it along with its registration. This unified UE identifier, however, shall not be accessible to the VAL servers and is only used by the AIMLE server for discovery purposes.

- There is no change needed in the FL member registration response.

* * * Next Change * * * *

##### 7.22.1.2.2	VFL training and inference

Assumptions:

- VAL server acting as VFL server coordinates other VAL servers acting as a VFL client to perform VFL training/inference.

- The sample space is associated with UEs which are locally identified in the VFL client and VFL server by AF-Specific UE IDs, IP addresses, or similar identifiers.

Pre-conditions:

- The VAL servers acting as VFL clients and the VAL server acting as the VFL server have registered with the AIMLE server as described in clause 7.22.1.2.1, providing metadata including supported feature sets, and local UE identifiers.

- The AIMLE server maps each local UE identifier to a corresponding unified UE identifier (e.g., GPSI such as MSISDN) via interaction with the Core Network functions as described in clause 7.22.1.2.1.

Figure 7.22.1.2.2-1: VFL training/inference with sample alignment procedure

- The VFL server sends a VFL training/inference request including required feature ID list, and required sample ID list (that is local UE identifier(s)) .

- The AIMLE server discovers and selects the desired VFL clients from registered VFL clients supporting the required feature ID and sample ID list. The UE identifier(s) in the sample ID list of the VFL clients and VFL server are matched by using the Core Network functions to resolve local identifiers (e.g. AF specific UE ID, IP addresses) into common UE identifiers (e.g, GPSI). The VFL clients are identified using the common UE identifiers for matching the sample space.

- The AIMLE server assigns a sample correlation ID for each UE identifier in the sample ID list for consistent referencing during training and inference while enabling VFL training/inference using their local identifiers of the UEs.

- The AIMLE server sends a VFL training/inference initiation request to the selected VAL servers acting as VFL clients, including the assigned sample correlation IDs to match the correlation IDs in the VFL training/inference task to their corresponding local identifiers of the UEs.

- The VAL servers acting as VFL clients send the initiation response to the AIMLE server confirming participation to the VFL training/inference.

- The AIMLE server sends a VFL training/inference response to the requesting VFL server, including the sample correlation IDs corresponding to the aligned UEs.

- The VFL server and the VFL clients proceed with the training/inference using the aligned sample ID list, with sample correlation IDs ensuring consistent sample alignment across participants.

* * * Next Change * * * *

### 7.22.3 	Corresponding APIs

7.22.3.1	VFL member registration request

The FL member registration request in Clause 8.4.4.2 of 3GPP TS 23.482 [2] can be expanded to enable registration of VFL members as shown in Table 7.22.3.1-1 with the additions indicated in bold.

Table 7.22.3.1-1: 3GPP TS 23.482 [2] Table 8.4.4.2-1: FL member registration request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration request. |
| FL member ID | M | The identifier of the candidate FL member. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| FL member endpoint | O | Endpoint information (e.g. URI) used to communicate with the FL member. |
| FL member supported AI/ML role | M | The supported AI/ML role of FL member which can be used as FL server or FL client. |
| Supported FL task types | O | The supported FL task type by the FL member such as VFL and HFL. |
| Feature ID list | O | List of features supported by the FL member. |
| Sample ID list | O | The list of local UE identifiers that are in the sample space of the FL member. |
| FL member capabilities | O (NOTE) | FL member capability information (e.g. ML application type, allowed resource usage level). |
| FL member velocity | O | Indicates the FL member velocity (e.g., mobile or static). |
| FL member Location information | O | Indicates the location information of the UE (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses). |
| FL member Availability schedule | O (NOTE) | Availability schedule of the FL member for a certain FL member. |
| Supported ML model ID list | O | The list of ML model IDs for which the FL member can be used. |
| Area of Interest | O | The area of interest (i.e. location co-ordinates) for which the registration applies. |
| Time validity | O | The time validity for the registration of the FL member. |
| NOTE: At least one of these shall be present. |  |  |

7.22.3.2	VFL member registration update request

The FL member registration update request in Clause 8.4.4.4 of 3GPP TS 23.482 [2] can be expanded to enable registration of VFL members as shown in Table 7.22.3.2-1 with the additions indicated in bold.

Table 7.22.3.2-1: 3GPP TS 23.482 [2] Table 8.4.4.4-1: FL member registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the existing registration for which the update request applies. |
| Security information | M | Information for ML repository to validate the registration update request |
| FL member ID | M | The identifier of the candidate FL member for which the update applies. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| Updated registration parameters | M | The updates of the registration parameters. |
| > FL member endpoint | O | The updated endpoint information (e.g. URI) used to communicate with the FL member. |
| > FL member supported AI/ML role | O (see NOTE) | The updated supported AI/ML role of FL member which can be FL server, FL client. |
| > Feature ID list | O | Updated list of features supported by the FL member. |
| > Sample ID list | O | The updated list of local UE identifiers that are in the sample space of the FL member. |
| > FL member Availability schedule | O (see NOTE) | Updated Availability schedule of the FL member. |
| > supported ML model ID list | O (see NOTE) | The updated list of ML model IDs for which the FL member can be used |
| > area of interest | O (see NOTE) | The updated area of interest for the registration update. |
| > time validity | O (see NOTE) | The update time validity for the FL member registration update. |
| NOTE: At least one of these information elements shall be present. |  |  |

7.22.3.3	VFL training/inference request

Table 7.22.3.3-1 describes information elements for the VFL training/inference request sent from the VFL server (coordinating VAL server) to the AIMLE server.

Table 7.22.3.3-1: VFL training/inference initiation request

| Information element | Status | Description |
| --- | --- | --- |
| VFL server identifier | M | The identifier of the coordinating VFL server. |
| FL task type | M | The requested FL task which can be either inference or training. |
| Required feature ID lists | M | The required feature ID(s) for each data domain for the FL task. At least two feature lists are provided, one for each data domain. |
| Required sample ID lists | M | The required list of local UE identifiers that are common in the sample space for the FL task. |

7.22.3.4	VFL training/inference initiation request

Table 7.22.3.4-1 describes information elements for the VFL training/inference initiation request sent from the AIMLE server to the selected VFL client(s).

Table 7.22.3.4-1: VFL training/inference initiation request

| Information element | Status | Description |
| --- | --- | --- |
| VFL server identifier | M | The identifier of the coordinating VFL server. |
| FL task type | M | The requested FL task which can be either inference or training. |
| Selected feature ID | M | The selected feature ID for the FL task for the VFL client. |
| Selected sample list | M | List of the selected samples for the VFL task. Each element includes the information described below. |
| > sample ID | M | The local UE identifier in the VFL client for the selected sample. |
| > correlation ID | M | The corresponding assigned correlation ID for the sample ID. |

7.22.3.5	VFL training/inference initiation response

Table 7.22.3.5-1 describes information elements for the VFL training/inference initiation response sent from the selected VFL client(s) to the AIMLE server.

Table 7.22.3.5-1: VFL training/inference initiation response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE: One of these IEs shall be present in the message. |  |  |

7.22.3.6	VFL training/inference response

Table 7.22.3.6-1 describes information elements for the VFL training/inference response sent from the AIMLE server to the VFL server.

Table 7.22.3.6-1: VFL training/inference initiation response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| > Selected VFL client(s) | M | List of selected VFL client(s). Each element includes the information described below. |
| >> Selected VFL client identifier | M | List of selected VFL client(s). Each element includes the information described below. |
| >> Selected Feature ID | M | The selected feature ID for the VFL client. |
| > Selected sample list | M | List of the selected samples for the VFL task. Each element includes the information described below. |
| >> sample ID | M | The local UE identifier in the VFL server for the selected sample. |
| >> correlation ID | M | The corresponding assigned correlation ID for the sample ID. |
| Failure response (NOTE) | O | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * Next Change * * * *

### 7.22.4	Solution Evaluation

The solution addresses Key Issue #7, which identifies sample alignment among VAL servers in VFL as an open issue. It defines procedures and APIs to support sample alignment, enabling VFL servers to initiate training or inference tasks with consistent sample identification. VFL clients register with the AIMLE server, providing supported feature ID(s) and sample ID(s). The UE in the sample space may be referred using UE identifiers used by the specific VAL servers (i.e. local UE Identifier) which can be AF specific UE ID, ip-address or other related identifiers. The AIMLE server uses unified UE identifiers obtained via Core Network interaction to select VFL clients matching the required sample list. Correlation IDs are assigned to samples to enable consistent referencing across the VFL server and participating VFL clients. The APIs extend existing registration and update procedures and define new interfaces for initiating and coordinating VFL tasks.

NOTE: The alignment of VFL client registration and update procedures as in proposed in clause 7.22.3.1 and 7.22.3.2 with the latest changes to 3GPP TS 23.482 [2] is to be done in normative.

* * * End of Changes * * * *