3GPP TSG-SA WG6 Meeting #70	S6-255426

Dallas, United States 17th – 21st November 2025	(revision of S6-255194)

Source:	Nokia, Ericsson

Title:	Pseudo-CR on Solution to KI#3

Spec:	3GPP TR 23.700-43 (v0.2.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (Sapan.shah@nokia.com), Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This contribution provides solution to KI#3.

2. Reason for Change

In TR 23.700-43, KI#3 is agreed with following open issue:

“This key issue will study approaches to managing entities (i.e., API Invoker or API Provider) without a valid CCF issued and signed certificate, specifically how such entities can be deleted from the CCF including associated resources to the entities like Security_Context associated to API Invokers and API resources published by APFs served by AEFs.”

In order to manage the entities, it is required to have valid certificates. And so, if the entities do not have valid certificate (either it is lost or corrupted), the entity needs to obtain reissued certificate. The certificate life cycle management is within scope of SA3. It is proposed that the key issue solution describes the dependency with SA3 and the potential aspects that remain under SA6 remit.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.2.0).

* * * First Change * * * *

## 6.x	Solution #x: Solution for certificate unavailability

### 6.x.1	General

This solution relates to KI #3.

### 6.x.2	Architecture Impacts

This solution does not impacts CAPIF architecture.

### 6.x.3	Solution description

The KI#3  has following open issue: “This key issue will study approaches to managing entities (i.e., API Invoker or API Provider) without a valid CCF issued and signed certificate, specifically how such entities can be deleted from the CCF including associated resources to the entities like Security_Context associated to API Invokers and API resources published by APFs served by AEFs.”

As per 3GPP TS 33.122 [3], clause 6.1, (1) the CAPIF Core Function may deliver a certificate to the API invoker in the onboarding response (figure 6.1-1, step 4) or , (2) the CAPIF Core Function may receive an API invoker's client certificate in the Onboard API Invoker request message (NOTE 2 of clause 6.1).

In the second case, for the API Invoker certificate (as well as for the AEF(s), APF(s) and AMF(s) certificates), any certificate management consideration is out of the scope of SA6. In both cases, the certificate life cycle management is within scope of SA3, and any further action in SA6 depends on the progress of the KI#4 of 3GPP TR 33.700-23 [y].

#### 6.x.3.1	Information flows

Editor's note:	The impact on the information flows of procedures in 3GPP TS 23.222 [2] depends on the outcome of KI#4 of 3GPP TR 33.700-23 [y] in SA3.

### 6.x.4	Solution evaluation

The solution is related to KI#3. Certificate management is within the scope of SA3, and any further action in SA6  depends on the progress of the KI#4 of 3GPP TR 33.700-23 [y].

NOTE:	SA6 impacts due to the SA3 outcome of the study on security aspects of credentials unavailability will be determined during normative phase.

* * * Next Change * * * *

## 9.2	Evaluation of key issues

### 9.2.3	Evaluation of key issue #3

Changes in SA6 (if required) depend on the progress of the KI#4 of 3GPP TR 33.700-23 [y] and will be considered directly in normative phase.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[4]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[y]	3GPP TR 33.700-23: "Study on security aspects of CAPIF phase 4".

* * * End of Change * * * *