3GPP TSG-SA6 Meeting #70	S6-255438

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025	(revision of S6-255119)

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0406 | rev | 1 | Current version: | 18.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEAL references |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (UK) Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2025-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Incorrect cross-references and missing reference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 3GPP TS 23.255 and corrected cross reference to 23.222 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading referencing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.2.2, 10.2.2, 14.2.2.1, 14.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Aligned changes with R18, rather than R19 of TS 23.434. |  |  |  |  |  |  |  |  |

==============First change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[5]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[13]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[14]	3GPP TS 23.002: "Network Architecture".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[18]	3GPP TS 23.203: "Policy and charging control architecture".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[20]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[21]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[22]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[23]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[24]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[25]	ETSI TS 102 894-2 (V1.2.1): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionaryMultimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[27]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

[28]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[29]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[30]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage3".

[31]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[32]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[33]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[36]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[37]	IEEE Std 802.1CB-2017: "Frame Replication and Elimination for Reliability".

[38]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[39]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[40]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[41]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[42]		3GPP TS 28.533: "Management and orchestration; Architecture framework".

[43]		3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[44]		3GPP TS 28.532: "Management and orchestration; Generic management services".

[45]		3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[46]		3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[47]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[48]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[49]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

[50]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[51]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3"

[52]	3GPP TS 23.256 "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[53]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[54]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[55]	Void

[TS23255]	3GPP TS 23.255: "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows".

==============Next change==============

### 9.2.2	On-network functional model description

Figure 9.2.2-1 illustrates the generic on-network functional model for location management.

Figure 9.2.2-1: On-network functional model for location management

The location management client communicates with the location management server over the LM-UU reference point. The location management client provides the support for location management functions to the VAL client(s) over LM-C reference point. The VAL server(s) communicate with the location management server over the LM-S reference point. The VAL client communicates with the VAL server over the VAL-UU reference point which is outside the scope of this document.

The location management server communicates with the SCEF via T8 reference point to obtain location information from the underlying 3GPP network system. The location management server obtains location information from the NEF via N33 reference point by mechanism defined in clause 5.2.6.2 of 3GPP TS 23.502 [11]. The location management server may obtain location information from the GMLC via Le reference point defined in clause 4.4.1 of 3GPP TS 23.273 [50]. The location management server may optionally obtains location information from the 3rd party location server via LM-3P reference point.

When the fused location function is present, the location management server, may use the location information from multiple sources to determine a more accurate UE location. The fused location function may select one or more location sources and location methods based on the requested location QoS which obtained from the location management server.

The location management server supports to report the requested value-added location information (e.g. Geofencing monitoring events, history location data, prediction related to UE location, etc.) to the VAL server over LM-S reference point.

NOTE 1:	Location information from LCS of 4G system is not exposed by SCEF.

NOTE 2:	Non-3GPP access used by the UE is out of scope of the present document.

Figure 9.2.2-2 exhibits the service-based interfaces for providing and consuming location management services. The location management server could provide service to VAL server and location management client through interface Slm.

Figure 9.2.2-2: Architecture for location management – Service based representation

Figure 9.2.2-3 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [10].

Figure 9.2.2-3: Architecture for location management utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 9.2.2-4 illustrates the service-based representation for utilization of the Core Network (5GC, EPC) northbound APIs via CAPIF.

Figure 9.2.2-4: Utilization of Core Network Northbound APIs via CAPIF – service based representation

The Location management server acts as authorized API invoker to consume services from the Core Network (5GC, EPC) northbound API entities like SCEF, NEF, SCEF+NEF which act as API Exposing Function as specified in 3GPP TS 23.222 [8].

==============Next change==============

### 10.2.2	On-network functional model description

Figure 10.2.2-1 illustrates the generic on-network functional model for group management.

Figure 10.2.2-1: On-network functional model for group management

The group management client communicates with the group management server over the GM-UU reference point. The group management client provides the support for group management functions to the VAL client(s) over GM-C reference point. The VAL server(s) communicate with the group management server over the GM-S reference point.

The group management server interacts with the NEF of the underlying 3GPP network system via N33 reference point to perform group management procedures for 5G Virtual Network (5GVN) groups.

Figure 10.2.2-2 exhibits the service-based interfaces for providing and consuming group management services. The group management server could provide service to VAL server and group management client through interface Sgm.

Figure 10.2.2-2: Architecture for group management – Service based representation

Figure 10.2.2-3 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [10].

Figure 10.2.2-3: Architecture for group management utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 10.2.2-4 illustrates the service-based representation for utilization of the Core Network northbound APIs via CAPIF.

Figure 10.2.2-4: Utilization of Core Network Northbound APIs via CAPIF – service based representation

The Group management server acts as authorized API invoker to consume services from the Core Network northbound API entities like NEF which act as API Exposing Function as specified in 3GPP TS 23.222 [8].

============== Next change==============

#### 14.2.2.1	Generic on-network functional model for network resource management

Figure 14.2.2-1 illustrates the generic on-network functional model for network resource management.

Figure 14.2.2.1-1: On-network functional model for network resource management

The network resource management client communicates with the network resource management server over the NRM-UU reference point. The network resource management client provides the support for network resource management functions to the VAL client(s) over NRM-C reference point. The VAL server(s) communicate with the network resource management server over the NRM-S reference point.

The network resource management server obtains and controls the multicast/broadcast resource from the underlaying 3GPP network system by communicating with the BM-SC via MB2-C and xMB-C reference points towards LTE eMBMS and with the 5G MBS control plane functions via Nmb13, Nmb10 or N33 towards the 5GS. The network resource management server communicates with the PCRF via Rx reference point or communicates with the PCF via N5 reference point to control the unicast resources from the underlying 3GPP network system. The network resource management server communicates with the SCEF via T8 reference point or communicates with the NEF via N33 reference point to perform event monitoring procedures from the underlying 3GPP network system. The network resource management server interacts with NEF via N33 to obtain QoS monitoring information from the 5GS.

Figure 14.2.2.1-2 exhibits the service-based interfaces for providing and consuming group management services. The network resource management server could provide service to VAL server and network resource management client through interface Snrm.

Figure 14.2.2.1-2: Architecture for network resource management – Service based representation

Figure 14.2.2.1-3 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [10].

Figure 14.2.2.1-3: Architecture for network resource management utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 14.2.2.1-4 illustrates the service-based representation for utilization of the Core Network northbound APIs via CAPIF.

Figure 14.2.2.1-4: Utilization of Core Network Northbound APIs via CAPIF – service based representation

The Network resource management server and VAL server act as authorized API invoker to consume services from the Core Network northbound API entities like NEF which act as API Exposing Function as specified in 3GPP TS 23.222 [8].

==============Next change==============

#### 14.3.5.1	General

This feature provides the SEAL NRM support for coordinated QoS/resource management for network assisted UE-to-UE communications. Such capability may be required for guaranteeing end-to-end QoS fulfilment (primarily for meeting end-to-end latency requirements) in network assisted UE to UE communications and may accommodate various vertical-specific application services, e.g.:

-	Network-assisted Command and Control (C2) communications in UASAPP, 3GPP TS 23.255 [TS23255], where the UAV controller navigates its UAV over the 5GS;

-	Teleoperated Driving (ToD) in eV2XAPP, 3GPP TS 23.286 [7], where the a V2X UE acting as server may remotely control a further V2X UE over the 5GS;

-	Network-assisted Device-to-Device communications in Factory of the Future (FF) use cases, such as control-to-control communications.

-	5G LAN-Type communication within a 5G VN group as specified in 3GPP TS 23.501 [10].

==============End of change==============