3GPP TSG-SA WG6 Meeting #70	S6-255618

Dallas, United States 17th – 21st November 2025	(revision of S6-255317)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0750 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service continuity during satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20, EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The current 3GPP TS 23.558 specification does not consider the service continuity aspects for the scenario when UE is stationary and satellite is orbiting or when UE is mobile and satellite is orbiting in the same direction of UE or when UE is mobile and satellite is orbiting in the opposite direction of UE. The existing ACR management events as defined in clause 8.6.3 of 3GPP TS 23.558 and various ACR scenarios as defined in clause 8.8.2 of TS 23.558 do not explicitly cover the need or potential need for initiating ACR due to UE going out of coverage from the serving satellite. - Consequently, when UE goes out of coverage from being served by an EAS onboard satellite (due to UE or the serving satellite mobility), the UE may experience service discontinuity or session interruption if the ACR is not initiated well timed. Therefore, there is a need for detecting, deciding and executing ACR to ensure service continuity during UE going out of coverage from serving satellite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the aspects when UE coverage by the serving satellite may be lost, satellite mobility triggered ACR. Also, updated the various ACR scenarios with the UE going out of coverage from the serving satellite. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service continuity during satellite access will remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All new - 11.1.3, 11.1.3.1, 11.1.3.2, 11.1.3.2.1, 11.1.3.2.2, 11.1.3.2.3, 11.1.3.2.4, 11.1.3.2.5, 11.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 11.1.3	Service continuity during satellite access

#### 11.1.3.1	General

Serving satellite mobility is another possible trigger for ACR other than the trigger scenarios described in clause 8.8.1.1A. The EEC, EAS, and EES detect the need for ACR due to the serving satellite mobility.

The EEC may detect the need, or potential need, for ACR based on ASCAI information such as geographical service area and availability time period available in the EES profile received in service provisioning to predict the coverage end of S-EES.

The EAS may detect the need, or potential need, for ACR by subscribing to the service continuity related events (eg., "out of service area" event as described in clause 8.6.3.2.2) and receive corresponding notifications (eg., "out of service area" event as described in clause 8.6.3.2.3).

The EES may detect the need, or potential need, for ACR through the ASCAI information (such as geographical service area and availability time period) available in the EES profile.

The serving satellite mobility triggered ACR event needs additional clarifications in the clauses 4.6, 8.6.3, 8.8.1 and 8.8.3 and 8.8.3 as described below.

#### 11.1.3.2	Updates to General clauses 4.6, 8.6.3 and 8.8.1

Clause 4.6 is clarified to consider scenario when a serving satellite moves outside the expected/predicted location of UE, different EASs can be more suitable for serving the UE.

"Out of service area" in clause 8.6.3 is clarified that this event supports to detect whether a UE  is out of the service area of the subscribing EAS (which is either deployed on ground or onboard satellite). Clause 8.6.3.2.2 step 1b is clarified to - The EAS may include the "out of service area" event to indicate the EES to notify the EAS when the EES detects that a UE is out of the subscribed EAS service area (which is either deployed on ground or onboard satellite). Clause 8.6.3.2.3 step 2 is clarified to - The EES includes the ACR management event notification information of the UE(s), and optionally the timestamp. If the event triggering the notification is user plane path change or serving area of interest change due to UE mobility or serving satellite mobility, the timestamp can be included to indicate the age of the user plane path management or mobility event notification information.

The serving satellite mobility triggered ACR event needs the following updates to clause 8.8.1.1.

When a UE loses coverage due to serving satellite mobility, different EASs can be more suitable for serving the ACs in the UE.

UE coverage by the serving satellite may be lost when:

-	UE is stationary and satellite is orbiting;

-	UE is mobile and satellite is orbiting in the same direction of UE; and

-	UE is mobile and satellite is orbiting in the opposite direction of UE.

Following intra-EDN, inter-EDN, between EDN and Cloud, and LADN (overlapping LADN service areas) related scenarios supported for service continuity additionally includes mobility of serving satellite.

A detection entity detects the need, or potential need, for ACR by monitoring various aspects including predicted serving satellite mobility based on ASCAI information available in EES profile.

ACR can be performed for service continuity planning, also when the serving satellite is predicted to move outside the the expected/predicted UE location. In such a case the T-EAS is expected to provide service to the UE either when it moves to the expected location or when the serving satellite is expected to move outside the expected/predicted UE location.

Clause 8.8.1.1A is clarified that apart from UE mobility serving satellite mobility is also a trigger for ACR. Implementation of ACR with service continuity planning as described in clause 8.8.1.2 also utilizes ASCAI information available in EES profile.

#### 11.1.3.3	Updates to scenarios described in clause 8.8.2

The EEC, EAS, and EES detect the need for ACR due to the serving satellite mobility and hence multiple scenarios need additional clarifications as below.

##### 11.1.3.3.1	Clarifications to initiation by EEC using regular EAS Discovery in clause 8.8.2.2

In this scenario, ACR is also a result of the serving EAS is predicted to move outside the expected/predicted location of the UE. The EEC is also triggered as a result of the serving satellite mobility as described in clause 11.1.3.1. In step 1, the EEC can also detect the predicted coverage end of the serving satellite based on the ASCAI information available in EES profile received during service provisioning. Step 2, is clarified that steps 3 onwards need to be skipped when the change in serving satellite’s location does not trigger a need to change the serving EAS. Step 3 Service Provisioning procedure results in a list of T-EESs that are relevant to the supplied applications and the expected/predicted location of UE while serving satellite moves to a new location.

##### 11.1.3.3.2	Clarifications to EEC executed ACR via S-EES in clause 8.8.2.3

In this scenario, the EEC is triggered as a result of the serving satellite’s movement as described in clause 11.1.3.1. Step 1, the EEC may detect that ACR may be required for predicted satellite movement as described in clause 11.1.3.1. Step 4a, the EEC performs ACR launching procedure (as described in clause 8.8.3.4) to the S-EES also with predicted serving satellite mobility.

##### 11.1.3.3.3	Clarifications to S-EAS decided ACR scenario in clause 8.8.2.4

In this scenario step 1, the S-EAS either receives ACR management notifications from S-EES indicating that ACR may also be required for "out of service area" event and S-EAS may detect that ACR may be required for an expected or predicted serving satellite movement as described in clause 11.1.3.1.

##### 11.1.3.3.4	Clarifications to S-EES executed ACR in clause 8.8.2.5

Pre-condition 5, the S-EES optionally subscribed to the monitoring event UE mobility for area of interest equivalent to the service area of the S-EAS of the serving satellite as described in clause 11.1.3.1. Step 2, detection entities (S-EAS, S-EES, EEC) detect that ACR may be required and identify the ACID and Predicted/Expected UE location or Expected AC Geographical Service Area or by an S-EES subscription to the monitoring event UE mobility for area of interest as described in clause 11.1.3.1. The detection by the S-EES that ACR may be required may also be determined based on the ASCAI information (e.g., Geographical area, Availability time period) available in EES profile as described in clause 11.1.2.1. Step 3, for S-EES detected case, the S-EES may decide to perform ACR based on the ASCAI information available in the EES profile. Step 4, the S-EES decides to execute ACR based on the ASCAI information available in EES profile.

##### 11.1.3.3.5	Clarifications to EEC executed ACR via T-EES in clause 8.8.2.6

In this scenario, the EEC is also triggered as a result of serving satellite mobility as described in clause 11.1.3.1. Step 1, the EEC may detect that ACR may be required for the predicted serving satellite mobility as described in clause 11.1.3.1.

#### 11.1.3.4	ACR with ISL and without ISL

When the serving EAS is onboard satellite, the service continuity shall support two scenarios i.e., with Inter Satellite Link (ISL) availability and without ISL availability. In scenarios supporting ISL, source and target satellites can directly communicate with each other. Therefore, when the S-EAS deployed on Satellite-1 is predicted to move out of the UE’s expected/predicted location, the ACR and the application context transfer can be performed directly with the T-EAS deployed on Satellite-2 using the ISL.

In scenarios without ISL support, source and target satellites may not have any direct communication link. Consequently, the S-EAS deployed on Satellite-1 may not directly communicate with the T-EAS deployed on Satellite-2. In such scenarios, an indirect path for transferring the application context shall be used to support service continuity. The S-EAS transfers the application context to an EAS deployed on ground, the application context is then forwarded to the T-EAS deployed on Satellite-2 (which is identified as the serving satellite for the UE).

# * * * * End of changes * * * *