3GPP TSG-SA WG6 Meeting #70	S6-255658

Dallas, USA, 17th – 21st November 2025	(revision of S6-255170, 255538, 255625)

Source:	Huawei, Hisilicon, interDigital

Title:	Pseudo-CR on converaged solution to KI#1

Spec:	3GPP TR 23.700-26 0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR propose to add the missing information flow table to solution#6.

2. Reason for Change

Currently, the information flow table is missing from the soltuion #6.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.2.0.

* * * First Change * * * *

7.6	Solution #6: Solution for AIoT service provision

7.6.1	Solution description

7.6.1.1	General

This solution address key issue#2, and #4 to introduce a solution for AIoT service provision.

AIoT network service can provide the information of what (AIoT device IDs) and where (AF ID and the external Target Area information), as shown in the TS 23.369[7]. However, these network exposured data can’t be directly consumed by the 3rd party appication layer and needs to be correlated with application information. The core idea is to establish a mapping relationship for network layer information and the application data.

7.6.1.2	Procedures

The figure 7.6.1.2-1 show the high-level procedures of AIoT service provisioning.

Figure 7.6.1.2-1: Service data provisioning

1.	The AIoT application layer consumer invokes the service provisioning service of AIoT enabler server by sending the AIoT service data provisioning request. The request contains the following information:

-	a.	Mapping of AIoT devece IDs and specific application objects (such as a product, a pallet, a tool, or a person) ID. This is the premise of all associations. When attaching AIoT devices to objects of the ASP, it is necessary to create a "AIoT devece ID and application object" association in theAIoT enabler server to record this binding relationship.

b.	The application object information including the object ID, name, type, other attributes, etc.

c.	AIoT application service regions and its associated target area information, the associated AIoT service scenarios.

d.	AIoT service scenario and its cooresponding service requriements and interested AIoT service events

The AIoT service scenario is tightly correlated with the application service meanings. The cooresponding service requirements and the interested AIoT service event will be used be later inforced during the invocation of AIoT network services and the processing of network exposured AIoT data before providing enriched information to the AIoT application layer.

The AIoT service scenario categorized based on the application requirements, such as inbound/outbound, inventory, monitor and tag command, and each type may need different kinds of 3GPP AIoT network service. For different scenarios, the interested AIoT service events may be different, which should be set according to the requirements of the application layer. For example, the warehouse storage inventory scenario may need the inventory result about what objects are present or what objects are absent or the number of objects.

An example of scenario type information is listed as table 7.6.1.2-1.

Table 7.6.1.2-1: an example of scenario type, service requirement, interested service event

| AIoT service scenario Type | AIoT network services requements | interested AIoT service events |
| --- | --- | --- |
| Inbond/Outbound | Rapid inventory and report with time interval X for result aggregation | Inbond/Outbound event/ Alarm Event |
| Storage Inventory | non-rapid inventory and report time interval Y for result aggregation | Inventory/Alarm/ Counting Event |
| Monitor | periodic inventory | Moving/Alarm Event |
| Tag command | Read/Write/Disable service | Read/Write/Disable result event |

2.	The AIoT enabler layer performs the authorization check. If authorized, it stores the information received in the request for futher usage during the AIoT task execution and notification phase.

3.	The AIoT enabler layer returns a AIoT service data provisioning response indicating the success of service provisioning.

7.6.1.3	Information flows

7.6.1.3.1 AIoT service data provisioning request

Table 7.6.1.3.1 describes the AIoT service data provisioning request from the consumer (VAL server) and the AIoT enabler server.

Table 7.6.1.3.1-1: AIoT service data provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requester identifier | M | The identifier of the requester |
| List of AIoT device mapping info | O (NOTE) | The list of mapping of AIoT devece ID and the application service data |
| > AIoT device mapping information | M | The mapping of AIoT devece ID and the application service data as described in table 7.6.1.3.1-2 |
| List of application objects | O(NOTE) | The list of application object information which describes the attributes of an application object |
| > Application object information | M | The attrbiutes of an application object as described in table 7.6.1.3.1-3 |
| List of application regions | O(NOTE) | The list of application region information which describes the attributes of an application layer region |
| > Application region information | M | The attributes of an application layer region as described in table 7.6.1.3.1-4 |
| List of service scenario | O(NOTE) | The list of application region information which describes the attributes of a service scenario |
| > Service scenario information | M | The attributes of a service scenario as described in table 7.6.1.3.1-5 |
| NOTE: At least one of them shall be present. |  |  |

Table 7.6.1.3.1-2: AIoT device mapping information

| Information element | Status | Description |
| --- | --- | --- |
| AIoT Device ID | M | The device ID of the tags. |
| Active state | M | Indicates whether the tag is activated at the application layer. |
| Application objects ID | M | The identifier of the application object. |
| Application region ID | O | The application region associated with the tag |
| Service scenario ID | O | The service scenario associated with the tag |
| VAL server information | O | The endpoint or identifier of the VAL server to receive the information related with this tag |

Table 7.6.1.3.1-3: Application object information

| Information element | Status | Description |
| --- | --- | --- |
| Application object ID | M | The unique identifier of the application object. |
| Application Object Type | M | The type of the application objects. |
| Application Object Name | O | The specific name of the application objects. |
| Other Application Attributes | O | Other attributes of the application objects at the application layer. |

Table 7.6.1.3.1-4: Application region informaiton

| Information element | Status | Description |
| --- | --- | --- |
| AIoT application region ID | M | The unique identifier of the AIoT application region. |
| AIoT application region name | O | The name of the AIoT application region. |
| Target service area info | O | The target service area information mapped to the AIoT application region |
| Service scenario ID | M | The AIoT service scenario that the region area belongs to (e.g., inbound/outbound, Storage Inventory, Monitor, Tag command) |

Table 7.6.1.3.1-5: Service scenario information

| Information element | Status | Description |
| --- | --- | --- |
| AIoT service scenario ID | M | The unique identifier of AIoT service scenario |
| AIoT service scenario name | O | The name of AIoT service scenario |
| AIoT network services requirements | M | AIoT network services requirements |
| >AIoT service operation type | M | Inventory/Read/Write/Disable service |
| >IS periodic | M | Single or periodic. |
| >Period | O | If periodic, the period of the AIoT service operation. |
| >device ID information | O | Device ID information (e.g., device id filter information, estimated quantity) |
| Interested AIoT service events | M | The information that the AIoT application services interests. |
| >Inbound event | O (NOTE) | Which application objects have been arriving at which AIoT application service region. |
| >Outbound event |  | Which application objects have been leaving at which AIoT application service region. |
| >Alarm event | O(NOTE) | What application objects are not present at which AIoT application service area. |
| > Inventory event | O(NOTE) | What application objects have been inventoried at which AIoT application service area. |
| >Counting event | O(NOTE) | The counting result of the devices that are inventoried. |
| >Moving event | O(NOTE) | What application objects have been moved from one AIoT application service area to another. |
| >service result event | O(NOTE) | The result of the AIoT read/write/disable service. |
| NOTE: At least one of them shall be present. |  |  |

NOTE:	The Application objects ID, Application region ID, Service scenario ID are unique within a specific application domain.

7.6.1.3.2 AIoT service data provisioning response

Table 7.6.1.3.2-1 describes the AIoT service data provisioning response from the AIoT enabler server and the consumer (VAL server).

Table 7.6.1.3.2-1: AIoT service data provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause. |
| NOTE: The IE is only present if the Result is failure. |  |  |