3GPP TSG-SA WG6 Meeting #70	S6-255667

Dallas , US 17th – 21th November 2025	(revision of S6-255606, S6-255195)

Source:	Nokia

Title:	Pseudo-CR on handling of Service API invocations based on location of API invoker

Spec:	3GPP TR 23.700-43 (v0.2.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@nokia.com)

1. Introduction

This contribution provides solution to KI#6.

2. Reason for Change

In SA6#69, KI#6 was agreed with following open issue:

- How to determine and ensure correct handling on Service API invocations based on location of API invoker.

In order to ensure API invoker access the API within service area of AEF, it is required that the CCF issues access token only after checking current location of the API invoker.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.2.0).

* * * First Change * * * *

## 6.x	Solution #x: Handling of Service API invocations based on location of API invoker

### 6.x.1	General

This solution relates to KI #5 and KI #6.

### 6.x.2	Architecture Impacts

This solution has no architecture impacts.

### 6.x.3	Solution description

Clause 8.11 of 3GPP TS 23.222 [2] has been modified as follows:

### 8.11.3 Procedure Figure 8.11.3-1 illustrates the procedure for obtaining authorization to access the service API. Pre-condition: 1. The API invoker is onboarded and has received an API invoker identity. Figure 8.11.3-1: Procedure for the API invoker obtaining authorization for service API access 1. The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API by including the API invoker information required for authorization, the information identifying the service API, and any information required for authentication of the API invoker. The request may include desired Network Slice Info of the service API. In addition to the information identifying the service API, the request may include finer granularity level for service API authorization e.g., service API operation and resource. 2. The CAPIF core function validates the authentication of the API invoker (using authentication information) and checks whether the API invoker is permitted to access the requested service API, considering also service API operation(s) and resource(s) restrictions locally configured, if available. The CCF may additionally obtain location of the API invoker by interacting with core network, and validates whether API invoker is within Serving Area of the service API for which authorization is requested. The CAPIF core function may additionally verify the Network Slice Info, e.g., check that the authorized Network Slice Info for the API invoker is included in the supported Network Slice Info for the indicated service API. Editor's Note: How CCF determines that the API invoker is on UE and its identity to interact with the CN to obtains the UE location is FFS. NOTE 1: The authorization of Network Slice Info is performed by the CAPIF core function within the PLMN trust domain. NOTE 2: The authentication process is specified in subclause 6.5.2 of 3GPP TS 33.122 [12]. NOTE 3: The CCF may reject the request if the API invoker is ourside the Serving Area of the service API. 3. Based on the API invoker's subscription information available in the CCF, the authorization information to access the service APIs, which may include access authorization per service operation, resource, optionally including access authorization per service operation and, optionally, resource, is sent to the API invoker in the obtain service API authorization response. NOTE 3: How the finer service API authorization granularity is indicated in the request from API invoker and is returned to the API invoker in the CCF authorization reply is specified in 3GPP TS 33.122 [12]. NOTE 4: The mechanism for distribution of the authorization information for the API invoker to the API exposing function over CAPIF-3 reference point is specified in subclause 6.5.2 of 3GPP TS 33.122 [12].

Further, the AEF upon receiving the service API invocation request can obtain API invoker’s location before serving the API invoker.

Editor’s Note:	The security aspects (including scenario where API invoker gets authorization token within service area but invokes outside the service area) are within scope of SA3.

Editor's Note:	How to handle ongoing service based on changes in UE location, is FFS.

#### 6.x.3.1	Information flows

The solution does not need any new information flows.

* * * End of Change * * * *