3GPP TSG-SA WG6 Meeting #70	S6-255677

Dallas, United States 17th – 21st November 2025	(revision of S6-255404, S6-255036)

Source:	KPN N.V

Title:	Completion and Evaluation of Solution #19 Cross-PLMN/Domain AIMLE client discovery, selection, monitoring

Spec:	3GPP TR 23.700-83 1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

This contribution completes Solution #19 by correcting and finalizing the procedure, and by adding the necessary API updates in accordance with the proposed approach. It also provides a comprehensive evaluation of the procedure.

2. Reason for Change

Solution #19 still contains several unresolved Editor Notes, and the procedure remains incomplete. This contribution addresses these gaps by correcting and completing the procedure, incorporating the required API updates, and providing a full evaluation of the procedure.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 V1.1.0.

* * * First Change * * * *

#### 7.19.1.2	Procedure

Figure 7.19.1.2-1: Cross-domain/cross-PLMN AIMLE client discovery, selection and monitoring

- A VAL server sends an AIMLE client selection subscription request to the AIMLE server as described in step 1 of clause 8.13.2.2 of 3GPP TS 23.482 [2]. The VAL server includes in its subscription an indication that federation with other AIMLE servers in another domain can be used.

- The AIMLE server sends the AIMLE client selection subscription response to the VAL server as described in step 3 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

- The AIMLE server monitors AIMLE clients determine whether they fulfil the selection criteria as per step 4 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

- The AIMLE Server obtains the identifiers of the AIMLE clients from the monitoring and selects the clients that fulfil the selection criteria and remove the AIMLE clients that do not fulfil the selection criteria as per step 5 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

- The AIMLE server determines that there are insufficient AIMLE clients in the current domain either because there were not enough AIMLE clients fulfilling the criteria or because previously qualifying AIMLE clients cease to fulfil the criteria or they are currently unavailable.

- The AIMLE server, when cross-domain/cross-PLMN federation is enabled, identifies an AIMLE server in another domain/PLMN (AIMLE server - 2) to federate for AIMLE client selection. The AIMLE servers in different domains, based on federation agreement, can either be preconfigured in the AIMLE server or stored in a central ML repository where different domains/PLMNs makes information about AIMLE servers in their domain make available as shown in Figure 6.2.1-2 of clause 6.2.

- The AIMLE server determines the discovery requirements based on the request in step 1 and missing AIMLE clients in step 5, send an AIMLE clients discovery request to the AIMLE server in the other domain (AIMLE sever - 2), and receives the AIMLE clients that fulfil the provided AIMLE client discovery criteria from the AIMLE server in the other domain. The AIMLE client discovery request can be sent as Table 8.8.3.1-1 of clause 8.8.3.1 of 3GPP TS 23.482 [2] where the requester is now AIMLE server and the number of required AIMLE clients is in line with the missing AIMLE clients and the requirements in federation indication provided in step 1.

- The AIMLE server establishes an AIMLE client selection subscription with the AIMLE server in the other domain (AIMLE server - 2). The AIMLE server sends AIMLE client selection subscription request, including in the selection criteria, the missing gap between the request in step 1 and the unavailable AIMLE clients from step 5, and it receives the subscription response from the AIMLE server in the other domain  (AIMLE server - 2). The AIMLE server following the change in monitoring as in step 11 updates the AIMLE client subscription update.

The AIMLE client selection subscription request is sent as Table 8.13.3.2-1 of clause 8.13.3.2 of 3GPP TS 23.482 [2] as updated in Table 7.19.3.1-1 of clause 7.19.3.1, where the requester is now AIMLE server and the number of required AIMLE clients is in line with the missing AIMLE clients and the requirements in federation indication provided in step 1.

The AIMLE client subscription update request is sent as Table 8.13.3.5-1 of clause 8.13.3.5 of 3GPP TS 23.482 [2] as updated in Table 7.19.3.2-1 of clause 7.19.3.2, where the requester is now AIMLE server and the number of required AIMLE clients is in line with what has changed in latest monitoring results in step 11 and the requirements in federation indication provided in step 1.

- The AIMLE server in the other domain (AIMLE server - 2) monitors AIMLE clients fulfilling the criteria from step 8. The AIMLE server in the other domain (AIMLE server - 2) interacts with the NEF and/or SEAL services (including SEALDD) within its own domain to establish monitoring as per step 4 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

The AIMLE server in the other domain (AIMLE server - 2) may determine the application QoS parameters (e.g. bandwidth, latency, jitter) for the AIML traffic session between the VAL server and the selected AIMLE client and configure the AIML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API) in accordance with step 5 of clause 8.13.2.2 of 3GPP TS 23.482 [2]. When the AIMLE clients no longer meet the criteria, the QoS adjustment is reversed as described in step 5 of clause 8.13.2.2 of 3GPP TS 23.482 [2].

- The AIMLE server receives the notification from the AIMLE server in the other domain (AIMLE server - 2) regarding selected and reselected AIMLE clients.

- Monitoring of AIMLE clients continues in the AIMLE server in the current domain and the AIMLE server in the federating domain (AIMLE server - 2). During monitoring, the AIMLE clients from the other domain can be replaced by the AIMLE clients from the current domain when suitable AIMLE clients become available locally by sending AIMLE client subscription update request as in step 8.

- The AIMLE server notifies the consolidated AIMLE client selection status to the VAL server after selecting the required AIMLE clients, including those from the other domain. The AIMLE server, in its notification, includes the AIMLE clients selected from both the current domain and other domain.

NOTE: When the VAL server unsubscribes from the AIMLE server by sending an AIMLE client selection unsubscribe request as specified in clause 8.13.3.7 of 3GPP TS 23.482 [2], the AIMLE server also cancels its own subscription by sending an AIMLE client selection unsubscribe request to the AIMLE server in the other domain (AIMLE server - 2) to which it had subscribed. The AIMLE server may also cancel its subscription when a sufficient number of AIMLE clients become available in the current domain.

* * * Next Change * * * *

### 7.19.3	Corresponding APIs

#### 7.19.3.1	AIMLE client selection subscription request

The AIMLE client selection subscription request is same as Table 8.13.3.2-1 of Clause 8.13.3.2 of 3GPP TS 23.482 [2], with the following additions in bold.

Table 7.19.3.1-1: 3GPP TS 23.482 [2] Table 8.13.3.2-1 AILME client selection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. VAL Server, AIMLE Server. |
| AIMLE client selection criteria | M | Selection criteria for finding suitable AIMLE clients for AIML operations as detailed in per clause 8.8.3.1-2. |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | M | Represents the endpoint at the requester (e.g. VAL server or AIMLE Server) for receiving the notifications on the selected AIMLE Client’s status update. |
| Federation indication | O | Indicates if federation with other AIMLE servers in another PLMN/domain may be used when the requester is not AIMLE Server. |

#### 7.19.3.2	AIMLE client subscription update request

The AIMLE client selection subscription request is same as Table 8.13.3.5-1 of Clause 8.13.3.5 of 3GPP TS 23.482 [2], with the following additions in bold.

Table 7.19.3.2-1: 3GPP TS 23.482 [2] Table 8.13.3.5-1 AIMLE client selection subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the existing subscription for which the update request applies. |
| AIMLE client selection criteria | O | Selection criteria for finding suitable AIMLE clients for AIML operations as detailed in per clause 8.8.3.1-2. |
| Number of the required AIML clients | O | Indicates the requested number of AIML clients to be selected based on member selection policies. |
| Notification endpoint for the selected AIMLE Client’s | O | Represents the endpoint at the requester (e.g. VAL server or AIMLE Server) for receiving the notifications on the selected AIMLE Client’s status update. |

* * * Next Change * * * *

### 7.19.4	Solution Evaluation

This solution addresses the cross-domain and cross-PLMN scenario, which Key Issue #3 identifies as a missing aspect in Rel-19. Specifically, it enables an AIMLE server to fulfil the need for additional AIMLE clients by subscribing to another AIMLE server in a different domain or PLMN. The solution extends the AIMLE client selection subscription and monitoring procedures to support federation with an AIMLE servers in another domain, allowing dynamic discovery and selection of AIMLE clients across domains.

The proposed solution is technically feasible and aligns with the existing architecture, extending established subscription, monitoring, and notification mechanisms in 8.13.2.2 of 3GPP TS 23.482 [2]. The solution enables the AIMLE servers in different domain/PLMN to monitor the AIMLE clients in their own domain and select AIMLE clients in a coordinated manner.

Service continuity is maintained by allowing dynamic replacement of AIMLE clients selected from the other domain with AIMLE clients in the current domain as they become available.

* * * End of Changes * * * *