3GPP TSG-CT WG1 Meeting #157	C1-256020

Sophia Antipolis, France, 13-17 October 2025

Source:	China Mobile

Title:	pCR on the cause value used in the procedures

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This p-CR proposes to define the cause values for AIoT device and AIOTF used in the command procedures in the 3GPP TS 24.369 specification.

2. Reason for Change

The rejected and abnormal cases were agreed in CT1#156 with the TBD cause values. It is suggested to re-discuss the cause values for the procedures in the 3GPP TS 24.369 specification and solve the EN.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if read command is not supported by the AIoT device, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #x1 "Command not supported".

Editor's note:	Whether to use the READ COMMAND REJECT message to reject the READ COMMAND message when the read command not supported is FFS.

Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #x2 "Command type specific parameters invalid".

5.3.2.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The read command is unsuccessfully implemented (e.g. user memory operation error).

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #x3 "Error, unspecified".

b)	The application data cannot be found in the requested place of the user memory.

The AIoT device shall send a READ COMMAND REJECT message with a cause set to #x4 "Data not found".

Editor's note:	It is FFS the command and inventory procedure collision case.

* * * Next Change * * * *

5.3.3.5	Write command procedure not accepted by AIoT device

Upon receiving a WRITE COMMAND message, if write command is not supported by the AIoT device, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #x1 "Command not supported".

Editor's note:	Whether to use the WRITE COMMAND REJECT message to reject the WRITE COMMAND message when the write command not supported is FFS.

Upon receiving a WRITE COMMAND message, if the command type specific parameters (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #x2 "Command type specific parameters invalid".

Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #x5 "Low energy".

5.3.3.6	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	The write command is unsuccessfully implemented (e.g. user memory operation error)

The AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #x3 "Error, unspecified".

* * * Next Change * * * *

7.2.8	Cause

The purpose of the cause information element is to indicate the cause of the failure of the read or write operation.

The cause information element is coded as shown in figure 7.2.8-1 and table 7.2.8-1.

The cause is a type 3 information element with length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 7.2.8-1: Cause information element

Table 7.2.8-1: Cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | Command not supported |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | Command type specific parameters invalid |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | Error, unspecified |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | Data not found |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | Low energy |
| All other values are spare and if received shall be interpreted as "Error, unspecified". |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

A.1	Causes related to command procedure

Cause #x1 – Command not supported

This cause is used to indicate that the requested read or write command is not supported by the AIoT device.

Cause #x2 – Command type specific parameters invalid

This cause is used to indicate that the command type specific parameters (e.g. the offset to read application data from the AIoT device user memory, the length of the application data to read, the offset where to write the application data, or the length of the application data to write) is invalid.

Cause #x3 – Error, unspecified

This cause is used to indicate that the requested read or write command was not executed successfully.

Cause #x4 – Data not found

This cause is used to indicate that the application data to be read cannot be found by the AIoT device in the requested place of the user memory.

Cause #x5 – Low energy

This cause is used to indicate that the write command procedure cannot be implemented because the energy will be run out.

* * * End of Changes * * * *