3GPP TSG-CT WG1 Meeting #157	C1-256026

# Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Inspirations from 5G pain points for 6G protocol design

Agenda item:	20.2

Document for:	DISCUSSION

Abstract of the contribution: This paper analyses the pain points of 5G NAS protocol and proposes that the pain points learnt from 5G can be considered when designing the 6G NAS protocol.

1 Introduction

The 5G NAS protocol has provided stable and comprehensive support for access and mobility management as well as session management, which enables diverse 5G services. With the ongoing study on 6G, the continuous expansion of network functions, the increasing variety of device types, and the emergence of new service requirements impose higher expectations on NAS protocol. The systematic identification and analysis of potential areas for improvement in the 5G NAS protocol design are needed, which will serve as the basis for the study of 6G NAS protocol.

2 Discussion

2.1 MM protocol has strong coupling with other protocols

Figure 1 and Figure 2 give a description on 5G UE-CN communication for communication services and the corresponding protocol stack.

Figure 1: 5G UE-CN communication for communication services

Figure 2: Control Plane Protocol Stacks between the UE and the 5GC

As shown in Figure 1 and Figure 2, AMF is the sole anchor for all other network functions, that means all UL/DL 5G NAS signalling are anchored at the centrally deployed AMF. Such design brings below drawbacks:

-	Inefficient network upgrade and maintenance: Scale-up/down and scale-in/out of other NFs will impact the AMF, and vice versa. These increase complexity and difficulty of network upgrade and maintenance.

-	Signalling detour and congestion due to the central deployment: All UL/DL 5G NAS signalling are anchored at the centrally deployed AMF, which causes signalling detour to the 5G NFs (e.g., SMF) who can be deployed close to the network edge. The centrally deployed AMF has a large serving area which can serve millions of UEs. This creates a signalling bottleneck for entire UE-CN communication, increasing the risk of signalling congestion and AMF failures.

Apart from the architecture impact, this also brings the following drawbacks to the protocol design, as all protocols are piggybacked in the MM protocol. That is, all protocol messages are coupled with 5GMM messages, and any procedure involving NFs other than the AMF can only be initiated after the UE triggers a connection management procedure and enters the connected state. Such design brings the following issue:

a)	IEI space run out: In R18, CT1 has ran out of type 6 IEIs in REGISTRATION ACCEPT message. To resolve this issue, CT1 had to introduce a type 6 IE container IE (see clause 9.11.3.98 of TS 24.501). The type 6 IE container IE introduces an additional IEI and length field, and within the content field it still uses the IEI to identify each type 6 IE.

b)	Payload container type is going to run out: A series of NAS messages, such as SMS, UE policy, and LCS, are piggybacked in a container of the 5GMM message and are identified by the payload container type IE. The encoding values for payload container type IE are limited (only 4 bits) and only 3 values are left, making it difficult to accommodate more new service protocols.

c)	NAS specification is overly bulky: The MM protocol, SM protocol, and policy delivery protocol are all described in a single TS 24.501. This document is overly large, slow to open, and hard to read.

2.2 Inefficient design of NAS protocol

2.2.1 Inefficient interaction for the capability negotiation

The UE is required to include the 5GMM capability IE towards the AMF every registration even if there is no change towards the UE capability.

2.2.2 Inefficient design of the capability IE

The capability negotiation is done by exchanging the 5GMM Capability IE. A large number of UE capabilities are introduced in the 5GMM Capability IE for various features with the evolution of the protocol. However, the design of the capability IE is inefficient:

a)	a lot of capabilities are related to a single feature; and

b)	the bits indicating a single feature are non-contiguous.

For example, there are 13 ProSe-related capabilities that occupy 13 bits of the 5GMM Capability IE (see TS 24.501-j30). Those bits are non-contiguous as they are introduced to the protocol in different releases. Moreover, for the UE does not support ProSe feature, none of those 13 bits will be supported. However, these bits still need to be exchanged within the 5GMM Capability IE, resulting in low efficiency of capability exchange.

Figure 3 gives an example of features that have multiple sub-features.

| Category | Capability bits in 5G MM capability IE | Number of capability bits |
| --- | --- | --- |
| Slice-related | NSSAA ER-NSSAI NSAG NSSRG RCMAN NSR TempNS SLVI PNS NSUC | 10 |
| Location-related | LPP 5G-LCS LCS-UPP SUPL MLCSUP | 5 |
| ProSe-related | 5G ProSe-l2relay 5G ProSe-dc 5G ProSe-dd 5G ProSe-l3rmt 5G ProSe-l2rmt 5G ProSe-l3relay 5G ProSe-l2end 5G ProSe-l3U2U relay 5G ProSe-l2U2U relay 5G ProSe-l3end 5G ProSe-L2IMrelay 5G ProSe-L3IMrelay 5G ProSe-MCI | 13 |

Figure 3: example of feature-specific capability bits

2.2.3 Inefficient design of L3 NAS message header info

According to TS 24.501, a complete plain 5GS NAS message shall be included in the NAS message container (e.g., Plain 5GS NAS message IE).

However, some information (e.g. EPD and security header type) is repeated in both the security protected NAS message header and the plain NAS message header. Such design may lead to unnecessary NAS overhead and parsing complexity.

| Extended PD |  | 1 octet |
| --- | --- | --- |
| spare half octet | Security header type | 1 octet |
| Message authentication code |  | 4 octets |
| Sequence number |  | 1 octet |
| Extended PD |  | 1 octet |
| spare half octet | Security header type | 1 octet |
| message type |  | 1 octet |

Figure 4: General message format example for a security protected 5GS NAS message

2.2.4 Inefficient design of NAS procedures for similar purposes

NAS protocol, serving as the control plane signaling between the UE and the core network, has a process design whose efficiency directly affects network performance and user experience. The NAS procedures for similar purposes are expected to have a unified design. However, in 5G, two similar NAS procedures, SOR and UPU, have been defined, wherein:

a)	SOR procedure: The UDM of HPLMN guides UE to access optimal or lower-cost networks through the SOR procedure; and

b)	UPU procedure: The UDM of the HPLMN delivers the updated configuration parameters to the UE through the UPU procedure.

Different NAS procedures defined for similar purposes result in inefficient design of the NAS procedures.

3 Proposal

The 6G NAS protocol needs to support efficient design to overcome the limitations of 5G NAS and support both communication and beyond communication services. It is proposed to analyse the pain points learnt from 5G NAS protocol and study whether and how to improve 6G NAS protocol design to avoid or eliminate such pain points.